3GPP TSG-RAN WG2 Meeting #111-e	R2-2008325

# Electronic Meeting, 17th – 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1995 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding notes for joint success and failure in crossRAT SL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, MediaTek Inc., OPPO, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-24-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the crossRAT SL feature, the LTE RRCConnectionReconfiguration message (including sidelink-related fields) embedded in the NR RRCReconfiguration message does not receive a separate acknowledgment. The reconfiguration procedure operates as a normal NR reconfiguration procedure, meaning that the UE will send an NR RRCReconfigurationComplete message when it has successfully applied the new configuration, or trigger re-establishment if a problem occurs. This means that a joint success/failure procedure is performed. However, this aspect needs to be clarified to avoid any misunderstanding. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 5.3.5.3 - Added a note to clarify that in case of the LTE RRCConnectionReconfiguration message (including sidelink-related fields) embedded in the NR RRCReconfiguration the UE does not need to build a separate complete message. Section 5.3.5.8.2 - Added a note to clarify that the compliance of the procedure also covers any E-UTRA sidelink configuration carried within an octet string, e.g. field sl-ConfigDedicatedEUTRA. I.e. the failure behaviour defined also applies in case the UE cannot comply with the embedded E-UTRA sidelink configuration. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: RRC reconfiguration Inter-operability: If the network implements the CR and the UE does not, there is no inter-operability issue. If the UE implements the CR and the network does not, there is no inter-operability issue. If the TX UE implements the CR and the RX UE does not, there is no inter-operability issue. If the RX UE implements the CR and the TX UE does not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the CR is not approved, the network may expect a separate acknowledge/failure handling for the LTE RRCConnectionReconfiguration message (including sidelink-related fields) embedded in the NR RRCReconfiguration message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.3, 5.3.5.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331 CR 4435 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

1>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution upon cell selection while timer T311 is running, as defined in 5.3.7.3:

2>	remove all the entries within VarConditionalReconfig, if any;

1>	if the RRCReconfiguration includes the daps-SourceRelease:

2>	release source SpCell configuration;

2>	reset the source MAC and release the source MAC configuration;

2>	for each DAPS bearer:

3>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

3>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

2>	for each SRB:

3>	release the PDCP entity for the source SpCell;

3>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

2>	release the physical channel configuration for the source SpCell;

2>	discard the keys used in the source SpCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

1>	if the RRCReconfiguration is received via other RAT (i.e., inter-RAT handover to NR):

2>	if the RRCReconfiguration does not include the fullConfig and the UE is connected to 5GC (i.e., delta signalling during intra 5GC handover):

3>	re-use the source RAT SDAP and PDCP configurations if available (i.e., current SDAP/PDCP configurations for all RBs from source E-UTRA RAT prior to the reception of the inter-RAT HO RRCReconfiguration message);

1>	else:

2>	if the RRCReconfiguration includes the fullConfig:

3>	perform the full configuration procedure as specified in 5.3.5.11;

1>	if the RRCReconfiguration includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

1>	if the RRCReconfiguration includes the masterKeyUpdate:

2>	perform AS security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration includes the sk-Counter:

2>	perform security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration includes the secondaryCellGroup:

2>	perform the cell group configuration for the SCG according to 5.3.5.5;

1>	if the RRCReconfiguration includes the mrdc-SecondaryCellGroupConfig:

2>	if the mrdc-SecondaryCellGroupConfig is set to setup:

3>	if the mrdc-SecondaryCellGroupConfig includes mrdc-ReleaseAndAdd:

4>	perform MR-DC release as specified in clause 5.3.5.10;

3>	if the received mrdc-SecondaryCellGroup is set to nr-SCG:

4>	perform the RRC reconfiguration according to 5.3.5.3 for the RRCReconfiguration message included in nr-SCG;

3>	if the received mrdc-SecondaryCellGroup is set to eutra-SCG:

4>	perform the RRC connection reconfiguration as specified in TS 36.331 [10], clause 5.3.5.3 for the RRCConnectionReconfiguration message included in eutra-SCG;

2>	else (mrdc-SecondaryCellGroupConfig is set to release):

3>	perform MR-DC release as specified in clause 5.3.5.10;

1>	if the RRCReconfiguration message includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the radioBearerConfig2:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	if the RRCReconfiguration message includes the dedicatedNAS-MessageList:

2>	forward each element of the dedicatedNAS-MessageList to upper layers in the same order as listed;

1>	if the RRCReconfiguration message includes the dedicatedSIB1-Delivery:

2>	perform the action upon reception of SIB1 as specified in 5.2.2.4.2;

NOTE 0:	If this RRCReconfiguration is associated to the MCG and includes reconfigurationWithSync in spCellConfig and dedicatedSIB1-Delivery, the UE initiates (if needed) the request to acquire required SIBs, according to clause 5.2.2.3.5, only after the random access procedure towards the target SpCell is completed.

1>	if the RRCReconfiguration message includes the dedicatedSystemInformationDelivery:

2>	perform the action upon reception of System Information as specified in 5.2.2.4;

1>	if the RRCReconfiguration message includes the dedicatedPosSysInfoDelivery:

2>	perform the action upon reception of the contained posSIB(s), as specified in sub-clause 5.2.2.4.16;

1>	if the RRCReconfiguration message includes the otherConfig:

2>	perform the other configuration procedure as specified in 5.3.5.9;

1>	if the RRCReconfiguration message includes the bap-Config:

2>	perform the BAP configuration procedure as specified in 5.3.5.12;

1>	if the RRCReconfiguration message includes the iab-IP-AddressConfigurationList:

2>	if iab-IP-AddressToReleaseList is included:

3>	for each IAB-IP-AddressIndex received in the iab-IP-AddressToReleaseList

4>	perform release of IP address as specified in 5.3.5.12a.1.1;

2>	if iab-IP-AddressToAddModList is included:

3>	for each IAB-IP-AddressIndex received in the iab-IP-AddressToAddModList

4>	perform IAB IP address addition/update as specified in 5.3.5.12a.1.2;

1>	if the RRCReconfiguration message includes the conditionalReconfiguration:

2>	perform conditional reconfiguration as specified in 5.3.5.13;

1>	if the RRCReconfiguration message includes the needForGapsConfigNR:

2>	if needForGapsConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap requirement information of NR target bands;

1>	if the RRCReconfiguration message includes the sl-ConfigDedicatedNR:

2>	perform the sidelink dedicated configuration procedure as specified in 5.3.5.14;

NOTE 0a:	If the sl-ConfigDedicatedNR was received embedded within an E-UTRA RRCConnectionReconfiguration message, the UE does not build an NR RRCReconfigurationComplete message for the received sl-ConfigDedicatedNR.

1>	if the RRCReconfiguration message includes the sl-ConfigDedicatedEUTRA:

2>	if sl-V2X-ConfigDedicated is included in sl-ConfigDedicatedEUTRA

3>	perform the V2X sidelink communication dedicated configuration procedure as specified in 5.3.10.15a in TS 36.331 [10];

2>	if sl-V2X-SPS-Config is included in sl-ConfigDedicatedEUTRA

3>	perform V2X sidelink SPS reconfiguration as specified in 5.3.10.5 in TS 36.331 [10];

1>	set the content of the RRCReconfigurationComplete message as follows:

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each MCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each MCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each SCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each SCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to eutra-SCG:

3>	include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3;

2> if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to nr-SCG:

3>	include in the nr-SCG-Response the RRCReconfigurationComplete message;

2>	if the RRCReconfiguration message was included in an RRCResume message:

3>	include the RRCReconfigurationComplete message in the nr-SCG-Response within the scg-Response in the RRCResumeComplete message;

2>	if the RRCReconfiguration message was included in E-UTRA RRCConnectionResume message:

3>	include the RRCReconfigurationComplete message in the E-UTRA MCG RRC message RRCConnectionResumeComplete in accordance with TS 36.331 [10], clause 5.3.3.4a;

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCReconfigurationComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCReconfigurationComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCReconfigurationComplete message;

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable in the RRCReconfigurationComplete message;

2>	if the RRCReconfiguration message was received in response to the MCGFailureInformation message:

3>	clear the information included in VarRLF-Report, if any;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	include rlf-InfoAvailable in the RRCReconfigurationComplete message;

2>	if the RRCReconfiguration message was received via SRB1, but not within mrdc-SecondaryCellGroup or E-UTRA RRCConnectionReconfiguration:

3>	if the UE is configured to provide the measurement gap requirement information of NR target bands:

4>	if the RRCReconfiguration message includes the needForGapsConfigNR; or

4>	if the NeedForGapsInfoNR information is changed compared to last time the UE reported this information:

5>	include the NeedForGapsInfoNR and set the contents as follows:

6>	include intraFreq-needForGap and set the gap requirement informantion of intra-frequency measurement for each NR serving cell;

6>	if requestedTargetBandFilterNR is configured, for each supported NR band that is also included in requestedTargetBandFilterNR, include an entry in interFreq-needForGap and set the gap requirement information for that band; otherwise, include an entry in interFreq-needForGap and set the corresponding gap requirement information for each supported NR band;

1>	if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC):

2>	if the RRCReconfiguration message was received via E-UTRA SRB1 as specified in TS 36.331 [10]; or

2>	if the RRCReconfiguration message was received via SRB3 within DLInformationTransferMRDC;

3>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution:

4>	submit the RRCReconfigurationComplete message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10], clause 5.6.2a.

3>	else:

4>	submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10], clause 5.3.5.3/5.3.5.4;

3>	if reconfigurationWithSync was included in spCellConfig of an SCG:

4>	initiate the Random Access procedure on the SpCell, as specified in TS 38.321 [3];

3>	else:

4>	the procedure ends;

NOTE 1:	The order the UE sends the RRCConnectionReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

2>	else (RRCReconfiguration was received via SRB3) but not within DLInformationTransferMRDC:

3>	submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

NOTE 2:	In (NG)EN-DC and NR-DC, in the case RRCReconfiguration is received via SRB1 or within DLInformationTransferMRDC via SRB3, the random access is triggered by RRC layer itself as there is not necessarily other UL transmission. In the case RRCReconfiguration is received via SRB3 but not within DLInformationTransferMRDC, the random access is triggered by the MAC layer due to arrival of RRCReconfigurationComplete.

1>	else if the RRCReconfiguration message was received via SRB1 within the nr-SCG within mrdc-SecondaryCellGroup (UE in NR-DC, mrdc-SecondaryCellGroup was received in RRCReconfiguration via SRB1):

2>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution:

3>	submit the RRCReconfigurationComplete message via the NR MCG embedded in NR RRC message ULInformationTransferMRDC as specified in clause 5.7.2a.3.

2>	if reconfigurationWithSync was included in spCellConfig in nr-SCG:

3>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

2>	else

3>	the procedure ends;

NOTE 2a:	The order in which the UE sends the RRCReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

1>	else if the RRCReconfiguration message was received via SRB3 (UE in NR-DC):

2>	if the RRCReconfiguration message was received within DLInformationTransferMRDC:

3>	if the RRCReconfiguration message was received within the nr-SCG within mrdc-SecondaryCellGroup (NR SCG RRC Reconfiguration):

4>	if reconfigurationWithSync was included in spCellConfig in nr-SCG:

5>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

4>	else:

5>	the procedure ends;

3>	else:

4>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2>	else:

3>	submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

1>	else (RRCReconfiguration was received via SRB1):

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2>	if this is the first RRCReconfiguration message after successful completion of the RRC re-establishment procedure:

3>	resume SRB2 and DRBs that are suspended;

1>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG, and when MAC of an NR cell group successfully completes a Random Access procedure triggered above:

2>	stop timer T304 for that cell group;

2>	stop timer T310 for source SpCell if running;

2>	apply the parts of the CSI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the respective target SpCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the respective target SpCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of that target SpCell;

2>	for each DRB configured as DAPS bearer, request uplink data switching to the PDCP entity, as specified in TS 38.323 [5];

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG:

3>	if T390 is running:

4>	stop timer T390 for all access categories;

4>	perform the actions as specified in 5.3.14.4.

3>	if T350 is running:

4>	stop timer T350;

3>	if RRCReconfiguration does not include dedicatedSIB1-Delivery and

3>	if the active downlink BWP, which is indicated by the firstActiveDownlinkBWP-Id for the target SpCell of the MCG, has a common search space configured by searchSpaceSIB1:

4>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13], of the target SpCell of the MCG;

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG; or:

2>	if the reconfigurationWithSync was included in spCellConfig of an SCG and the CPC was configured

3>	remove all the entries within VarConditionalReconfig, if any;

3>	for each measId of the source SpCell configuration, if the associated reportConfig has a reportType set to condTriggerConfig:

4>	for the associated reportConfigId:

5>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

4>	if the associated measObjectId is only associated to a reportConfig with reportType set to cho-TriggerConfig:

5>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

4>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup; and

2>	if the UE transmitted a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide UE assistance information for the corresponding cell group:

3>	initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3;

2>	if SIB12 is provided by the target PCell; and the UE transmitted a SidelinkUEInformationNR message indicating a change of NR sidelink communication related parameters relevant in target PCell (i.e. change of sl-RxInterestedFreqList or sl-TxResourceReqList) during the last 1 second preceding reception of the RRCReconfiguration message including reconfigurationWithSync in spCellConfig of an MCG:

3>	initiate transmission of the SidelinkUEInformationNR message in accordance with 5.8.3.3;

2>	the procedure ends.

NOTE 3:	The UE is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

END OF CHANGE

START OF CHANGE

##### 5.3.5.8.2	Inability to comply with RRCReconfiguration

The UE shall:

1>	if the UE is in (NG)EN-DC:

2>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over SRB3;

3>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	if MCG transmission is not suspended:

4>	initiate the SCG failure information procedure as specified in subclause 5.7.3 to report SCG reconfiguration error, upon which the connection reconfiguration procedure ends;

3>	else:

4>	initiate the connection re-establishment procedure as specified in TS 36.331 [10], clause 5.3.7, upon which the connection reconfiguration procedure ends;

2>	else, if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over SRB1;

3>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	initiate the connection re-establishment procedure as specified in TS 36.331 [10], clause 5.3.7, upon which the connection reconfiguration procedure ends.

1>	else if RRCReconfiguration is received via NR (i.e., NR standalone, NE-DC, or NR-DC):

2>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over SRB3;

NOTE 0:	This case does not apply in NE-DC.

3>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	if MCG transmission is not suspended:

4>	initiate the SCG failure information procedure as specified in subclause 5.7.3 to report SCG reconfiguration error, upon which the connection reconfiguration procedure ends;

3>	else:

4>	initiate the connection re-establishment procedure as specified in clause 5.3.7, upon which the connection reconfiguration procedure ends;

2>	else if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over the SRB1 or if the upper layers indicate that the nas-Container is invalid:

NOTE 0a:	The compliance also covers the SCG configuration carried within octet strings e.g. field mrdc-SecondaryCellGroupConfig. I.e. the failure behaviour defined also applies in case the UE cannot comply with the embedded SCG configuration or with the combination of (parts of) the MCG and SCG configurations.

NOTE 0b:	The compliance also covers the E-UTRA sidelink configuration carried within an octet string, e.g. field sl-ConfigDedicatedEUTRA. I.e. the failure behaviour defined also applies in case the UE cannot comply with the embedded E-UTRA sidelink configuration.

3>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	if AS security has not been activated:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other'

3>	else if AS security has been activated but SRB2 and at least one DRB or, for IAB, SRB2,have not been setup:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the reconfiguration procedure ends;

1>	else if RRCReconfiguration is received via other RAT (Handover to NR failure):

2>	if the UE is unable to comply with any part of the configuration included in the RRCReconfiguration message or if the upper layers indicate that the nas-Container is invalid:

3>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT.

NOTE 1:	The UE may apply above failure handling also in case the RRCReconfiguration message causes a protocol error for which the generic error handling as defined in clause 10 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure.

NOTE 3:	It is up to UE implementation whether the compliance check for an RRCReconfiguration received as part of ConditionalReconfiguration is performed upon the reception of the message or upon CHO and CPC execution (when the message is required to be applied).

END OF CHANGE