3GPP TSG-RAN WG2 Meeting #111-e	R2-2008400

# Electronic Meeting, 17th – 28th Aug 2020                   Revision of R2-2007709

| CR--Form--v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4405 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Compliance check failure for CPC configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2020-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - According to the current RRC spec, all stored CPC configuration will be released upon PCell change. For the PCell change without SN involved, since the SN is not aware of the PCell change, the SN has no idea whether the stored CPC configuration has been released or not on the UE side, which may cause the configuration mismatching between the SN side and the UE side. - After PCell change without SN involved, if the SN wants to modify the previous CPC configuration, the SN may send the RRCConnectionReconfiguration message including an updated triggerCondition but without condReconfigurationToApply or an updated condReconfigurationToApply but without triggerCondition to the UE. In such case, a reconfiguration failure may happen since the UE may consider the received CPC configuration as a new one but the triggerCondition and the condReconfigurationToApply may not be carried simultaneously, which shall cause the undesired recovery procedure (e.g. SCG failure information procedure or the RRC re-establishment procedure). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a note in 5.3.5.9.3 to specify that the UE does not consider the message as erroneous if triggerCondition and condReconfigurationToApply are not simultaneously included in an entry without the matching condReconfigurationId exists in the condReconfigurationList within the VarConditionalReconfiguration for CPC. And the UE ignores the entry. Impact analysis Impacted architecture options: LTE, (NG)EN-DC Impacted functionality: CHO Inter-operability: - If the network is implemented according to the CR and the UE is not, in case the SN sends a RRCConnectionReconfiguration message including an updated triggerCondition but without condReconfigurationToApply or an updated condReconfigurationToApply but without triggerCondition to the UE after PCell change, the UE might think the message as erroneous and then initiate the unnecessary SCG failure information procedure or RRC re-establishment procedure. - If the UE is implemented according to the CR and the network is not, there is no inter-operability problem. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The UE may think the received RRCConnectionReconfiguration message including an updated triggerCondition but without condReconfigurationToApply or an updated condReconfigurationToApply but without triggerCondition as erroneous and then initiate the unnecessary SCG failure information procedure or RRC re-establishment procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

5.3.5.9.3	Conditional reconfiguration addition/modification

The UE shall:

1>	for each condReconfigurationId included in the condReconfigurationToAddModList:

2>	if an entry with the matching condReconfigurationId exists in the condReconfigurationList within the VarConditionalReconfiguration:

3>	replace the entry with the values received for this condReconfigurationId;

3>	if the entry entry in condReconfigurationToAddModList includes a triggerCondition;

4>	replace the entry with the value received for this condReconfigurationId

3> else:

4>	keep the stored triggerCondition as the target candidate configuration for this condReconfigurationId;

3>	if the entry entry in condReconfigurationToAddModList includes an condReconfigurationToApply;

4>	replace the entry condReconfigurationToApply with the value received for this condReconfigurationId;

3> else:

4>	keep the stored condReconfigurationToApply as the target candidate configuration for this condReconfigurationId;

2>	else:

3>	add a new entry for this condReconfigurationId within the VarConditionalReconfiguration;

3>	store the associated RRCConnectionReconfiguration in VarConditionalReconfiguration;

NOTE:  The UE does not consider the message as erroneous if triggerCondition and condReconfigurationToApply are not simultaneously included in an entry without the matching condReconfigurationId exists in the condReconfigurationList within the VarConditionalReconfiguration for CPC. And the UE ignores the entry.

CHANGE END