3GPP TSG-RAN WG2 Meeting #111-e		R2-200xxxx

Electronic, 17th Aug. – 28th Aug. 2020

Source:	vivo

Title:	Summary of offline discussion #003: NR L1 Parameters

Agenda Item:	5.4.1.1

Document for:	Discussion and Decision

- Introduction

The document is to report the summary of the following email discussion:

- [AT111-e][003][NR15] L1 Parameters (vivo) Scope: Treat R2-2007057, R2-2007058, R2-2007504, R2-2006683, R2-2006995, R2-2006996 (proponents to drive) Part 1: Decision whether to make corrections, identify agreeable parts. Identify Controversial issues for on-line treatment (if any). Deadline: Aug 20, 0900 UTC. Part 2: For agreeable parts, continuation to agree CRs. Deadline: Aug 26, 0900 UTC.

Note: As per Chairman instruction, please kindly provide your company name and email address in the Annex at the end of this document. Thanks.

- Discussion

## Issue 1: Corrections on pathlossReferenceRSs

The contributions [1-2] indicate that the description of pathlossReferenceRSs is not correctly captured in TS38.331.

In section 6.3.2, the pathlossReferenceRSs is defined as optional IE with Need M.

pathlossReferenceRSs SEQUENCE (SIZE (1..maxNrofPUCCH-PathlossReferenceRSs)) OF PUCCH-PathlossReferenceRS OPTIONAL, -- Need M

However, the field description says “When the field is absent, the UE uses the SSB as reference signal (see TS 38.213 [13], clause 7.2)”. The original intention of this sentence should be for the case where the configuration is never received, UE uses the SSB as the reference signal. Thus, some correction is needed in field description to align with the meaning of Need M.

It was proposed to fix this issue as follows [1-2]:

- PUCCH-PowerControl field descriptions

pathlossReferenceRSs A set of Reference Signals (e.g. a CSI-RS config or a SS block) to be used for PUCCH pathloss estimation. Up to maxNrofPUCCH-PathlossReference-RSs may be configured. If the field is not configured, the UE uses the SSB as reference signal (see TS 38.213 [13], clause 7.2).

Q1: Companies are invited to provide their views whether they agree with the above issue 1 and the corresponding text proposal.

| Company | Yes/No | Comments, if any |
| --- | --- | --- |

## Issue 2: Correction on aperiodicSRS resource

The contributions [3-4] indicate that the description of aperiodicSRS resource is not correctly captured in TS38.331.

For the field aperiodicSRS-ResourceTriggerList, it is not just used for non-codebook based UL transmission (clause 6.1.1.2 in TS 38.214), but also used for UE sounding procedure for DL CSI acquisition (clause 6.2.1.2 in TS 38.214). Current reference to RAN1 specification (just clause 6.1.1.2) may cause misunderstanding that this parameter is only used for non-codebook based UL transmission.

Thus, it is proposed in [3-4], in the field description of IE aperiodicSRS-ResourceTriggerList, change the RAN1 reference clause to both non-codebook based UL transmission and DL CSI acquisition, as below:

SRS-ResourceSet field descriptions

aperiodicSRS-ResourceTriggerList An additional list of DCI "code points" upon which the UE shall transmit SRS according to this SRS resource set configuration (see TS 38.214 [19], clause 6). When the field is not included during a reconfiguration of SRS-ResourceSet of resourceType set to aperiodic, UE maintains this value based on the Need M; that is, this list is not considered as an extension of aperiodicSRS-ResourceTrigger for purpose of applying the general rule for extended list in clause 6.1.3.

Q2-1: Companies are invited to provide their views whether they agree with the above issue 2-1 and the corresponding text proposal.

| Company | Yes/No | Comments, if any |
| --- | --- | --- |

For the field slotOffset, current specification indicates that if the field of slotOffset is absent the UE applies no offset (value 0). Actually, according to RAN1 conclusion, only the UEs supporting zeroSlotOffsetAperiodicSRS, could be configured with value 0. Such restriction has not been captured in any RAN1 specification.

Thus, it was proposed in [3-4] to clarify that only the UEs supporting zeroSlotOffsetAperiodicSRS could be configured with value 0 in the field description of IE slotOffset, as below:

SRS-ResourceSet field descriptions

slotOffset An offset in number of slots between the triggering DCI and the actual transmission of this SRS-ResourceSet. If the field is absent the UE supporting zeroSlotOffsetAperiodicSRS applies no offset (value 0).

Q2-2: Companies are invited to provide their views whether they agree with the above issue 2-2 and the corresponding text proposal.

| Company | Yes/No | Comments, if any |
| --- | --- | --- |

## Issue 3: Correction on Cross Carrier Scheduling Configuration

The contributions [5-6] indicate that the description of Cross Carrier Scheduling Configuration is not correctly captured in TS38.331. In the section 6.3.2, there is a restriction for the IE searchSpace, i.e. “For a scheduled cell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent”.

However, the presence condition of the sub-field controlResourceSetId is SetupOnly which is described as following:

| SetupOnly | This field is mandatory present upon creation of a new SearchSpace. It is absent, Need M, otherwise. |
| --- | --- |

As per the presence condition description, the field controlResourceSetId should be present upon creation of new SearchSpace, which conflicts with the previously mentioned restriction. So if the searchSpace is configured for scheduled SCell in case of cross carrier scheduling, there is a confusion whehther the controlResourceSetId should be present or not.

It is also specified “If this IE is used for the scheduled cell in case of cross carrier scheduling, the fields other than searchSpacesToAddModList and searchSpacesToReleaseList are absent” for the PDCCH-Config. So it is clear that it is useless to configured the controlResourceSetId for a scheduled scell in case of cross carrier scheduling.

Same issue can be identified in the presence condition of Setup and Setup2 (only applied in Rel-16).

As the nrofCandidates should be configured no matter it is scheduling cell or scheduled scell, the contributions proposed to keep the current presence condition, but move it to a new presence condition Setup1.

Taking Rel-16 CR as an example, the proposed text is following [6]:

- –	SearchSpace

The IE SearchSpace defines how/where to search for PDCCH candidates. Each search space is associated with one ControlResourceSet. For a scheduled cell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent.

SearchSpace information element

-- ASN1START

-- TAG-SEARCHSPACE-START

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId                    ControlResourceSetId                                        OPTIONAL,   -- Cond SetupOnly

monitoringSlotPeriodicityAndOffset      CHOICE {

sl1                                     NULL,

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639),

sl1280                                  INTEGER (0..1279),

sl2560                                  INTEGER (0..2559)

}                                                                                                   OPTIONAL,   -- Cond Setup

duration                                INTEGER (2..2559)                                           OPTIONAL,   -- Need R

monitoringSymbolsWithinSlot             BIT STRING (SIZE (14))                                      OPTIONAL,   -- Cond Setup

nrofCandidates                          SEQUENCE {

aggregationLevel1                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel2                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel4                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel8                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel16                      ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}

}                                                                                                   OPTIONAL,   -- Cond Setup1

searchSpaceType                         CHOICE {

common                                  SEQUENCE {

dci-Format0-0-AndFormat1-0              SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-0                           SEQUENCE {

nrofCandidates-SFI                      SEQUENCE {

aggregationLevel1                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16                      ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-1                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-2                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-3                           SEQUENCE {

dummy1                                  ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20}  OPTIONAL,   -- Cond Setup

dummy2                                  ENUMERATED {n1, n2},

...

}                                                                                           OPTIONAL    -- Need R

},

ue-Specific                                 SEQUENCE {

dci-Formats                                 ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1},

...,

[[

dci-Formats-MT-r16                   ENUMERATED {formats2-5}                                OPTIONAL,    -- Need R

dci-FormatsSL-r16                    ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1, formats3-0, formats3-1,

formats3-0-And-3-1}                        OPTIONAL,    -- Need R

dci-FormatsExt-r16                   ENUMERATED {formats0-2-And-1-2, formats0-1-And-1-1And-0-2-And-1-2}

OPTIONAL     -- Need R

]]

}

}                                                                                                   OPTIONAL    -- Cond Setup2

}

SearchSpaceExt-r16 ::=                   SEQUENCE {

controlResourceSetId-r16                ControlResourceSetId-r16                                    OPTIONAL,   -- Cond SetupOnly2

searchSpaceType-r16                     SEQUENCE {

common-r16                              SEQUENCE {

dci-Format2-4-r16                       SEQUENCE {

nrofCandidates-CI-r16                   SEQUENCE {

aggregationLevel1-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16-r16                  ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-5-r16                      SEQUENCE {

nrofCandidates-IAB-r16                  SEQUENCE {

aggregationLevel1-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16-r16                  ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-6-r16                       SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

...

}

}                                                                                                   OPTIONAL,    -- Cond Setup3

searchSpaceGroupIdList-r16                      SEQUENCE (SIZE (1.. 2)) OF INTEGER (0..1)           OPTIONAL,    -- Need R

freqMonitorLocations-r16                        BIT STRING (SIZE (5))                               OPTIONAL     -- Need R

}

-- TAG-SEARCHSPACE-STOP

-- ASN1STOP

| Conditional Presence | Explanation |
| --- | --- |
| Setup | This field is mandatory present upon creation of a new SearchSpace except for scheduled cell in case of cross carrier scheduling. It is optionally present, Need M, otherwise. |
| Setup1 | This field is mandatory present upon creation of a new SearchSpace. It is optionally present, Need M, otherwise. |
| Setup2 | This field is mandatory present when a new SearchSpace is set up, if the same SearchSpace ID is not included in searchSpacesToAddModListExt-r16 of the parent IE with the field searchSpaceType-r16 included except for scheduled cell in case of cross carrier scheduling. Otherwise it is optionally present, Need M. |
| Setup3 | This field is mandatory present when a new SearchSpace is set up, if the same SearchSpace ID is not included in searchSpacesToAddModListExt (without suffix) of the parent IE with the field searchSpaceType (without suffix) included. Otherwise it is optionally present, Need M. |
| SetupOnly | This field is mandatory present upon creation of a new SearchSpace except for scheduled cell in case of cross carrier scheduling. It is absent, Need M, otherwise. |
| SetupOnly2 | In PDCCH-Config, the field is optionally present upon creation of a new SearchSpace and absent, Need M upon reconfiguration of an existing SearchSpace. In PDCCH-ConfigCommon, the field is absent. |

Q3: Companies are invited to provide their views whether they agree with the above issue 3 and the corresponding text proposals for Rel-15 and Rel-16.

| Company | Yes/No | Comments, if any |
| --- | --- | --- |

- Conclusions

TO BE ADDED.

- Reference

- R2-2007057	Clarification on the absence of pathlossReferenceRSs	Huawei, HiSilicon	CR	Rel-15	38.331	15.10.0	1773	-	F	NR_newRAT-Core

- R2-2007058	Clarification on the absence of pathlossReferenceRSs	Huawei, HiSilicon	CR	Rel-16	38.331	16.1.0	1774	-	A	NR_newRAT-Core

- R2-2007504	Correction on aperiodicSRS resource	vivo	CR	Rel-15	38.331	15.10.0	1851	-	F	NR_newRAT-Core

- R2-2006683	Correction on aperiodicSRS resource	vivo	CR	Rel-16	38.331	16.1.0	1729	-	A	NR_newRAT-Core

- R2-2006995	Correction on the Cross Carrier Scheduling Configuration	CATT	CR	Rel-15	38.331	15.10.0	1763	-	F	NR_newRAT-Core

- R2-2006996	Correction on the Cross Carrier Scheduling Configuration	CATT	CR	Rel-16	38.331	16.1.0	1764	-	F	NR_newRAT-Core

- Annex

| Company Name | Contact person, email address |
| --- | --- |
| vivo | Chenli, chenli5g@vivo.com |