3GPP TSG-RAN WG2 Meeting #112-e		R2-20xxxxx

Electronic, 02 November - 13 November 2020

Agenda item:	6.15

Source:	Apple

Title:	Summary on [AT112-e][023][R4 NR16] UL 7.5kHz Shift (Apple)

Document for:	Discussion

# 1   Introduction

This document is to capture offline discussions for the below.

[AT112-e][023][R4 NR16] UL 7.5kHz Shift (Apple)

Treat R2-2008740, R2-2009466, R2-2009467, R2-2009468, R2-2009469, R2-2009470, R2-2009471, R2-2009700, R2-2009701, R2-2010227

Intended outcome: Intermediate: Determine agreeable parts. Final: For agreeable parts, agreed CRs.

Deadline: Intermediate deadline(s) by Rapporteur, Final: Discussion stop at Wed Nov 11, 1200 UTC

# 2   Contact Information

| Company | Email |
| --- | --- |
| Apple (Yuqin Chen) | yuqin_chen@apple.com |
| Huawei, HiSilicon (Yang Zhao) | zhaoyang@huawei.com |
| Qualcomm Incorporated (Masato Kitazoe) | mkitazoe [at] qti.qualcomm.com |
| CATT (Erlin Zeng) | Erlin.zeng@catt.cn |
| Nokia (Amaanat) | amaanat.ali@nokia.com |
| Ericsson (Tony) | antonino.orsino@ericsson.com |
| MediaTek (Felix) | Chun-Fan.Tsai@mediatek.com |
| Intel (Youn) | Youn.hyoung.heo@intel.com |

# 3   Discussion

There were recently some discussions in RAN4 on UL 7.5kHz shifting especially on whether UE is mandatory to support it on NR TDD spectrums for dynamic spectrum sharing. This topic is in response to RAN4 LS R4-2011746 [1] which proposes that RAN4 understanding is if a UE does not support UL 7.5kHz shift for the given network configuration, the UE should avoid camping on this cell and consider this cell as barred.

R2-2009466 [2] presents three alternatives to introduce the access barring mechanism. R2-2009701 [3] proposes a way similar to the 3rd alternative in R2-2009466 [2]. While R2-2010227 [10] has a different view and thinks that there is no need to prevent UE camping in RAN2 specification for TDD 7.5kHz shift function for Rel-16 onwards UEs. And for Rel-15 UE which supports the TDD bands but not support 7.5kHz shift, R2-2010227 [10] proposes to rely on RAN4 spec [38.101] which specifies that “A UE that does not support it will be unable to communicate with a network that signals Δshift = 7.5 kHz.” and there is no need of RAN2 spec change to support UL 7.5kHz shift for TDD bands.

Q1: Should we change RAN2 spec to support the RAN4 agreement that if a UE does not support UL 7.5kHz shift for the given network configuration, the UE should avoid camping on this cell and consider this cell as barred?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Apple | Yes | As explained in our paper R2-2009466, the access barring procedure is needed in order to support: 1) legacy Rel-15 UE which does not implement the potential CR to be introduced; 2) UL shift is only mandatory for Band n40 from Rel-17, thus Rel-16 UE can still have UL 7.5kHz shift optional. |
| Huawei, HiSilicon | No | We understand RAN4 agreement is as follows: - For n38, it is mandatory to support 7.5KHz shift for 15KHz SCS; - For n38, 7.5KHz shift for 30KHz SCS is not supported. So in this case currently all the 7.5KHz support agreed by RAN4 so far is mandatory for UEs and we don’t see need to address the case that a UE does not support it. If in the future it appears optional support for 7.5KHz, we can discuss whether to have some mechanism in RAN2 but this can be discussed only when there is new agreement from RAN4 to optionally support 7.5KHz for some bands. |
| Qualcomm Incorporated | Yes | We agree to Huawei’s observation. So the requirement for the UE as suggested is only for the forward compatibility, i.e. to address future cases where 7.5kHz shift is introduced in some new configuration combination that is not supported by the standard today. |
| CATT | No | We agree with Huawei’s comments. |
| Nokia | No | We agree with Huawei’s comments. Furthermore, RAN4 has not agreed any optional UE capability for 7.5 kHz UL shift or requested any special handling in the RAN2 specifications. If in the future RAN4 considers optional UE support for 7.5 kHz UL shift in some case, RAN4 will then also request the corresponding UE capability from RAN2. Until then no UE capability or special handling in the RAN2 specs should be defined for 7.5 kHz UL shift UE support. |
| Ericsson | Maybe Yes | We do not have a strong preference on this, but if majority of companies believe it is helpful to clarify the UE behaviour mentioned in the RAN4 LS, we are okay to do it. We have the same understanding as QC that this change is suggested only for the forward compatibility. |
| MediaTek | Maybe Yes | We agree the observation from Huawei but also understand the change (if agreed) is for forward compatibility as commented by QC. |
| Intel | Not sure | We have the same understanding that RAN4 has not decided 7.5kHz for 30kHz SCS. So, the current discussion should focus on 7.5kHz for 15kHz SCS . However, even for that, it is not clear what RAN2 need to resolve. According to RAN4 spec, n38, n48, n90 needs 7.5kHz as mandatory. - n38: defined from Rel-15 - n48, n90: defined from Rel-16 (n90 is a new band of n41 to address legacy issue due to 7.5kHz) The legacy issue may happen for n38 band. However, RAN4 add the following NOTE for Rel15 spec (not for Rel-16 spec) NOTE: A UE operating n38 should support application of FREF, shift for UL transmissions. A UE that does not support it will be unable to communicate with a network that signals Δshift = 7.5 kHz. We are wondering if the NOTE is enough to indicate the consequence of not supporting mandatory feature or if RAN2 needs to address. If we go with the latter option, the change would be required for Rel-15 not Rel-16 because it is mandatory from Rel-16. However, based on RAN4 LS, it is not clear whether RAN4 requested to address this NOTE in RAN2 spec. So, it is desirable to ask for clarification. Regarding forward compatibility, we are not aware of what exact case RAN4 is referring to. So, it is hard to comment. |

Q2: Do you think we should introduce a solution to let legacy Rel-15 UE to properly bar the cell configured with UL 7.5kHz shift?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Apple | Yes | When RAN4 discussed the DSS problem for band n41, the main reason why they finally selected the solution to introduce a new band number n90 was exactly to not impact the legacy UE. Thus, we feel similar argument applies here and we prefer to have a backward compatible solution, i.e., let legacy Rel-15 UE to properly bar the cell configured with UL 7.5kHz shift. |
| Huawei, HiSilicon | No | To change Rel-15 is NBC, and would impact UEs which already support 7.5KHz shift. |
| Qualcomm Incorporated | Yes | This is sensible proposal in order to to address future cases where 7.5kHz shift is introduced in some new configuration combination that is not supported by the standard today. |
| CATT | No |  |
| Nokia | No | RAN4 did not request any special handling for the legacy Rel-15 UEs and the Rel-15 CR approved in RP-202093 in RAN#89 includes an informative note to reflect that some legacy devices may not support the feature and therefore such legacy UE are not able to communicate with a network that signals UL shift of 7.5 kHz. No special handling was requested by RAN#89 either. For future devices no optional capability has been agreed and therefore, nothing is needed for the future purposes. |
| Ericsson | No | According to current Rel-15 specification, even if the UE (that does not support the shift) will try to perform RACH towards the gNB, the RACH will fails and the UE will trigger RLF (or do cell reselection/stay in IDLE). From this point of view there is nothing broken, and we would like to not touch Rel-15. |
| MediaTek | No | We prefer not to change R15 SPEC. |
| Intel | Not yet | We need more information. |

Q3: If the answer to Q2 is Yes, which alternative is preferred?

- Approach 1: Alternative 1 in R2-2009466

- Approach 2: Alternative 2 in R2-2009466

- Approach 3: R2-2010983 (Only if the UE behaviour needs to be clarified)

| Company | Approach | Comments |
| --- | --- | --- |
| Apple | Approach 2 | We believe both Approach 1 and Approach 2 are feasible and prefer Approach 2 since it is cleaner and not impact the existing field frequencyShift7p5khz in SIB1. |
| Huawei, HiSilicon | None | See our response to Q2. |
| Qualcomm Incorporated | Approach 3 | Alternative 3 in R2-2009466 |
| CATT | None |  |
| Nokia | None | Nothing is needed as discussed in our earlier responses. |
| Ericsson | Approach 3 in R2-2010983 | This CR is only to clarify the UE behaviour according to what is stated in the RAN4 LS. No new capability is introduced. |
| MediaTek | None or Approach 3 in R2-2010983 | Similar view as Ericsson |
| Intel | None | We need more information. |

In R2-2009466 [2] and R2-2009700/9701 [8][9], UE capability is raised to support mobility case where UE is handed over from a band without UL 7.5kHz shift to a TDD band with UL 7.5kHz shift. Further, R2-2009466 [2] proposes to have a per SCS UE capability for future proof.

Q4: Do you agree that a corresponding UE capability for UL 7.5kHz shift is needed? If Yes, should we make it per SCS UE capability?

| Company | UE Capability on UL 7.5kHz shift? (Yes/No) | Per SCS UE capability? (Yes/No) | Comments |
| --- | --- | --- | --- |
| Apple | Yes | Yes (can consult with RAN4) | Even though the capability has no use during initial access, we can run into cases where the UE goes into connected state on one band where UL 7.5kHz shift is not applicable (where DSS is not possible) and afterwards NW can handover the UE to the cell which operates on the band with UL shift is configured. In order to do that, NW has to know if the UE actually supports the UL 7.5kHz shift. Secondly, it’s not clear to us whether 30kHz SCS would be applicable later with UL 7.5kHz SCS, thus we should better consult with RAN4 on whether a per SCS UE capability on UL 7.5kHz shift is required. |
| Huawei, HiSilicon | No | No | As we explained in Q2, currently 7.5KHz shift support is mandatory. There is no exceptional case defined in RAN4 to have optional support for 7.5KHz shift. So we should not introduce new UE capability now. |
| Qualcomm Incorporated | No | No | Same understanding as Huawei. Only thing we need to address at this moment is forward compatibility. |
| CATT | No | No | Agree with Huawei comment. |
| Nokia | No | No | As discussed in our earlier comments and we agree with Huawei’s comments |
| Ericsson | No | No |  |
| MediaTek | No | No |  |
| Intel | No | No | It is supported as mandatory features so far. |

Q5: Which Release to start the change in RAN2 spec? If it starts from Rel-16, should it be marked as early implementable?

| Company | Starting Release | Early implementable? (Yes/No) | Comments |
| --- | --- | --- | --- |
| Apple | Rel-16 | Yes |  |
| Huawei, HiSilicon | NA | No | See our response to Q4. |
| Qualcomm Incorporated | NA | NA |  |
| CATT | NA | No |  |
| Nokia | NA | No | As discussed in our earlier responses, nothing is needed to the RAN2 specs. |
| Ericsson | Rel-16 (only if we want to clarify the UE behaviour – capability is not needed) | No |  |
| MediaTek | Rel-16 | No |  |
| Intel | NA | No | We need more information. |

# 4	Conclusions

TBD

# 5	References

- R4-2011746  LS on clarification for the UE behaviour when UL 7.5kHz shift is optionally supported by a UE

- R2-2009466  Discussion on UL 7.5kHz shift in NR TDD bands   Apple

- R2-2009467  UL 7.5kHz shifting for NR TDD bands – Alt1    38.331 CR    Apple

- R2-2009468  UL 7.5kHz shifting for NR TDD bands – Alt 2    38.331 CR    Apple

- R2-2009469  UL 7.5kHz shifting for NR TDD bands – Alt 3     38.331 CR    Apple

- R2-2009470  UL 7.5kHz shifting for NR TDD bands     38.306 CR         Apple

- R2-2009471  Draft LS to RAN4 on UE capability for UL 7.5kHz shifting for NR TDD bands in DSS  Apple

R2-2009700	UE capability for UL 7.5KHz shift in NR TDD with 30KHz SCS	Ericsson	CR	Rel-16	38.306	16.2.0	0433	-	F	NR_n48_LTE_48_coex-Core

R2-2009701	UE behaviour 1when UL 7.5KHz shift is not supported	Ericsson	CR	Rel-16	38.331	16.2.0	2107	-	F	NR_n48_LTE_48_coex-Core

R2-2010227	Discussion on supporting 7.5KHz shift for TDD bands	Huawei, HiSilicon	discussion	Rel-16	NR_n48_LTE_48_coex-Core