3GPP TSG-RAN WG2 Meeting #112-e	R2-2011095

Online, 2 – 13 November 2020

Agenda item:	6.7.2

Source:	Nokia (RAN2 Vice-chair )

Title:	Summary of [AT112-e][216][MOB] Check CHO-related parts of R2-2010229 (RAN2 VC)

Document for:	Discussion and Decision

1	Brief scope of the contributions

This document contains the summary of email discussion [216] as per below:

[AT112-e][216][MOB] Check CHO-related parts of R2-2010229 (RAN2 VC)

Scope:

    - Check whether the CHO-related parts of R2-2010229 are agreeable.

Intended outcome:

    - Discussion summary in R2-2011095 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  2nd week Thu, UTC 15:00

    - Initial deadline (for rapporteur's summary in R2-2011095):  2nd week Fri, UTC 05:00

By Email [216]

Editorials:

R2-2010229	Support of Rel-16 features for SCG in EN-DC and NR-DC	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2192	-	F	NR_IAB-Core, NR_Mob_enh-Core

Moved from 6.1.3 to 6.2.4, then Moved from 6.2.4 to here

Main session requested to check the CHO-related parts

Offline [216]

Hence, there is only one document to check in this discussion, and only the CHO-related parts in the document.

2	CHO-related parts of R2-2010229

There are two changes aspects related to CHO in the CR R2-2010229:

- When MR-DC is released, CPC commands are also released autonomously by UE (as part of the MR-DC context)

- Clarifying that SCG CG information in CG-Config inter-node message can also contain the field conditionalReconfiguration (i.e. CPC command can be included in the SCG configuration to UE)

#### 5.3.5.10 MR-DC release The UE shall: 1> as a result of MR-DC release triggered by E-UTRA or NR: 2> release SRB3, if established, as specified in 5.3.5.6.2; 2> release measConfig associated with SCG; 2> if the UE is configured with NR SCG: 3> release the SCG configuration as specified in clause 5.3.5.4; 3> release otherConfig associated with the SCG; 3> stop timers T346a, T346b, T346c, T346d and T346e associated with the SCG, if running; 3> release bap-Config associated with the SCG, if configured; 3> release iab-IP-AddressConfigurationList associated with the SCG, if configured; 3> release conditionalConfiguration associated with the SCG, if configured;

Table 1: CHO-related change #1 in R2-2010229

scg-CellGroupConfig Contains the RRCReconfiguration message (containing only secondaryCellGroup and/or measConfig and/or otherConfig and/or bap-Config and/or iab-IP-AddressConfigurationList and/or conditionalReconfiguration): - to be sent to the UE, used upon SCG establishment or modification, as generated (entirely) by the (target) SgNB. In this case, the SN sets the RRCReconfiguration message in accordance with clause 6 e.g. regarding the "Need" or "Cond" statements. or - including the current SCG configuration of the UE, when provided in response to a query from MN, or in SN triggered SN change in order to enable delta signaling by the target SN. In this case, the SN sets the RRCReconfiguration message in accordance with clause 11.2.3. The field is absent if neither SCG (re)configuration nor SCG configuration query nor SN triggered SN change is performed, e.g. at inter-node capability/configuration coordination which does not result in SCG (re)configuration towards the UE. This field is not applicable in NE-DC.

Table 2: CHO-related change #2 in R2-2010229

Company comments are invited to each in the table below.

| Company | Is the change needed? (Yes/No) | Comments |
| --- | --- | --- |
| Intel | No | The first change in 5.3.5.10 on release of CPC configuration is not needed since it has been covered by 5.3.5.4. 5.3.5.4 Secondary cell group release The UE shall: 1> as a result of SCG release triggered by E-UTRA (i.e. (NG)EN-DC case) or NR (i.e. NR-DC case): 2> reset SCG MAC, if configured; 2> for each RLC bearer that is part of the SCG configuration: 3> perform RLC bearer release procedure as specified in 5.3.5.5.3; 2> release the SCG configuration; 2> if CPC was configured, 3> remove all the entries within VarConditionalReconfig, if any; Last change on CPC has been covered by Google’s CR in the email discussion 211. |
| OPPO | No | It seems to be already addressed by the step: 3> release the SCG configuration as specified in clause 5.3.5.4; |

Table 3. Comments to the change#1 in R2-2010229

| Company | Is the change needed? (Yes/No) | Comments |
| --- | --- | --- |
| OPPO | Yes |  |

Table 3. Comments to the change#2 in R2-2010229

3	Conclusions

Conclusions:

TBA

4	List of referenced documents

[1]	R2-2010229	Support of Rel-16 features for SCG in EN-DC and NR-DC	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2192	-	F	NR_IAB-Core, NR_Mob_enh-Core