3GPP TSG-RAN WG2 Meeting #113-e	R2-210xxxx

Online, 25 January–5 February 2021

# Agenda item:	5.3.1

# Source:	Samsung

# Title:	Report of [AT113-e][002][NR15] User Plane I (Samsung)

# Document for:	Discussion and Agreement

# 1	Introduction

This is to report the result of the following email discussion in RAN2#113-e Meeting [1].

[AT113-e][002][NR15] User Plane I (Samsung)

Scope: MAC Treat R2-2100206, R2-2100207, R2-2101510, R2-2101337, R2-2101769, R2-2101351, R2-2101593, R2-2101522, R2-2101523, R2-2101524, R2-2101525

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed CRs.

Deadline: Schedule A

# 2	Contact Information

| Company | Contact: Name (E-mail) |
| --- | --- |
| Samsung | Jaehyuk JANG (jack.jang@samsung.com) |

# 3	Discussion

## 3.1	Miscellaneous corrections

R2-2100206	Miscellaneous corrections	Samsung	CR	Rel-15	38.321	15.11.0	1003	-	F	NR_newRAT-Core

R2-2100207	Miscellaneous corrections	Samsung	CR	Rel-16	38.321	16.3.0	1004	-	A	NR_newRAT-Core

| Company | Agree as is; Agree with changes; Disagree | Detailed Comments |
| --- | --- | --- |
| Samsung | Agree as is (Rel-15) | Editorial corrections as indicated in the coversheet |

Conclusion:

TBD

## 3.2	CG and DRX Inactivity Timer

(The following four contributions are discussed together here.)

R2-2101510	Activation of CG and DRX Inactivity Timer	LG Electronics Inc.	discussion	Rel-15	NR_newRAT-Core

R2-2101337	Activation of CG and DRX Inactivity Timer	Ericsson	discussion	Rel-15	NR_newRAT-Core	R2-2010621

R2-2101769	Further discussions on DRX InactivityTimer operations	Huawei, HiSilicon	discussion	Rel-15	NR_newRAT-Core

R2-2101351	Activation of CG/SPS and DRX Inactivity Timer	Apple	discussion	Rel-15	NR_newRAT-Core, TEI15

The issue has been discussed for a long time, and rappoteur thinks that it would be difficult to change Rel-15 behavior anyway as UE vendors have different implementation from the discussion at the last meeting. If so, we can start the discussion for Rel-16 onwards, and the following options can be considered:

-	Option 1: Leave it to UE implemenation as in Rel-15
(which implies that network should not start drx-InactivityTimer, and no specification changes would be needed)

-	Option 2: UE (re-)starts drx-InactivityTimer when it receives PDCCH to activate DL SPS/UL CG type 2
(which implies that we could add a NOTE to clarify this)

-	Option 3: UE does not (re-)start drx-InactivityTimer when it receives PDCCH to activate DL SPS/UL CG type 2
(which implies that we could add a NOTE to clarify this)

-	Option 4: …

| Company | Which option do you prefer? | Detailed Comments |
| --- | --- | --- |
| Samsung | Option 1 or 2 | For Rel-15, we can leave it to UE implementation to avoid any impact to UEs in the field. For Rel-16, we still prefer to have the same behaviour as in LTE (i.e. to Option 2), but can go with Option 1. Regarding drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL, drx-RetransmissionTimerDL, and drx-RetransmissionTimerUL in R2-2101510, we can rely on the behaviours in the existing text anyway (i.e. under 'if a MAC PDU is received/transmitted…'), and thus no specification changes would be needed. |

Conclusion:

TBD

## 3.3	CG Type 1 upon TA expired

(The following five contributions are discussed together here.)

R2-2101593	Discussion on the handling of CG type 1 resources when TA timer is expired	ZTE Corporation, Sanechips	discussion	Rel-15	NR_newRAT-Core

R2-2101522	CR on CG type 1 resources handling when timeAlignmentTimer is expired-Opt 1	ZTE Corporation, Sanechips	CR	Rel-15	38.321	15.11.0	1038	-	F	NR_newRAT-Core

R2-2101523	CR on CG type 1 resources handling when timeAlignmentTimer is expired-Opt 2	ZTE Corporation, Sanechips	CR	Rel-15	38.321	15.11.0	1039	-	F	NR_newRAT-Core

R2-2101524	CR on CG type 1 resources handling when timeAlignmentTimer is expired-Opt 1	ZTE Corporation, Sanechips	CR	Rel-16	38.321	16.3.0	1040	-	F	NR_newRAT-Core

R2-2101525	CR on CG type 1 resources handling when timeAlignmentTimer is expired-Opt 2	ZTE Corporation, Sanechips	CR	Rel-16	38.321	16.3.0	1041	-	F	NR_newRAT-Core

For your convenience, proposals in R2-2101593 are copied below:

Proposal 1: RAN2 confirm that the RRC configuration for type 1 configured grant will not be released in case the timeAlignmentTimer expires (i.e. delta configuration is allowed. e.g. for pusch-RepTypeIndicator-r16). Proposal 2: RAN2 confirm that, after the expiration of timeAlignmentTimer, the type 1 configured grant will not become available unless the type 1 configured grant is initialized again (i.e. will not become available automatically after the start of timeAlignmentTimer ). Proposal 3:RAN2 confirm that, after the expiration of timeAlignmentTimer, the type 1 configured grant will become unavailable unless a new RRC configuration for type 1 configured grant is received (i.e. although the RRC configuration for type 1 configured grant is not released, RRC configuration for type 1 configured grant should be included in RRC signaling to enable the type 1 configured grant). Proposal 4: If companies have the same understanding on the proposals 1-3, then the specs are clear. Otherwise, a CR is needed to clarify the expected behaviour on the handling of type1 configured grant in case the timeAlignmentTimer expires.

| Company | Do you agree with proposals 1 to 3 in R2-2101593? Do you support either Option 1 in R2-2101522 or Option 2 in R2-2101523? | Detailed Comments |
| --- | --- | --- |
| Samsung | No; Option 1 partially (clear CG type 2 only) | We think that CG type 1 is not released upon expiry of TAT, and UE resumes CG type 1 without (re-)initializing it upon TAT becomes running, as in the current specification. For the actual changes, RAN2 may consider updating the specification from Rel-15 that UE clears configured uplink grants of CG type 2 upon expiry of TAT. |

Conclusion:

TBD

# 4	Conclusion

TBD

# 5	References

[1]	RAN2 113-e Chairman Notes 2021-01-25 0900 UTC