3GPP TSG-RAN WG2 Meeting #113 bis electronic	R2-2102919

Online, April 12 – April 20, 2021

Source:	CATT

Title:	UE sub-grouping mechanism with Paging Enhancement

Agenda Item:	8.9.2

Document for:	Discussion and Decision

Introduction

In RAN2#113-e, the following conclusions were made:

There is support to have UE ID based enhancement There is still significant interest to have other additional methods (but also some concerns). The approach to have a single mechanism that can take several aspects into account can be a way forward. There are still questions on the details, e.g. whether CN or RAN would provide a parameter.

In addition, RAN1 sent a reply LS on paging enhancement [2]. In this contribution, we would like to give our considerations on UE sub-grouping mechanism.

Discussion

Based on the RAN1 reply LS [2], RAN1 agreed:

Carrying UE subgroups information is considered in physical layer design for paging enhancement - Note: The number of UE subgroups RAN1 has evaluated ranges from 2 to 16 for a PO

From RAN2 point of view, we would like to confirm that UE sub-grouping paging enhancements do not add more POs on top of the legacy POs, but rather divide the UEs in a PO in further sub-groups. We think this is worth clarifying.

Proposal 1: The R17 UE sub-grouping paging enhancements do not add more POs on top of legacy POs but divide the UEs in a PO in further sub-groups.

- UE sub-grouping method

We now address the UE sub-grouping method.

During last RAN2#113 e-meeting, we focused on the following two UE sub-grouping mechanisms:

Option 1: Sub-grouping based on UE-ID;

Option 2: Network configures sub-grouping ID.

Option 2 is flexible. Network can configure sub-grouping ID to the UE based on network implementation, e.g. considering the paging probability, power consumption profile and so on. But there are some related issues as follows:

- Introduce additional exchange between CN and RAN nodes. It is not clear which node will decide sub-grouping ID of a UE. If sub-grouping ID is decided by CN, it needs to be sent from CN to RAN nodes for both CN paging and RAN paging, and be exchanged between RAN nodes for RAN paging. If sub-grouping ID is decided by the RAN node which releases or suspends RRC connection between RAN and the UE, sub-grouping ID also needs to be exchanged between the CN and RAN nodes for CN paging, and be exchanged/harmonized between RAN nodes for RAN paging.

- It is hard to ensure how much power saving gain sub-grouping ID can be achieved. No matter sub-grouping ID is decided by CN or RAN node which releases or suspends RRC connection between RAN and the UE, the CN or RAN node cannot know which UEs will belong to the same PO as the monitored PO by the UE is decided by UE ID and paging configuration of current camped cell.

Observation 1: The network-configured paging sub-grouping method, although flexible, brings a lot of complexity for unclear power saving benefit (compared with UE-ID method).

Based on the RAN1 LS as follows, we can observe that, compared with paging early indication without UE sub-grouping, the additional power saving gain with UE sub-grouping indication carried within paging early indication is not much.

Agreements: Observation: For NR idle/inactive-mode UEs, UE sub-grouping indication carried in paging early indication can provide the following power saving gains w.r.t Rel-16: - If the original group paging rate is 10%: - [10.6%] –[19.1%] where the baseline assumes 1 SS burst for synchronization before PO reception - [16.0%] –[36.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [14.3%] –[46.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception - Some sources also evaluated performance if the original group paging rate is in the range between 20% and 60% and showed following results: - [8.0%] –[19.1%] where the baseline assumes 1 SS burst for synchronization before PO reception - [18.1%] –[34.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [20.6%] –[42.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception The additional power saving gains w.r.t. paging early indication without UE sub-grouping are given as follows: - If the original group paging rate is 10%: - [0.6%] –[2.7%] where the baseline assumes 1 SS burst for synchronization before PO reception - [0.6%] –[4.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [0.6%] –[4.7%] where the baseline assumes 3 SS bursts for synchronization before PO reception - Some sources also evaluated performance if the original group paging rate is in the range between 20% and 60% and showed following results: - [1.3%] –[8.0%] where the baseline assumes 1 SS burst for synchronization before PO reception - [2.1%] –[13.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [3.3%] –[16.1%] where the baseline assumes 3 SS bursts for synchronization before PO reception The number of UE sub-groups evaluated ranges from 2 to 16. The power saving gains are dependent on the assumptions about placement of PEI and PO relative to SSB. Note: It is FFS in RAN1 another group paging rate > 10% for the evaluation of Rel-17 paging enhancement. Note: Not all sources providing results for paging early indication without UE sub-grouping also provide results for paging early indication with UE sub-grouping

Observation 2: Compared with paging early indication without UE sub-grouping, the additional power saving gain with UE sub-grouping indication carried within paging early indication is limited.

Considering the limited additional power saving gain introduced by UE sub-grouping mechanism w.r.t paging early indication without UE sub-grouping, we prefer to adopt a simple UE sub-grouping mechanism and avoid the impact on other groups as much as possible. Hence, we prefer to consider sub-grouping based on UE-ID only in R17.

Proposal 2: Only UE-ID based sub-grouping is considered in R17.

- UE ID based sub-grouping details

Getting to the next level of details, the UE sub-grouping mechanism based on UE ID could be:

- The UE sub-group ID is determined by:

UE sub-group ID=floor (UE_ID/(N*Ns)) mod X;

UE_ID: 5G-S-TMSI mod 1024;

N: number of total paging frames in T, T is DRX cycle of the UE;

Ns: number of paging occasions for a PF;

X: The total number of UE sub-grouping.

Proposal 3: The sub-grouping paging mechanism based on UE-ID is:

- The UE sub-group ID is determined by:

UE sub-group ID=floor (UE_ID/(N*Ns)) mod X;

UE_ID: 5G-S-TMSI mod 1024;

N: number of total paging frames in T, T is DRX cycle of the UE;

Ns: number of paging occasions for a PF;

X: The total number of UE sub-grouping.

- Sub-grouping indication

With the Paging Early Indication (PEI), the UE monitors the PEI before the PO to check if it needs to wake-up and receive the PDCCH in the PO.

RAN1 is considering two options for sub-grouping indication:

- The sub-grouping is indicated in the PEI ahead of the PO

- The sub-grouping is indicated in the PDCCH of the PO

In the former case, only the UEs in the indicated subgroup need to wake-up. And in the latter case, if PEI indicates the next PO will receive paging, all UEs wake up and read the PDCCH to know which sub-group is concerned by the paging, and only the UEs in the indicated subgroup continue with the PDSCH reception. On the other hand, when the paging is for a SI modification or an Emergency Message reception, all subgroups must receive the PDCCH. So when sub-grouping is indicated in the PEI, it must explicitly indicate all subgroups, whereas if it is indicated in the PO’s PDCCH, the presence of the SI modification or an Emergency Message notification bits can implicitly indicate all subgroups. Therefore, there are balanced pros/cons for the two above options from RAN2 perspective and so we prefer leaving the final design to RAN1.

Proposal 4: All sub-groups are targeted when the paging indicates a SI modification or an Emergency Message reception.

Proposal 5: Leave it to RAN1 to decide which signal/channel carries the subgroup indication.

Conclusion

In this contribution, we give our considerations on UE grouping mechanism with paging enhancements based on RAN1’s LS. We observe and propose:

Proposal 1: The R17 UE sub-grouping paging enhancements do not add more POs on top of legacy POs but divide the UEs in a PO in further sub-groups.

Observation 1: The network-configured paging sub-grouping method, although flexible, brings a lot of complexity for unclear power saving benefit (compared with UE-ID method).

Proposal 2: Only UE-ID based sub-grouping is considered in R17.

Proposal 3: The sub-grouping paging mechanism based on UE-ID is:

- The UE sub-group ID is determined by:

UE sub-group ID=floor (UE_ID/(N*Ns)) mod X;

UE_ID: 5G-S-TMSI mod 1024;

N: number of total paging frames in T, T is DRX cycle of the UE;

Ns: number of paging occasions for a PF;

X: The total number of UE sub-grouping.

Proposal 4: All sub-groups are targeted when the paging indicates a SI modification or an Emergency Message reception.

Proposal 5: Leave it to RAN1 to decide which signal/channel carries the subgroup indication.

Reference

- R1-2009801, LS on Paging Enhancement, MediaTek, RAN2#103e

- R1-2102136, Reply LS on Paging Enhancement, MediaTek, RAN2#104e