3GPP TSG-RAN WG2 Meeting #113 bis electronic                      	             R2-2103165

Online, April 12 – April 20, 2021

Agenda Item:	8.13.2.2

Source: 	vivo

Title:         	Discussion on signalling model of 2-step RACH report

Document for: 	Discussion and Decision

- Introduction

RAN2#113-e meeting [1] discussed 2-step related SON such as the the reporting granularity of an indicator relevant to DL beam quality, and left two FFS on the fallback/RA type indications.

Agreements 1 The reporting granularity of whether the DL beam quality, associated to the used 2 step RA resource, is above or below the msgA-RSRP-ThresholdSSB is per-RA-attempt. 2 The RA report includes an indication that enables the network to know that the fallback from 2 step RA to 4 step RA was performed by the UE. FFS: Implicit vs explicit indication. 3 Choose ‘per RA procedure’ for the granularity of RA type (2 step RA vs 4 step RA) indication. FFS: Implicit vs explicit indication.

These issues were further investigated in the post email discussion [2], the email rapporteur in the end proposed a series of potential agreements based on the convergent views from companies.

However, the signalling model of 2-step RACH report has not been touched during the offline, so we expect to discuss the structure design in this paper.

- Discussion

Basically all the information that should be included in the 2-step RA report has been gone through over the last few meetings, and yet it should be high time that we focus on the signalling design of 2-step RA report.

The structure of the legacy RA report is shown below:

Figure 1. Signalling structure of the legacy RA report

There could be at most 8 RA reports recorded, currently the RA report only denotes the single RA type (4-step RA), and the detailed information about each of the random access attempts in the chronological order of the random access attempts are provided in perRAInfoList.

As 2-step RA could fall back/switch to 4-step RA within a single RA procedure, thus the RA attempts should be included in the same RA report. However, the frequency-related parameters for 4-step RA are different from the information for 2-step RA (e.g., msg1-FrequencyStart-r16 vs. msgA-FrequencyStart-r17), so with the introduction of 2-step RA report, we can either try to a) enhance the legacy field ra-InformationCommon to include 2-step RA related frequency parameters, or to b) adopt a new field ra-InformationCommon-2step to include all 2-step specific information. To be more specific, the structure of these two signalling models are given below:

a) enhance the legacy field ra-InformationCommon b) adopt a new field ra-InformationCommon-2step

Figure 2. Signalling structure of 2-step RA report

For option a), once UE falls back/switches to 4-step RA, the 4-step related frequency parameters will be recorded and the index for PerRAInfo can be counted consistently; instead, option b) would create a new field ra-InformationCommon to collect the information for 4-step RA attempts, where the index for PerRAInfo starts from 0 again. Though both options can work well, our preference is option b). Nevertheless, this can be further discussed by RAN2, therefore we propose:

RAN2 to choose one of the following options as the signalling model of 2-step RA report:

- enhance the legacy field ra-InformationCommon to include 2-step RA related frequency parameters;

- adopt a new field ra-InformationCommon-2step to include all 2-step specific information.

- Conclusion

In this paper, the following observations and proposal are given:

Proposal 1	      RAN2 to choose one of the following options as the signalling model of 2-step RA report:

a) enhance the legacy field ra-InformationCommon to include 2-step RA related frequency parameters;

b) adopt a new field ra-InformationCommon-2step to include all 2-step specific information.

- References

- R2-2101956, Report from SON/MDT session, 3GPP TSG-RAN WG2 Meeting #113 electronic Online, January, 2021

- [Post113-e][852][NR17 SONMDT]  2-s RA and other SON changes (CATT).