3GPP TSG-RAN WG2 #113bis electronic	R2-2103298

Online, April 12 – April 20, 2021

Source:		NEC

Agenda item:		8.13.2.3   Other WID related SON features

Title:		Discuss on signalling aspects of successful handover report

Document for:		Discussion and decision

# 1. Introduction

The concept and general idea of successful handover report is introduced in the study phase of NR SON [1], however not specified as Rel-16 SON feature due to lack of time.

In Rel-17 SON/MDT WI, RAN2 has agreed to introduce successful handover report. And in the last meeting, the following agreement related was made [2]:

Agreements:

Contents of the HO success report:

The source cell and target cell related identifiers and measurements are to be included in the successful HO report.

The triggering condition and content of successful handover report was discussed in [Post113-e][851].In this paper, we mainly discuss on the signalling and procedure aspects of successful handover report.

# 2. Discussion

For handover failure report carried in RLF-report, no configuration on whether to support is sent from the network, and the UE shall always collect information for handover failure event, and report to network at any possible chance as it is a mandatory feature for Rel-16 UE. However, for successful handover report (SHR), to avoid unnecessary collecting and reporting, whether the SHR information is collected should be under network control, which means that the network can configure the support of SHR only when the gNB needs such information. Additionally, to avoid too much SHR information, triggering conditions to compile the SHR are defined, hence the report would be triggered only if the conditions are met. And network can configure the interested triggering conditions to the UE.

As the handover is initiated by the source gNB, the SHR information is useful for the source gNB to check and enhance the imperfect triggering timing or configuration for one successful handover event. In this sense, whether to perform successful handover report should be configured by the source gNB. With regarding to requirement from target gNB for RACH configuration enhancement for one handover procedure, as RACH report can already enable that, there is no need for the target node to configure SHR for the UE.

Proposal 1. The source gNB configures successful handover report for the UE.

To configure the SHR, one option could be by broadcasting in system information. In this way, all UEs performing successful handover initiated in the cell shall apply the SHR configuration. As an alternative, the SHR can be configured per UE by RRC message dedicatedly. For example, the SHR is configured in RRC message for handover (RRCReconfiguration with reconfigurationWithSync), in this way, the SHR is configured per UE per handover event. As the handover command is generated by the target node, the source node needs to provide SHR configuration to the target node during handover preparation. To avoid inter-node interworking, another approach for dedicate SHR configuration is by RRCReconfiguration message before handover.

Proposal 2: RAN2 discuss on the options to configure successful handover report:

- Option 1. By system information

- Option 2. By handover command

- Option 3. By RRC message before handover

One typical procedure for SHR could be, upon receiving RRCReconfiguration with reconfigurationWithSync, the UE configured with SHR checks if the triggering conditions (e.g. running of T310/T312) for SHR are fulfilled, stores the information (e.g. T310/T312 value and measurement results) if at least one of the triggering conditions is met, and generates RRCReconfigurationComplete message with availability indication for the SHR. The target node requests for the SHR by UEInformationRequest from the UE. The UE discards the stored SHR information once it is reported to the network by UEInformationResponse. Otherwise, the stored SHR information is discard 48 hours since it is stored.

However, when the SHR is stored, the UE has no idea of whether the handover is successful or not, as the random access procedure is not completed yet. If the handover fails finally, the UE shall discard the stored SHR information for the corresponding handover event.

Proposal 3: Upon T304 expiry during a HO, the UE shall discard the stored successful handover report information related to this HO.

During Rel-16, there are some new features have impact on handover procedure. For example CHO, DAPS are introduced for mobility enhancement to improve handover reliability and service interruption. Additionally, in fast MCG recovery, MCG RLF is can also be recovered by sending handover message to UE through the SCG link. To support CHO and DAPS for SHR, there was some initial discussion on the scenarios, triggering conditions and content of the SHR in [Post113-e][851].

Here, we want to address one signalling issue of SHR for DAPS. For SHR concerning DAPS handover, one case is UE successfully performed a DAPS HO towards the target cell, however RLF is experienced in the source cell while performing DAPS. For this case, the source RLF usually happen after RRCReconfigureComplete message has been generated, so the question is how to report the availability of SHR to the network. One option is no special handling for this case, thus the SHR availability is not reported to the target cell, and will be reported to the network after further handover or (re)establish to a new cell. However, this will introduce delay for SHR report. The second option could be to report the SHR availability in RRC message (e.g. UEAssistanceInformation) after DAPS complete. The third option is to report SHR availability in RRCReconfigurationComplete message in response to the RRCReconfiguration message to release source. We think the third option is the best as compared to option 2, the IE in RRCReconfiguraitonComplete message can be reused, and there is no such delay as in option 1.

Proposal 4: The successful handover report availability indicator of DAPS handover can be sent in RRCReconfigurationComplete message in response to the RRCReconfiguration message to release the source.

# 3. Conclusion

In this contribution, we provide our consideration on the support of successful handover report, and have the following proposals:

Proposal 1. The source gNB configures successful handover report for the UE.

Proposal 2: RAN2 discuss on the options to configure successful handover report:

- Option 1. By system information

- Option 2. By handover command

- Option 3. By RRC message before handover

Proposal 3: Upon T304 expiry during a HO, the UE shall discard the stored successful handover report information related to this HO.

Proposal 4: The successful handover report availability indicator of DAPS handover can be sent in RRCReconfigurationComplete message in response to the RRCReconfiguration message to release the source.

# Reference

[1] TR 37.816 v1.0.0, Study on RAN-centric data collection and utilization.

[2] Meeting report of RAN2 #113e.