3GPP TSG-RAN WG2 #113bis-e	Tdoc R2-2103646

Electronic meeting, 2021-04-12 – 2021-04-20

Agenda Item:	8.8.2

Source:	Ericsson

Title:	On solution for RAN slicing enhancement

Document for:	Discussion, Decision

# 1	Introduction

RAN#91e agreed a New WID on enhancement of RAN Slicing for NR

In this document, we examine a possible solution for the WI objective 1.a, while the principle that all cells in a TA support the same set of slices is maintained.

- To assist cell reselection, broadcast the supported slice info of the current cell and neighbour cells, and cell reselection priority per slice in system information message.

# 2	Discussion

## 2.1 Solution outline for slice-based cell re-selection

According to existing slicing mechanisms, a UE indicates its wish to communicate via certain slice in NAS registration procedure. If access to the slice is granted by the CN, the S-NSSAI is listed in “Allowed NSSAI”. This is valid until next time the UE triggers a NAS registration, typically at RA border. This means that UE expects that the list of slices in “Allowed NSSAI” is available for access in all cells that broadcast a TAI of the TA list that UE receives in the NAS registration response.

As consequence, there is no need to introduce slice information in SIB for a (camped) cell for this purpose.

There is no need to introduce new slice information in SIB to indicate slice availability for a cell, this is already indicated by the TAI (TAC + PLMN id in SIB1).

To satisfy the WI objective on assisting cell re-selection, slice information (and cell re-selection priority) can be introduced per neighbouring frequency (inter-frequency) in SIB. This information can be used by network operator to steer the UE to camp on a cell on a frequency that the network operator prefers that UE shall use to access the slice. A UE that at some time intends to access a slice can prepare this by following inter-frequency  slice information in SIB, and re-select to cell on other frequency (if in coverage). Otherwise UE will remain camped on the first cell.

With existing mechanisms, it is possible to handover or redirect the UE to cell on other frequency, provided network knows UE is in coverage of cell on the other frequency (by UE measurements or overlapping coverage). The network can (already when UE performs NAS registration) provide the UE with dedicated frequency priorities (in RRCRelease) to steer the UE cell re-selection. This camping preference remains until next time UE enters (or rather leaves) RRC_Connected, e.g. next time UE performs NAS registration (or RAU in RRC_Inactive).

Advantages with a SIB solution where during the SI phase claimed as e.g. :

- Delay due to measurements/re-direct/handover is avoided (size of the delay was never quantified in the system study)

- Network processing for redirect/handover can be avoided

- Network preferences for UE cell/frequency re-selection and camping may vary within the registration area

Some companies also claimed these gains are not enough motivation for change.

Providing slice information in SIB per inter-frequency gives some advantages as compared to existing mechanisms based on network handover/re-direct and dedicated frequency priorities in RRCRelease.

RAN2 to discuss if solution outlined above can be used as baseline solution for this WI.

## 2.2 Further details on the solution

Despite that the solution outlined in previous section aligns with existing principles on that the same set of slices are available in all cells of a TA/RA, there are several parts of the solution that need not only RAN2 discussion, but coordination with other groups (and likely need to be covered in specifications by other groups).

In TR 38.832 the following is stated:

“The slice info may be: providing only SST, on-demand SIB, SIB segmentation, slice grouping (if any), or slice associated UAC information where other solutions are not precluded. Details can be discussed in WI phase.”

Using only the SST would not be enough as it would not give the sufficient flexibility to differentiate different requirements for network slices. There may also be similar security/privacy concern as for broadcasting S-NSSAI (SA3 issue). “On-demand SIB” and “SIB segmentation” are existing mechanisms to handle SIB payload size issues but are not own solutions of slice information format. Slice association to UAC information could be possible, but the slice information in UAC is limited in size and the UAC grouping is for a different purpose (to handle congestion situations) rather than steering UE cell camping to cells/frequencies. In our view a “slice grouping” is the only feasible candidate. A straightforward solution to allocate such “slice grouping” would be in the NAS registration procedure.

Furthermore, in existing solution based on dedicated signalling the operator preference is reflected by RFSP and Allowed NSSAI. RAN2 and other groups need to study how this principle is maintained with new slice groupings and priorities in SIB inter-frequency information. We expect that the need for inter-WG coordination is anyway smaller if the principle that all cells in a TA support the same set of slices is maintained.

RAN2 to discuss further details on the solution and involve other groups when needed, e.g. on the content and format of “slice information” and “priority” in SIB Inter-frequency information, as well as in dedicated signalling (RRCRelease).

# Conclusion

In the previous sections we made the following observations:

Observation 1	There is no need to introduce new slice information in SIB to indicate slice availability for a cell, this is already indicated by the TAI (TAC + PLMN id in SIB1).

Observation 2	Providing slice information in SIB per inter-frequency gives some advantages as compared to existing mechanisms based on network handover/re-direct and dedicated frequency priorities in RRCRelease.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss if solution outlined above can be used as baseline solution for this WI.

Proposal 2	RAN2 to discuss further details on the solution and involve other groups when needed, e.g. on the content and format of “slice information” and “priority” in SIB Inter-frequency information, as well as in dedicated signalling (RRCRelease).

# References

New WID on enhancement of RAN Slicing for NR, RP-210912

- [2]	TR 38.832, Study on enhancement of Radio Access Network (RAN) slicing, v1.0.0