3GPP TSG RAN WG2 Meeting #113bis-e                                                        R2-2103648

e-Meeting, 12th – 20th April, 2021

Agenda item:	8.5.2

Source:	Samsung

Title:	CG Harmonization for UCE

Document for:	Discussion and Decision

1	Introduction

In RAN2#113-e meeting, RAN2 made the following agreements:

Agreements:

- LCH based prioritization and cg-RetransmissionTimer can be configured together in Rel-17 (consensus)

- Option 1: AutoTx and CGRT are responsible for deprioritized MAC PDU and LBT-failed MAC PDU, respectively.  If CGRT is not configured, LBT-failed MAC PDU is not retransmitted. If AutoTx is not configured, deprioritized MAC PDU is not retransmitted.

- the MAC entity stops cg-RetransmissionTimer when the CG resource associated with the timer is deprioritized due to LCH-based prioritization.

- FFS With cg-RetransmissionTimer and LCH-based prioritization configured, the MAC entity can prioritize between initial transmissions and retransmissions on a CG based on priority of multiplexed LCH(s) -or to be multiplexed

- LBT failure is not considered when determining a grant priority for intra-UE prioritization (17/22)

- Configuring a subset of HARQ processes as “restricted processes” for transmission of data from higher priority LCHs is not supported (18/22)

- Enhancements for handling conflicting DG-CG transmissions of the same HARQ process are not supported (18/22)

This document discusses remaining issues on CG harmonization.

2	Discussion

In RAN2#113-e meeting, it was agreed to stop CGRT when a CG resource with CGRT (cg-RetransmissionTimer) is de-prioritized. The reason is to support fast Autonomous Transmission (autonomousTx) without waiting for the expiry. For the same reason, it was agreed during Rel-16 IIOT discussion that MAC behaviour is that CGT (configuredGrantTimer) is stopped when CG resource configured with autonomousTx is de-prioritized, and the MAC specification has been implemented accordingly. However, if the de-prioritized CG is not configured with autonomousTx, CGT is not stopped at all, in order to give sufficient time to gNB to allocate the retransmission resource. In that case, the MAC entity shall not stop CGRT. Referring to TS 38.321-g40, if CGRT is configured and not running and CGT is running, Autonomous Retransmission will occur due to de-prioritization by applying the green highlighted text as follows:

2> else if the cg-RetransmissionTimer for the corresponding HARQ process is configured and not running, then for the corresponding HARQ process: 3> if the configuredGrantTimer is not running, and the HARQ process is not pending (i.e. new transmission): 4> consider the NDI bit to have been toggled; 4> deliver the configured uplink grant and the associated HARQ information to the HARQ entity. 3> else if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant (i.e. retransmission on configured grant): 4> deliver the configured uplink grant and the associated HARQ information to the HARQ entity

This means that stopping cg-RetransmissionTimer mandates Autonomous Retransmission of the de-prioritized CG. It is not aligned with RAN2’s latest agreement: AutoTx and CGRT are responsible for deprioritized MAC PDU and LBT-failed MAC PDU, respectively. Thus, we apply the same rule for CGRT, i.e. the timer is stopped when CG configured with AutonomousTx is de-prioritized.

Proposal 1. cg-RetransmissionTimer is stopped when CG configured with AutonomousTx is de-prioritized. cg-RetransmissionTimer is NOT stopped when CG NOT configured with AutonomousTx is de-prioritized.

One difference between Autonomous Retransmission in NR-U and Autonomous Transmission in IIOT is that Autonomous Transmission occurs within a CG configuration whereas Autonomous Retransmission may occur across different CG configurations. Thus, the mixture of both mechanism requires CG configuration handling in case of the harmonization. The simplest way to avoid the confusion would be not to allow HARQ process sharing between CG configurations when LCH-based Prioritization is configured.

Proposal 2. When LCH-based Prioritization is configured, HARQ process is not shared between CG configurations.

A criterion to perform Autonomous Retransmission is whether the corresponding HARQ process is pending. The concept of pending or not pending HARQ process is used and introduced for only NR-U. More specifically, when CGT is not running and the HARQ process is not pending, Autonomous Retransmission will not occur. TS 38.321 defines three cases that HARQ process is considered as not pending as follows:

When cg-RetransmissionTimer is configured and the HARQ entity obtains a MAC PDU to transmit and LBT failure indication is received from lower layer, the corresponding HARQ process is considered to be pending. For a configured uplink grant, configured with cg-RetransmissionTimer, each associated HARQ process is considered as not pending when: - a transmission is performed on that HARQ process and LBT failure indication is not received from lower layers; or - the configured uplink grant is initialised and this HARQ process is not associated with another active configured uplink grant; or - the HARQ buffer for this HARQ process is flushed.

However, a missing part is how to handle a de-prioritized CG transmission in unlicensed band as Rel-16 NR-U did not consider the LCH-based Prioritization. In RAN2#113-e, RAN2 agreed that a MAC PDU for a de-prioritized CG will be transmitted by Autonomous Transmission. Thus, the existing Rel-16 text is not aligned with the agreement. We can consider the following scenario

:

- At t1: according to the specification, CG is no transmitted due to LBT failure  HP is considered as "pending" and Autonomous Retransmission is performed as defined in NR-U.

- At t2: CG for Autonomous Retransmission is de-prioritized but whether it is pending is not defined. The latest HARQ process status is “pending”, Autonomous Transmission should be performed at t3. This contradicts the RAN2 agreement for Rel-17.

A simple way would be that a HARQ process for a de-prioritized CG is considered as not pending process. Then, it will be aligned with the latest agreement.

Proposal 3. HARQ process for a de-prioritized CG is considered as not pending process.

The next open issue is change of CG size in unlicensed spectrum. In IIOT, Autonomous Transmission is performed only if the size of the stored MAC PDU matches with CG. However, NR-U, change of the size is not assumed. For harmonization, we need to have the same assumption as IIOT.

Proposal 4. Autonomous retransmission (CGRT) is performed only if CG size does not change (i.e. MAC PDU size.

The last issue is whether prioritization between initial transmission and retransmission is needed. A motivation is that when new high-priority data arrives and a low-priority MAC PDU is stored for transmission, the high-priority data may need to be transmitted quickly. However, both NR-U and IIOT assume that CG resource is shared by LCHs with similar QoS requirements, especially for URLLC. It is not likely that URLLC data and eMBB share the same resource and prioritization between them is needed. In this context, we think prioritization between initial transmission and retransmission is not needed and only NR-U rule is sufficient.

Proposal 5. Retransmission is prioritized over new transmission as in Rel-16 NR-U.

3	Conclusion

Based on the discussion, RAN2 is requested to agree the following proposals:

Proposal 1. cg-RetransmissionTimer is stopped when CG configured with AutonomousTx is de-prioritized. cg-RetransmissionTimer is NOT stopped when CG NOT configured with AutonomousTx is de-prioritized.

Proposal 2. When LCH-based Prioritization is configured, HARQ process is not shared between CG configurations.

Proposal 3. HARQ process for a de-prioritized CG is considered as not pending process.

Proposal 4. Autonomous retransmission (CGRT) is performed only if CG size does not change (i.e. MAC PDU size.

Proposal 5. Retransmission is prioritized over new transmission as in Rel-16 NR-U.