3GPP TSG-RAN WG2 Meeting #113 bis electronic	                                      R2-2103705

Online, April 12 – April 20, 2021

Agenda Item:	8.1.3

Source:	CMCC

Title:	Discussion on delivery mode 2 transmission

Document for:	Discussion

Introduction

In RAN2#113, MBS idle mode supporting/delivery mode 2 was discussed based and some agreements were made [1]:

The two-step based approach (i.e. BCCH and MCCH) as adopted by LTE SC-PTM is reused for the transmission of PTM configuration for NR MBS delivery mode 2.

Assume it is possible to reuse LTE SC-PTM mechanism for the CONNECTED UEs to receive the PTM configuration for NR MBS delivery mode 2, i.e. broadcast based manner.

Assume that MCCH change notification mechanism is used to notify the changes of MCCH configuration due to session start for delivery mode 2 of NR MBS (other cases FFS, if any).

Also, an email discussion [Post113-e][053][MBS17] was proposed, in which MCCH transmission is discussed, including the issues related to beam sweeping.

In this contribution we would provide our view on delivery mode 2 transmission with beam sweeping.

Discussion

In NR design, due to the wide bandwidth, coreset and search space were introduced for PDCCH detection, which could save resource, reduce the detection complexity, and increase system flexibility.

Common search spaces are defined in TS 38.331 [2] for SIB1, paging and OSI.  And the search space could be configured separately from search space for SIB1, or reuse the search space for SIB1.

Similarly, MCCH is also a kind of common channel, therefore, the common search space could be defined for MCCH, which could be configured separately from search space for SIB1 or not.

Proposal 1: RAN2 should confirm that common search space for MCCH is needed, and whether it could be separately configured from search space for SIB1 or not.

If CSS for MCCH reuse CSS for SIB1, the same mapping rules of PDCCH occasions and SSB for paging or OSI could be used, and this may be decided by RAN1.

Proposal 2: The similar association rules of PDCCH occasions and SSB for paging or OSI could be applied to MCCH occasions if CSS for MCCH reuses CSS for SIB1.

Regarding to the other case, when a separate CSS is configured, we need to consider the association of PDCCH occasion and SSBs.

For SI, the association between PDCCH occasions and SSBs are defined in 38.331 as following:

-------------copy from TS 38.331 [2] ------------- ##### 5.2.2.3.2 Acquisition of an SI message For SI message acquisition PDCCH monitoring occasion(s) are determined according to searchSpaceOtherSystemInformation. If searchSpaceOtherSystemInformation is set to zero, PDCCH monitoring occasions for SI message reception in SI-window are same as PDCCH monitoring occasions for SIB1 where the mapping between PDCCH monitoring occasions and SSBs is specified in TS 38.213[13]. If searchSpaceOtherSystemInformation is not set to zero, PDCCH monitoring occasions for SI message are determined based on search space indicated by searchSpaceOtherSystemInformation. PDCCH monitoring occasions for SI message which are not overlapping with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from one in the SI window. The [x×N+K]th PDCCH monitoring occasion (s) for SI message in SI-window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in SI-window/N). The actual transmitted SSBs are sequentially numbered from one in ascending order of their SSB indexes. The UE assumes that, in the SI window, PDCCH for an SI message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB and thus the selection of SSB for the reception SI messages is up to UE implementation.

For paging, the association between PDCCH occasions and SSBs are defined in 38.304 as following:

-------------copy from TS 38.304 [3] ------------- The PDCCH monitoring occasions for paging are determined according to pagingSearchSpace as specified in TS 38.213 and firstPDCCH-MonitoringOccasionOfPO and nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured as specified in TS 38.331 [3]. When SearchSpaceId = 0 is configured for pagingSearchSpace, the PDCCH monitoring occasions for paging are same as for RMSI as defined in clause 13 in TS 38.213. When SearchSpaceId = 0 is configured for pagingSearchSpace, Ns is either 1 or 2. For Ns = 1, there is only one PO which starts from the first PDCCH monitoring occasion for paging in the PF. For Ns = 2, PO is either in the first half frame (i_s = 0) or the second half frame (i_s = 1) of the PF. When SearchSpaceId other than 0 is configured for pagingSearchSpace, the UE monitors the (i_s + 1)th PO. A PO is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for paging in the PO corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. The PDCCH monitoring occasions for paging which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for paging in the PF. When firstPDCCH-MonitoringOccasionOfPO is present, the starting PDCCH monitoring occasion number of (i_s + 1)th PO is the (i_s + 1)th value of the firstPDCCH-MonitoringOccasionOfPO parameter; otherwise, it is equal to i_s * S*X. If X > 1, when the UE detects a PDCCH transmission addressed to P-RNTI within its PO, the UE is not required to monitor the subsequent PDCCH monitoring occasions for this PO.

As agreed in last meeting, the two-step based approach (i.e. BCCH and MCCH) as adopted by LTE SC-PTM for the transmission of PTM configuration for NR MBS delivery mode 2, and MCCH is transmitted periodically in SC-PTM [4], therefore, both of the two association ways for SI and paging could be considered for association MCCH CSS PDCCH occasions and SSBs, since both SI is transmitted periodically per SI window, and paging has its periodicity.

SystemInformationBlockType20-r13 ::=	SEQUENCE {

sc-mcch-RepetitionPeriod-r13		ENUMERATED {rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256},

sc-mcch-Offset-r13				INTEGER (0..10),

sc-mcch-FirstSubframe -r13		INTEGER (0..9),

sc-mcch-duration-r13			INTEGER (2..9)	OPTIONAL,

sc-mcch-ModificationPeriod-r13	ENUMERATED {rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256,

rf512, rf1024, r2048, rf4096, rf8192, rf16384, rf32768,												rf65536},

However, there are still some differences between the two association ways. SI is transmitted in beam sweeping way for all UEs, while for paging, PO is used to distribute UEs, and POs are transmitted in beam sweeping way.  Considering that MCCH is for all UEs in a cell, the association way for SI seems more suitable.

Proposal 3: The association between PDCCH occasions and SSBs for SI could be considered for MCCH search space design.

Besides, for paging occasion decision, firstPDCCH-MonitoringOccasionOfPO was introduced, this could also be considered in the MCCH CSS design.

Proposal 4: The indication of first PDCCH monitoring occasion could be considered in MCCH search space design.

What’s more, there is another issue, whether MTCH search space should be separately defined from MCCH search space. In LTE SC-PTM, MBS data transmission could be configured with DRX.

For each SC-MTCH, the following scheduling information is provided on SC-MCCH [5]:

-	SC-MTCH scheduling cycle;

-	SC-MTCH on-duration: duration in downlink subframes that the UE waits for, after waking up from DRX, to receive PDCCHs. If the UE successfully decodes a PDCCH indicating the DL-SCH to which this SC-MTCH is mapped, the UE stays awake and starts the inactivity timer;

-	SC-MTCH inactivity-timer: duration in downlink subframes that the UE waits to successfully decode a PDCCH, from the last successful decoding of a PDCCH indicating the DL-SCH to which this SC-MTCH is mapped, failing which it re-enters DRX. The UE shall restart the inactivity timer following a single successful decoding of a PDCCH.

Considering the similarity of MTCH and MCCH, both of them could be scheduled in a periodic way, we think the same search space could be used for MTCH and MCCH, if MCCH search space design reuses the same principle for SI, but via same other parameters to adapt for their difference, like different reception window.

Proposal 5: RAN2 need confirm whether the same search space could be used for MTCH and MCCH.

The design of search space is related to RAN1, after negotiation with RAN1 colleagues, we noted that RAN1 had no idea of the difference from the MCCH and MTCH. Therefore, RAN2 should provide more information of MCCH and MTCH for RAN1’s better understand on the MCCH/MTCH and to facilitate RAN1 progress on the search space design, so we draft the LS to RAN1 [6].

Proposal 6: RAN2 should send LS to RAN1, informing our progress and providing more information of MCCH and MTCH for their discussion on search space design.

Conclusions

In this paper, we discussed delivery mode 2 transmission, especially the search space design related issues, based on the discussion, we propose:

Proposal 1: RAN2 should confirm that common search space for MCCH is needed, and whether it could be separately configured from search space for SIB1 or not.

Proposal 2: The similar association rules of PDCCH occasions and SSB for paging or OSI could be applied to MCCH occasions if CSS for MCCH reuses CSS for SIB1.

Proposal 3: The association between PDCCH occasions and SSBs for SI could be considered for MCCH search space design.

Proposal 4: The indication of first PDCCH monitoring occasion could be considered in MCCH search space design.

Proposal 5: RAN2 need confirm whether the same search space could be used for MTCH and MCCH.

Proposal 6: RAN2 should send LS to RAN1, informing our progress and providing more information of MCCH and MTCH for their discussion on search space design.

References

- RAN2 113-e Chairman Notes 2021-02-05 EOM

- 3GPP TS 38.331 V16.4.1 (2021-03) Radio Resource Control (RRC) protocol specification (Release 16)

- 3GPP TS 38.304 V16.4.0 (2021-03) User Equipment (UE) procedures in Idle mode and RRC Inactive state (Release 16)

- 3GPP TS 36.331 V16.4.0 (2021-03) Radio Resource Control (RRC) (Release 16)

- 3GPP TS 36.300 V16.5.0 (2021-03) Overall description; Stage 2 (Release 16)

- R2-XXXXX LS on delivery mode 2 transmission CMCC