3GPP TSG-RAN WG2 Meeting #113bis electronic	R2-2103747

Online, April 12 – April 20, 2021

Agenda Item:	8.10.3.1

Source:	Ericsson

Title:	Aspects for Earth fixed and Earth moving beams for NTN

Document for:	Discussion, Decision

# 1	Introduction

A new WI on solutions for NR to support non-terrestrial networks (NTN) was approved at RAN#86, with an updated WID approved at RAN#88 [1]. The WI aims to specify the following control plane enhancements:

- Idle mode:

  - Definition of additional assistance information for cell selection/reselection (e.g. using UE location information, satellite Ephemeris information)

  - Definition of NTN (satellite/HAPS) cell specific information in SIB

- Connected mode

  - Enhancement necessary to take into account location information (UE & Satellite/HAPS) and/or ephemeris in determining when to perform hand-over, in order to have a high degree of hand-over control for hand-over robustness and coverage management.

  - Enhancement to existing measurement configurations to address absolute propagation delay difference between satellites (e.g. SMTC measurement gap adaptation to the SSB/CSI-RS measurement window) [RAN2/4].

- Service continuity for mobility from TN to NTN and from NTN to TN systems (to be addressed when connected mode mobility has sufficiently progressed)

- Identify potential issues associated to the use of the existing Location Services (LCS) application protocols to locate UE in the context of NTN and specify adaptations if any [RAN2/3]

Agreements from RAN2#113e:

Agreements:

- The NTN ephemeris is divided into serving cell’s ephemeris and neighbour’s ephemeris. FFS how would they differ regarding e.g. the required accuracy or signalling impact.

- Consider pre-configuration in uSIM, NAS, SIB and RRC signalling for providing the NTN ephemeris. Further discussion depends on the agreed ephemeris contents.

Agreements:

- RAN2 thinks that a UE needs to know whether the network is a TN or NTN no later than SIB1 reception

- The information on when a cell is going to stop serving the area and/or the timing information (e.g. timer or absolute time) about new upcoming cell is supported at least in Earth-fixed NTN scenario. FFS if both types of information are needed. FFS if this is known from system information and/or the ephemeris.

In this contribution, we provide our view on details on how the information on “when a cell is going to stop serving the area and/or the timing information (e.g. timer or absolute time) about new upcoming cell is supported at least in Earth-fixed NTN scenario.” is impacting the UE behaviors excluding CHO related  aspects which are discussed in

# 2 Solutions specific to soft feeder link switch and service link switch for Earth fixed beams

The soft and hard feeder link switch is depicted for Earth moving and Earth fixed beams in Figure 1. Here, as gNBs are at the GW, the Uu interface is over the air via feeder link and then via service link.

Figure 1 Soft(upper) and hard(lower) feeder link switch for both Earth moving and Earth fixed beams

The service link switch depicted in Figure 2. Due to the movement of the non-GEO satellites in relation to the surface of the earth, at some point in time one satellite leaves and is not able to serve a certain geographical area.

Figure 2 Service link switch for Earth fixed beams

One relevant and common aspect for both Fig 1 and Fig 2 is that one cell/PCI is going to stop serving an area. The remaining time Tservice until the cell/PCI is going to stop serving an area can be defined. While the exact definition can vary and can be discussed further, it is possible to discuss the relevance of the Tservice with respect cell selection/reselection without the exact definition settled.

For feeder and service link switches one cell/PCI is going to stop serving an area at a certain time.

The remaining time Tservice until the cell/PCI is going to stop serving an area can be defined.

For those scenarios, location and RSRP are not enough to trigger cell reselection for a UE that is in the center of the cell that is going to vanish.

The location and RSRP are not enough to trigger cell selection for a UE that is in the center of the cell that is going to vanish

RAN2 should take the Tservice into account for cell selection and reselection

2.1 RSRP measurements for cell reselection

Another aspect is idle mode measurements. In current NR, the UE is expected to regularly perform RSRP/RSRQ measurements for cell reselection purposes in idle mode on inter-frequency and intra-frequency neighbouring cells. There is however a set of exceptions to these rules which are based on whether the signal strength/quality of the current cell is above certain thresholds, which is generally as below:

- the cell selection RX level value (related to RSRP measurements) Srxlev > ThresholdP

- the cell selection quality value (related to RSRQ measurements) Squal > ThresholdQ

If these are fulfilled the UE may choose not to perform intra-frequency, or inter-frequency, measurements whichever applies. For feeder link and service link switch scenarios, RSRP is not enough to trigger measurements for a UE that is in the center of the cell that is going to vanish. One relevant and common aspect for both Fig 1 and Fig 2 is that one cell/PCI is going to stop serving an area. The remaining time Tservice until the cell/PCI is going to stop serving an area can be defined. While the exact definition can vary and can be discussed further, it is possible to discuss the relevance of the Tservice with respect RSRP measurement for cell reselection without the exact definition settled.

The location and RSRP are not enough to trigger cell selection for a UE that is in the center of the cell that is going to vanish

RAN2 should take the Tservice into account for the idle mode measurement rules.

# Conclusion

In the previous sections we made the following observations:

Observation 1	For feeder and service link switches one cell/PCI is going to stop serving an area at a certain time.

Observation 2	The remaining time Tservice until the cell/PCI is going to stop serving an area can be defined.

Observation 3	The location and RSRP are not enough to trigger cell selection for a UE that is in the center of the cell that is going to vanish

Observation 4	The location and RSRP are not enough to trigger cell selection for a UE that is in the center of the cell that is going to vanish

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 should take the Tservice into account for cell selection and reselection

Proposal 2	RAN2 should take the Tservice into account for the idle mode measurement rules.

# References

RP-201256, “Solutions for NR to support non-terrestrial networks (NTN),” 3GPP TSG RAN #88e, June 2020.

TR 38.821, “Solutions for NR to support Non-Terrestrial Networks (NTN),” V16.0.0.

TR 38.811, “Study on New Radio (NR) to support non-terrestrial networks,” V15.3.0

R2-2007537, “NTN scope, scenarios, architecture, and requirements,” 3GPP TSG RAN WG2 Meeting #111, Ericsson, August 2020.

Aida, S., Patzelt, T., Leushacke, L., Kirschner, M., and Kiehling, R., ‘Monitoring and Mitigation of Close Proximities in Low Earth Orbit’, 21st International Symposium on Space Flight Dynamics – 21st ISSFD, Toulouse, France, 2009.

R1-2005502, “On UL time and frequency synchronization enhancements for NTN,” 3GPP TSG RAN WG1 Meeting #102, Ericsson, August 2020.

R2-2009818, Idle mode for NTN, Ericsson