3GPP TSG-RAN WG2 #113bis-e	Tdoc R2-2103808

Electronic meeting, 2021-04-12 – 2021-04-20

Agenda Item:	8.2.2.2

Source:	Ericsson

Title:	UE measurements and reporting in deactivated SCG

Document for:	Discussion, Decision

# 1	Introduction

This contribution covers UE measurements and reporting in deactivated SCG, including related UE procedures when the SCG is deactivated, such as maintaining DL sync, time alignment handling, beam management and radio link monitoring. At least the following FFSs from last meeting RAN2#113e are addressed:

- FFS for option 2a): in the SCG deactivated state, the UE monitors some DL beams (FFS if the same as BFD or RLM) and, if the UE sees that the beams are not good enough (details FFS), the UE either (one of the options to be selected):

- -	will perform random access upon reception of the next SCG activation indication from the MCG

- -	reports measurement results (details FFS) via the MCG and wait for reconfiguration.

- 5	Continue to discuss whether some kind of beam monitoring (similar to RLM/BFD) should be supported when the SCG is deactivated. FFS if this only applies to when TAT is running.

6	Clarify the meaning of "the UE maintains DL sync while the SCG is deactivated" (e.g. whether that is a consequence of doing RRM measurements of the PSCell or something more is needed).

- FFS for option 2a): in the SCG deactivated state, the UE monitors some DL beams (FFS if the same as BFD or RLM) and, if the UE sees that the beams are not good enough (details FFS), the UE either (one of the options to be selected):

- -	will perform random access upon reception of the next SCG activation indication from the MCG

- -	reports measurement results (details FFS) via the MCG and wait for reconfiguration.

5	Continue to discuss whether some kind of beam monitoring (similar to RLM/BFD) should be supported when the SCG is deactivated. FFS if this only applies to when TAT is running.

FFS if in absence of PDCCH monitoring and UL transmission, and it is possible to assume that TA is valid when the TA timer has not expired.

In addition, these remaining FFSs from RAN2#112 are addressed in this contribution

FFS1: Details on the performed measurements (e.g. all SN configured measurements or subset based on certain criteria, restrictions on inter-frequency/RAT)

8a: It is FFS whether the network can configure the UE stop certain configured RRM measurements while the SCG is deactivated, or can release certain RRM measurements at SCG deactivation.

8b: Relaxation of RRM measurement requirements (as compared with non-DRX activated cell requirements) while the SCG is deactivated is FFS.

# 2	Discussion

## 2.1	RRM measurements

RAN2 has agreed that

As a baseline, MN-configured RRM measurement/reporting procedures do not depend on the SCG activation state (deactivated or activated). Further optimisations are not precluded. 

FFS1: Details on the performed measurements (e.g. all SN configured measurements or subset based on certain criteria, restrictions on inter-frequency/RAT)

5: When the SCG is in deactivated state, the UE sends MeasurementReport messages for measurement results of SN-configured measurements embedded in the E-UTRA (if the MCG is EUTRA) or in the NR (if the MCG is NR) ULInformationTransferMRDC message via SRB1

8a: It is FFS whether the network can configure the UE stop certain configured RRM measurements while the SCG is deactivated, or can release certain RRM measurements at SCG deactivation.

The deactivation of SCG is performed in order save power both in the UE and in the network. The UE needs to perform SCG configured RRM measurements also when the SCG is deactivated in order to support PSCell addition/change and SCell addition/change (of deactivated SCells). However, as there is naturally a trade-off between UE power consumption and mobility robustness, the amount of RRM measurements performed when the SCG is deactivated should be reduced.

In baseline, the UE performs measurements and reporting according to whatever is configured in measConfig by the MN and the SN. We have also previously agreed that RRC signalling is the baseline for performing SCG deactivation/activation. Upon SCG deactivation or activation, this means the network can explicitly reconfigure measurements (using SCG measConfig) with the same RRC message that performs the SCG activation/deactivation.

This is illustrated by the figure below for the case of MN initiated SCG activation/deactivation.

Figure 1. Explicit reconfiguration of SCG RRM measurements at MN initiated SCG activation/deactivation

We propose therefore:

The network may explicitly reconfigure the SCG RRM measurements in the same RRC message (de)activating the SCG.

During  SCG activation/deactivation we think explicit SCG RRM measurement reconfiguration between a "full set" and a "limited set" of RRM measurements works well, assuming the reconfiguration of the SCG measConfig can be provided by the SN and sent in the same RRC message which triggers the SCG (de)activation in the UE.

One potential drawback of relying on explicit reconfiguration of SCG RRM measurements is if the activation/deactivation can be performed without an SCG RRCReconfiguration message (such as an MN RRCReconfiguration message or a MAC CE). In those cases, the network may of course provide the reconfiguration of the SCG measConfig separately, after the activation/deactivation has been performed. This adds extra signalling and prolongs the time the UE switches between a "limited set" and  "full set" of RRM measurements. Even if the activation/deactivation is done by normal RRC signalling, it causes extra overhead to reconfigure the measurements every time the SCG is activated / deactivated, especially since the measurements while the SCG is activated and the measurements while the SCG is deactivated are likely the same every time. It would therefore be beneficial if the network could configure measurements for activated or deactivated SCG respectively, so that reconfiguration of measurements is normally not needed when the SCG is activated or deactivated. The measurements for deactivated SCG may be configured as a subset of the measurements for activated SCG, to avoid unnecessary signalling overhead.

The network can configure separate measurement configurations for activated and deactivated SCG, respectively, where the measurements for deactivated SCG may be a subset of the measurements for activated SCG.

Another way of reduce the amount of RRM measurements is by relaxations. As a relevant well-known example of RRM measurement relaxation, for deactivated SCell, it is currently possible to configure a relaxed measurement cycle by RRC as follows:.

measCycleSCell                      ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}  OPTIONAL    -- Need R

measCycleSCell The parameter is used only when an SCell is configured on the frequency indicated by the measObjectNR and is in deactivated state, see TS 38.133 [14]. gNB configures the parameter whenever an SCell is configured on the frequency indicated by the measObjectNR, but the field may also be signalled when an SCell is not configured. Value sf160 corresponds to 160 sub-frames, value sf256 corresponds to 256 sub-frames and so on.

As we also discussed in our contribution [1] for the last RAN2 meeting, longer measurement cycles may lead to slower triggering of measurement events but in case of deactivated SCG, this may not be critical. We think that the principles for relaxation of RRM measurements for deactivated SCell can be reused also for deactivated SCG (deactivated PSCell) in order to save UE power. This needs to be studied bý RAN4, but may also lead to RRC impact, e.g. by adding a measCyclePSCell field in measObjectNR to be used to determine measurement cycles when SCG is deactivated.

Relaxation of RRM measurements to further reduce UE power consumption during when SCG is deactivated may be beneficial from UE power consumption point of view.

Relaxation of RRM measurements for deactivated SCG should be considered and studied by RAN4.

Send an LS to RAN4 to ask them to study relaxed RRM measurements for deactivated SCG.

A draft LS is provided in the annex.

## 2.2	Maintaining DL sync

As explained in [2], establishing the DL synchronization is one of the main delay components during PSCell addition, especially for FR2 SCG with long SMTC. This delay component can be eliminated for the activation of deactivated SCG by the UE maintaining the DL fine synchronization for deactivated SCG.

This was discussed in RAN2#113e, and the following was noted:

- Agreements

- . . .

6	Clarify the meaning of "the UE maintains DL sync while the SCG is deactivated" (e.g. whether that is a consequence of doing RRM measurements of the PSCell or something more is needed).

RAN2 has agreed that the UE continues to perform RRM measurements according to the SCG measConfig while the SCG is deactivated. Thus, some level of downlink synchronization can be achieved. In the previous section, we observed that relaxations of RRM measurements for deactivated SCG, such as longer measurement cycle for PSCell, may be beneficial from UE power consumption point of view. However, in order to reach the desired latency reduction, the UE needs to maintains fine DL synchronization to the extent that it is able to receive a command to activate the SCG and “immediately” start monitoring the PSCell’s PDCCH/CORESET or, to access the PSCell e.g. by transmitting a random access preamble at the next available PRACH occasion on the PSCell or a PUCCH scheduling request (if the UL is also assumed to be synchronized). This should be possible if PSCell RRM measurements are dense enough to maintain DL fine synchronization. Given that up to 160ms SSB interval is supported and there is no scaling of the measurement cycle for PSCell, our understanding is that a measurement cycle of 160 ms should be sufficient to maintain the DL fine sync. This still gives a significant power saving gain over keeping the PSCell activated, for which DRX cycle can be 40-80ms. However, exactly how, and under which side conditions, to meet the requirement on maintained downlink synchronization, e.g. whether it would be required to introduce shorter measurement cycles for measurements on PSCC than currently defined for measurements on deactivated SCC, needs to be studied by RAN4, see draft LS in Annex A. Note that the measurement cycle is provided to the UE in dedicated signaling, so to which extent the UE maintains the DL sync can in this was be controlled by the network, depending on user service, traffic situation, system load, etc.

In addition to performing PSCell RRM measurements, the UE also needs to keep track of SSB and MIB of the PSCell, to keep SFN timing such that the UE is ready to transmit the random access preamble or Scheduling Request (SR) at the next available PRACH or SR/PUCCH occasion when the SCG is to be activated. In this context, keeping track of an SSB means the UE knows at least one suitable SSB (whose SS-RSRP is above the suitability threshold) that maps to a PRACH occasion. The alternative would be to investigate different means of speeding up the DL fine synchronization, e.g. through the use of temporary RS, as being discussed in RAN1 for SCell activation, though that will require RAN1 to look into also temporary RS for PSCell activation, which is currently not withing the planned Rel-17 scope.

The UE maintains DL sync while the SCG is deactivated (e.g. including SFN timing and SSB selection) such that the UE is ready to transmit in next PRACH or SR occasion having processed the SCG activation command. FFS possible dependency on the measurement configuration.

Send an LS to RAN4 to ask about RRM measurement requirements for maintaining DL fine sync.

## 2.3	Time alignment handling

In [5]  we have discussed the SCG activation delays for obtaining UL and DL sync when the SCG is deactivated. In order to reduce the SCG activation delay, it has been proposed that the UE maintains DL sync (see Proposal 4). The results in [5] show that maintaining also the UL sync can reduce the SCG activation delay by avoiding random access. This was topic was discussed during RAN2#113e in offline #230 [3], where in Q3 a clear majority (22 out of 20) of participating companies supported that in some scenarios random access is not used during SCG activation. One opposing company expressed the concern about how to ensure validity of the TA in absence of PDCCH monitoring and UL transmission for the deactivated SCG, meaning that TA MAC CE cannot be sent and there is no UL signal for the network to measure on to determine the required TA. The concern resulted in the following FFS:

FFS if in absence of PDCCH monitoring and UL transmission, and it is possible to assume that TA is valid when the TA timer has not expired.

The above concern addresses active maintenance of the TA. In our view, actively maintaining the TA will require more complex solutions as the TA command would have to be transmitted to the UE via MCG and furthermore the network would probably need to receive some transmission from the UE in the SCG in order to determine the need for TA command, which would also increase UE power consumption. However, keeping the timer running when SCG is deactivated, and assume TA while TA timer is running (to possibly avoid random access if the SCG is to be activated) seems a good compromise. This way of passively maintaining the TA will also result in lower power consumption at the UE, since no UL transmissions are needed for tracking and correcting the time alignment.

When the SCG is deactivated the TA timer is not stopped.

A follow up question is the UE behaviour when TA timer expires while the SCG is deactivated. One possibility, aiming for better readiness, would be to update the UL sync i.e. trigger a random access procedure with the PSCell and receive the TA command. However, as a valid concern from a number of companies is the UE power consumption while the SCG is deactivated, another alternative could be that the UE only acts on that when it needs to perform an UL transmission. For example, if the SCG is to be activated and the TA has expired, the UE knows it needs to access the PSCell by performing random access.

The other remaining question is what happens when the deactivated SCG needs to be activated and the TA timer is still running, i.e., the UL can be assumed to be synchronized. If the UE receives a command from via the MCG to activate the SCG, the UE could in principle directly monitor the PSCell’s PDCCH and transmit on PUCCH, without the need for random access. However, if the PSCell has multiple SSBs (e.g. multiple SSBs transmitted in multiple beams in FR2), this is only possible if beam alignment is also maintained, which will be discussed in the following. Further details about the UE behaviour upon activation, depending on TA timer, can be found in [2].

If the TA timer is still running upon SCG activation, the UE assumes it is UL synchronized, which enables the UE to access the PSCell without the need of random access. If TA timer has expired upon SCG activation, the UE performs random access in the PSCell.

## 2.4	Beam management/CSI and BFR/BFD

As discussed in 2.3, maintaining UL and DL sync enables the UE to activate the SCG without the need for random access. However, if the PSCell has multiple SSBs associated to it, that may not be sufficient depending whether beam alignment can still be assumed or not.

BFD/BFR

The support of BFD while SCG is deactivated was part of the email discussion [3], but the summary is not conclusive. As rapporteur summarizes, companies that answered “no” consider the benefit not worth the cost while there is no data transmission via the SCG. However, readiness for SCG transmission / reception while the SCG is deactivated should also be considered. The UE should be as ready as possible to be scheduled in the SCG after the SCG is deactivated, otherwise SCG deactivation would not present significant gains compared to a legacy SCG release and add procedure. If BFD is not supported, and at the same time, beam management / CSI is not supported, the UE would always require random access upon activation, since even if the TA timer would be running, the UE may have lost the beam alignment. Always resorting to random access is not preferred, taking into account the time needed for resource / beam selection.

Supporting BFD on top of already agreed RRM measurements for deactivated SCG is not expected to be very costly in terms of UE power consumption. This was also indicated in [4], which showed that “In SCG deactivated, UE performing RLM and/or BFD on the PSCell should not result in significant additional power consumption if RRM measurements are already being performed”.

If none of BFD, beam management, CSI are supported, UE always requires random access upon SCG activation which increases the activation delay.

The UE performs BFD monitoring for deactivated SCG. FFS BFD configuration while SCG is deactivated.

BFD configuration while SCG is deactivated

In RAN2#113e the following was captured concerning beam monitoring:

- Agreements

- FFS for option 2a): in the SCG deactivated state, the UE monitors some DL beams (FFS if the same as BFD or RLM) and, if the UE sees that the beams are not good enough (details FFS), the UE either (one of the options to be selected):

- -	will perform random access upon reception of the next SCG activation indication from the MCG

- -	reports measurement results (details FFS) via the MCG and wait for reconfiguration.

- 5	Continue to discuss whether some kind of beam monitoring (similar to RLM/BFD) should be supported when the SCG is deactivated. FFS if this only applies to when TAT is running.

. . .

The text above first points to the question on the BFD configuration while the SCG is deactivated (FFS BFD configuration while SCG is deactivated). Up to Release 16, BFD resources to be monitored are configured together with the Radio Link Monitoring (RLM) resources and may overlap or not with RLM beams/ resources. There are two main ways to configure BFD resources:

- Explicit list of SSBs and/or CSI-RS;

- Implicit SSB or CSI-RS based on TCI state associated to the PDCCH being monitored (If no RSs are provided for the purpose of beam failure detection, the UE performs beam monitoring based on the activated TCI-State for PDCCH)

***************************************************************************************************************************

RadioLinkMonitoringConfig information element

-- ASN1START

-- TAG-RADIOLINKMONITORINGCONFIG-START

RadioLinkMonitoringConfig ::=       SEQUENCE {

failureDetectionResourcesToAddModList   SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS 		OPTIONAL, -- Need N

[…]

beamFailureInstanceMaxCount             ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}                          OPTIONAL, -- Need R

beamFailureDetectionTimer               ENUMERATED {pbfd1, pbfd2, pbfd3, pbfd4, pbfd5, pbfd6, pbfd8, pbfd10}  OPTIONAL, -- Need R

...

}

RadioLinkMonitoringRS ::=           SEQUENCE {

radioLinkMonitoringRS-Id            RadioLinkMonitoringRS-Id,

purpose                             ENUMERATED {beamFailure, rlf, both},

detectionResource                   CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

},

...

}

-- TAG-RADIOLINKMONITORINGCONFIG-STOP

-- ASN1STOP

failureDetectionResourcesToAddModList A list of reference signals for detecting beam failure and/or cell level radio link failure (RLF). The limits of the reference signals that the network can configure are specified in TS 38.213 [13], table 5-1. The network configures at most two detectionResources per BWP for the purpose beamFailure or both. If no RSs are provided for the purpose of beam failure detection, the UE performs beam monitoring based on the activated TCI-State for PDCCH as described in TS 38.213 [13], clause 6. If no RSs are provided in this list for the purpose of RLF detection, the UE performs Cell-RLM based on the activated TCI-State of PDCCH as described in TS 38.213 [13], clause 5. The network ensures that the UE has a suitable set of reference signals for performing cell-RLM.

***************************************************************************************************************************

For BFD while the SCG is deactivated, we see no strong reason to deviate from this: the UE should be able to perform BFD monitoring based on an explicit list of beams (SSBs and/or CSI-RS) or based on the activated TCI-State for PDCCH, depending on network configuration. The exact resources and/or the way to perform BFD is currently determined by RRC signaling (e.g. RRCReconfiguration). Hence, assuming that the network indicates an SCG deactivation via RRC, that same signaling could reconfigure or not the way BFD is performed, as any other RRCReconfiguration. So at least as baseline, nothing needs to be done in RRC to support BFD monitoring while SCG is deactivated, and the network may re-configure BFD (any parameter) in the same message deactivating the SCG.

BFD may be reconfigured by the network in the same RRC message deactivating the SCG.

Actions upon BFD while SCG is deactivated

There can be different possible alternatives to handle BFD while SCG is deactivated. A first alternative is to follow the behaviour defined while the SCG is activated i.e. upon BFD the UE performs BFR, which leads to a random access to the PSCell. The benefit of this alternative is that beam alignment would always be assumed while SCG is deactivated. The drawback is however increased UE power consumption, similar to the case of maintaining the UL Time Alignment and the need to support random access to the PSCell while the SCG is deactivated. A second alternative agreed as a possibility in RAN2#113e is the reporting of PSCell BFR via the MCG e.g. in an RRC message or in a MAC CE. The benefit of this alternative is that the network can decide to take some action or not e.g. trigger an SCG re-configuration and/or SCG activation. The exact content of the BFR report via MCG and the format (if RRC or MAC CE) could be FFS for the time being.

Upon BFD while SCG is deactivated the UE stops to perform BFD monitoring and reports the BFD via the MCG.

In response, the network can transmit an RRCReconfiguration to release the SCG, or trigger SCG mobility, activate the SCG, re-configure BFD, etc. However, there should be no requirement for the network to transmit such a message. Hence, upon BFD the UE simply becomes aware that when it is time to activate the SCG it needs to perform random access.

If the SCG is to be activated and BFD was declared, random access is needed when the UE activates the PSCell.

In other words, when the SCG is deactivated, the UE performs BFD and keeps TA timer running. If the SCG is to be activated and TA timer is still running and BFD was not declared, the UE activates the PSCell without random access.

When the SCG is to be activated, if TA timer is still running and BFD was not declared, there is no need for random access when the UE activates the PSCell.

The procedure for SCG activation, and relations to BFD and random access is further discussed in [2].

## 2.5	Radio Link Monitoring

A legacy UE operating in MR-DC performs both MCG RLM (M-RLM) and SCG RLM (S-RLM). If an S-RLF (RLF triggered on the PSCell) is triggered while the MCG link is not suspended, the UE transmits an SCG failure information message including available measurements.

To summarize our view, we believe it is important that the UE continues to perform S-RLM while the SCG is deactivated. Performing S-RLM can also help UE to maintain DL synchronization, which will reduce the activation delay for the SCG, see [5]. Reduced SCG activation time is important to maximize MCG offloading, see simulation results in [5].. We are sensitive to the issue related to power consumption, though there are ways to reduce power consumption due to S-RLM e.g. defining relaxed requirements. On the other hand, perhaps a more critical issue is the fact that S-RLM may need to change due to intra-cell mobility, as the UE has a limited number of resources monitored for RLM, and updates may require the reception of RRC messages and/or MAC CEs from the SCG (though UE does not monitor SCG PDCCH). Hence, RAN2 should discuss RLM when the implications of beam management and CSI reporting/measurements are settled for a deactivated SCG.

If SCG is deactivated, UE performs some level of S-RLM and SCG failure information procedure is supported to report the failure.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Relaxation of RRM measurements to further reduce UE power consumption during when SCG is deactivated may be beneficial from UE power consumption point of view.

Observation 2	If the TA timer is still running upon SCG activation, the UE assumes it is UL synchronized, which enables the UE to access the PSCell without the need of random access. If TA timer has expired upon SCG activation, the UE performs random access in the PSCell.

Observation 3	If none of BFD, beam management, CSI are supported, UE always requires random access upon SCG activation which increases the activation delay.

Observation 4	If the SCG is to be activated and BFD was declared, random access is needed when the UE activates the PSCell.

Observation 5	When the SCG is to be activated, if TA timer is still running and BFD was not declared, there is no need for random access when the UE activates the PSCell.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The network may explicitly reconfigure the SCG RRM measurements in the same RRC message (de)activating the SCG.

Proposal 2	The network can configure separate measurement configurations for activated and deactivated SCG, respectively, where the measurements for deactivated SCG may be a subset of the measurements for activated SCG.

Proposal 3	Send an LS to RAN4 to ask them to study relaxed RRM measurements for deactivated SCG.

Proposal 4	The UE maintains DL sync while the SCG is deactivated (e.g. including SFN timing and SSB selection) such that the UE is ready to transmit in next PRACH or SR occasion having processed the SCG activation command. FFS possible dependency on the measurement configuration.

Proposal 5	Send an LS to RAN4 to ask about RRM measurement requirements for maintaining DL fine sync.

Proposal 6	When the SCG is deactivated the TA timer is not stopped.

Proposal 7	The UE performs BFD monitoring for deactivated SCG. FFS BFD configuration while SCG is deactivated.

Proposal 8	BFD may be reconfigured by the network in the same RRC message deactivating the SCG.

Proposal 9	Upon BFD while SCG is deactivated the UE stops to perform BFD monitoring and reports the BFD via the MCG.

Proposal 10	If SCG is deactivated, UE performs some level of S-RLM and SCG failure information procedure is supported to report the failure.

# 4	References

R2-2101094, Mobility and RRM for deactivated SCG, Ericsson, 3GPP TSG-RAN WG2 #113e, Electronic meeting, January 25th – February 5th, 2021

R2-2103809, SCG activation procedures, Ericsson, 3GPP TSG-RAN WG2 #113bis-e, Electronic meeting, April 12th – 20th, 2021

R2-2101969, Summary of [AT113-e][230][eDCCA] Solution alternatives for SCG activation and deactivation, Huawei

R2-2101871, UE behaviour in SCG deactivated state, Qualcomm, 3GPP TSG-RAN WG2 #113e, Electronic meeting, January 25th – February 5th, 2021

R2-2101095, On the need for random access during SCG activation, Ericsson, 3GPP TSG-RAN WG2 #113e

# Annex A: Draft LS to RAN4 on RRM measurements

3GPP TSG-RAN WG2 #113bis-e	R2-21xxxxx

Electronic meeting, 2021-04-12 – 2021-04-20

Title:	LS on RRM measurements for deactivated SCG

Release:	Rel-17

Work Item:	LTE_NR_DC_enh2-Core

Source:	RAN WG2

To:	RAN WG4

Cc:

Contact Person:

#### Name:	Stefan Wager

E-mail Address:	stefan.wager@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- Overall Description:

RAN2 has started the work within the Rel-17 further MR-DC/CA enhancements on the objective "Efficient activation/deactivation mechanism for one SCG and Scells". As part of this objective, RAN2 is currently studying, for power saving, a "deactivated SCG" mode of operation which is characterized by that, for example, the UE does not monitor the PDCCH and does not perform SRS transmission. Other procedures such as RLM, maintaining time alignment, CSI/RRM measurement/reporting, beam management, etc may be restricted or not performed at all when SCG is deactivated.

RAN2 has discussed RRM measurements, used for mobility, such as PSCell change, while the SCG is deactivated and have agreed that SN will be able to configure SCG RRM measurements and that the corresponding measurement reports would be sent to the network in the MCG.

If the UE performs RRM measurements on the PSCell while the SCG is deactivated  some level of downlink synchronization can be achieved. In order to reach the desired latency reduction, the UE needs to maintains fine DL synchronization to the extent that it is able to receive a command to activate the SCG and “immediately” start monitoring the PSCell’s PDCCH/CORESET or, to access the PSCell e.g. by transmitting a random access preamble at the next available PRACH occasion on the PSCell or a PUCCH scheduling request (if the UL is also assumed to be synchronized). RAN2 also has observed that some level of downlink synchronization can be achieved if the UE continues to perform RRM measurements at least on the PSCell.

Moreover, while the SN may explicitly reconfigure the RRM measurements upon SCG deactivation with existing and/or further optimized, RRC signalling mechanisms (e.g. in the measConfig RRC parameter), RAN2 sees also the possibility, if needed, to further restrict or relax these RRC configured RRM measurements upon SCG deactivation.

One example of an already existing RRM measurement relaxation mechanism is the measCycleSCell RRC parameter specified for deactivated SCell.

RAN2 asks RAN4 the following:

- Does RAN4 see potential benefits, or any issues, with relaxed RRM measurements (e.g. to reduce UE power consumption) during when SCG is deactivated?

- If RAN4 sees potential benefits with relaxed RRM measurements, what would be the longest measurement cycle that would allow fine downlink synchronization to PSCell in deactivated SCG to be maintained?

- If RAN4 sees potential benefits, RAN2 respectfully asks RAN4 start to study RRM measurements relaxations for deactivated SCG.

- Actions:

To RAN4:	RAN2 respectfully asks RAN4 to consider the information above including the above questions.

- Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #114	2021-05-19 to 2021-05-27		E-Meeting

TSG-RAN WG2 Meeting #115	2021-08-16 to 2021-08-27 (TBC)	E-Meeting