3GPP TSG-RAN WG2 #113bis-e	Tdoc R2-2103809

Electronic meeting, April 12th – 20th, 2021

Agenda Item:	8.2.2.3

Source:	Ericsson

Title:	SCG activation procedures

Document for:	Discussion, Decision

# 1	Introduction

This contribution discusses procedures for SCG activation. The following FFSs from last meeting RAN2#113e are addressed:

- Agreements

- 1a 	SCG activation can be requested by MN/SN/UE. FFS on how to accept/reject the procedure. FFS which signalling is used.

- Agreements

- 2	The UE behaviour when the SCG activation is indicated to the UE via the MCG is one or more of the following options:

- option 1)	similar to reconfiguration with sync, i.e. the UE always initiates random access to the PSCell.

- option 2)	in certain cases:

- -	the UE does not initiate random access and monitors PDCCH on the PSCell (at the latest after the specified processing time).

- -	the SCG can schedule data transmission on the PDCCH

- The UE decides not to perform random access (one option to be selected):

- option 2a) if the TA timer is still running and possibly other conditions (FFS how TAT starts)

- option 2b) based on the contents of the SCG activation indication

- 8	Further discuss the comparison between

- -  define a mechanism for SCG activation upon UL data arrival on SCG bearers

- -	use split bearer with primary path on MCG (network sees UL data and can initiate activation)

- Agreements

- 1 	Confirm that there is no PUSCH transmission on deactivated SCG. FFS if any other UL is allowed towards SCG.

# 2	Discussion

## 2.1	SCG activation delay

In [1], we analysed the delay components of the SCG activation delay. From that analysis also summarized in figure 1, it was clear that processing time on L1 (Tprocessing) and RRC (TRRC) were among the main delay components of the SCG activation procedure. In the following we analyze a bit deeper what impacts TRRC and Tprocessing in order to understand better how these could be reduced.

The RRC processing time (TRRC=16ms) could be reduced by defining a new case for RRC reconfiguration for SCG activation, including no or limited changes to the deactivated SCG configuration. In this way it should be possible to define a much lower RRC processing time for this special case. The approach is very similar to what is already defined for RRC resume in both LTE and NR, for which there is a lower RRC processing time defined that applies with certain limitations to what the RRCResume message may contain.

Reduced processing time is defined for RRCResume in both 36.331 and 38.331.

Reduced processing time was also discussed in offline [230] during RAN2#113-e [2]. According to the summary there was rather low support for reducing the RRC processing time and companies wondered “what limited changes to the SCG configuration could be”. The result was somewhat surprising, given that we already have reduced processing time for RRCResume and also since many of those companies supported MAC CE signalled SCG activation, which functionally equals the case of RRC signalled SCG activation with no other change to the SCG configuration. Thus, rather than adding yet another MAC CE for this special case, it would be preferable from specification point of view to define a reduced RRC processing time for the case where no change to the RRC configuration is included. Even better would be to cater for a few typical updates that can be expected for the SCG activation case, e.g. configuration of contention free resources for random access.

Define a reduced processing time for RRCReconfiguration for activating SCG with limited or no change to the SCG configuration.

The physical layer processing (Tprocessing = 40ms for SCG addition) currently consists of 20ms for loading software and 20ms for initializing data structures and buffers. For the software loading, it should be realistic to assume that it can be performed during SCG addition, and that the UE can keep the software loaded for the deactivated SCG. There would then be no need to reload the software during SCG activation. This would save 20ms from Tprocessing. This is already applied in RAN4 for intra frequency range PSCell change.

For the initialization of data structures and buffers, it should be possible for UEs to keep the memory initialized for deactivated SCG, which is already the case for SCell activation where the cell or frequency is not changed. So, if there is no mobility for the deactivated SCG, it should be possible to assume Tprocessing = 0ms, but this needs to be confirmed with RAN4. From performance point it would be good, since it would shave off another 40ms from the SCG activation delay, which will help in increasing the MCG offloading potential, see [1]. We provide a draft LS in Annex A.

Send LS to RAN4 to confirm whether Tprocessing = 0ms could be assumed for SCG activation, without cell or frequency change.

In Figure 1, the total activation delay according to [1] is shown for the FR2 case with 160ms SMTC. The baseline bar shows the delay without improvements compared to SCG addition. As can be seen, the main part of the SCG activation delay is consumed to establish DL synchronization and processing on L3 and L1 and that by addressing these the SCG activation delay can be reduced significantly.

Figure 1: Factors affecting SCG activation delay

## 2.2	Procedures for SCG activation

In RAN2#113e it was agreed to support MN, SN and UE initiated SCG activation, see below.

- Agreements

- 1a 	SCG activation can be requested by MN/SN/UE. FFS on how to accept/reject the procedure. FFS which signalling is used.

In this section we discuss the procedures for MN, SN and UE initiated SCG activation, the signalling used and how the procedure can be accepted/rejected.

### 2.2.1	MN initiated SCG activation

The activation of a deactivated SCG can be initiated by the MN, e.g. in case of traffic arriving on MN terminated DRBs using the SCG or based on Activity Notification from the SN for SN terminated DRBs. RAN3 has agreed to reuse the SN modification request procedure for MN initiated SCG activation and are discussing the details of the new IE for indicating the SCG activation state. The procedure is shown in Figure 2.

Figure 2. MN initiated activation of deactivated SCG

The procedure is started with MN sending the SN modification request message to the SN, including the request to activate the SCG. Upon receiving the SN modification request message, the SN should have the right to accept/reject the request. Reasons to reject may be, e.g.

- According to MR-DC principles, SN is in charge of its own resources and should thus have the right to accept or reject the request from the MN

- SN may not have resources to serve activate the SCG

- SCG mobility. PSCell may be out of coverage or SN may be triggering a PSCell change procedure

The SN may accept or reject the MN request to activate SCG.

If the SN accepts the SCG activation, it returns the SN modification request acknowledge message to the MN, including the SCG activation state. In case it wants to make a change in the SCG configuration, e.g. activate one or more SCells or change measurement configuration it can do this by including the SCG configuration. The MN then initiates the RRC reconfiguration procedure towards the UE, including the indication to activate the SCG and possible updates to the SCG configuration. Same as for any change in the SCG configuration, there may or may not be a need for the UE to perform random access towards the SCG as a result of the RRC reconfiguration. If the received RRC reconfiguration includes reconfiguration with sync for the SCG, UE shall perform the random access towards the SCG.

If the RRC reconfiguration triggering SCG activation includes reconfiguration with sync for the SCG, the UE performs random access towards the SCG.

If the received RRC reconfiguration does not include reconfiguration with sync for the SCG, whether UE performs random access or not during SCG activation will depend on whether the Time Alignment (TA) timer is still running and whether UE reported Beam Failure Detection (BFD) while the SCG was deactivated. The details of this is discussed in [3]. In short, the following shall apply:

- If the TA timer is still running upon SCG activation and no BFD was detected, the UE assumes it is UL synchronized, which enables the UE to access the PSCell without the need of random access.

- If TA timer has expired upon SCG activation or BFD was detected, the UE performs random access in the PSCell.

If reconfiguration with sync was not included for the SCG, if the TA timer is still running upon SCG activation and no BFD was detected, the UE assumes it is UL synchronized, which enables the UE to access the PSCell without the need of random access.

If reconfiguration with sync was not included for the SCG, if TA timer has expired upon SCG activation or BFD was detected, the UE performs random access in the PSCell.

### 2.2.2	SN initiated SCG activation

The activation of a deactivated SCG can also be initiated by the SN, e.g. in case of traffic arriving on SN terminated DRBs using the SCG. In this case, the SN could send an Activity Notification to the MN as shown in Figure 2 in order to make the MN aware of the changed traffic situation and the need to activate the SCG. However, for the case where the SN wants to make a change in the SCG configuration upon activation, e.g. to activate one or more SCells or change measurement configuration, there should also be the option for the SN to trigger the SN initiated SN modification procedure as shown in Figure 3 in order to avoid the extra handshake between MN and SN. Draft LS to RAN3 is provided in Annex B.

The SN can initiate SCG activation using the SN initiated SN modification required procedure.

Send LS to RAN3 that SN initiated SN modification procedure should also support SCG activation of deactivated SCG.

Figure 3. SN initiated activation of configured SCG

If the MN accepts the SCG activation, it sends the RRCReconfiguration to the UE, including the SCG activation state and possible updates to the SCG configuration and returns the SN modification confirm to the SN. The MN may also reject the SN modification as normal, e.g. in case MN has a prepared SN modification request procedure at the same time. In that case, the MN may respond with SN modification refuse to indicate unsuccessful operation.

The MN may accept or reject the SN request to activate the SCG.

The rest of the steps of the SN initiated SCG activation procedure are the same as for MN initiated SCG activation.

### 2.2.3	UE initiated SCG activation

For UE initiated activation of deactivated SCG, as discussed in RAN2#113e there procedure will depend on the DRB type.

- 8	Further discuss the comparison between

- -  define a mechanism for SCG activation upon UL data arrival on SCG bearers

- -	use split bearer with primary path on MCG (network sees UL data and can initiate activation)

As baseline, we consider the case of split DRB, for which UE initiated SCG activation can be handled with a combination of existing BSR reporting mechanism and the MN initiated SCG activation described in section 2.3.1, see Figure 4. This will minimize the standardization effort and provides the network full control of the SCG activation procedure and the possibility to update the SCG configuration if needed during the SCG activation, e.g. for allocating contention free resources in the SCG, in case random access is needed to establish uplink synchronisation.

For split DRB, UE initiated SCG activation is triggered by BSR reporting via MCG, based on which the network can decide to activate the SCG.

Figure 4. UE initiated activation of deactivated SCG

For the SCG DRB, BSR reporting via MCG is currently not possible and would require significant standardization effort. Buffer status for SCG DRBs could be reported via MCG either on RRC or MAC. On RRC, the UE assistance information message could be extended to inform the network of traffic on the SCG DRBs. This could be just an indication to request SCG activation or include also the buffer status. On MAC, something similar could be achieved, though it would break the MAC level independence between MCG and SCG that has so far been maintained for dual connectivity. Therefore, out of these we prefer the RRC approach, but least specification work would be to agree that SCG DRB is not supported for deactivated SCG, meaning that SCG for which a SCG DRB is configured should not be deactivated by the network. We think the SCG deactivation as a feature is more geared towards the split DRB, which flexibly can utilize MCG or SCG, and thus quickly adapt between SCG being activated or deactivated.

SCG DRB is not supported for deactivated SCG.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Reduced processing time is defined for RRCResume in both 36.331 and 38.331.

Observation 2	If the RRC reconfiguration triggering SCG activation includes reconfiguration with sync for the SCG, the UE performs random access towards the SCG.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Define a reduced processing time for RRCReconfiguration for activating SCG with limited or no change to the SCG configuration.

Proposal 2	Send LS to RAN4 to confirm whether Tprocessing = 0ms could be assumed for SCG activation, without cell or frequency change.

Proposal 3	The SN may accept or reject the MN request to activate SCG.

Proposal 4	If reconfiguration with sync was not included for the SCG, if the TA timer is still running upon SCG activation and no BFD was detected, the UE assumes it is UL synchronized, which enables the UE to access the PSCell without the need of random access.

Proposal 5	If reconfiguration with sync was not included for the SCG, if TA timer has expired upon SCG activation or BFD was detected, the UE performs random access in the PSCell.

Proposal 6	The SN can initiate SCG activation using the SN initiated SN modification required procedure.

Proposal 7	Send LS to RAN3 that SN initiated SN modification procedure should also support SCG activation of deactivated SCG.

Proposal 8	The MN may accept or reject the SN request to activate the SCG.

Proposal 9	For split DRB, UE initiated SCG activation is triggered by BSR reporting via MCG, based on which the network can decide to activate the SCG.

Proposal 10	SCG DRB is not supported for deactivated SCG.

# 4	References

R2-2101095, On the need for random access during SCG activation, Ericsson, 3GPP TSG-RAN WG2 #113e

R2-2101969, Summary of [AT113-e][230][eDCCA] Solution alternatives for SCG activation and deactivation, Huawei

R2-2103808, “UE measurements and reporting in deactivated SCG”, Ericsson, 3GPP TSG-RAN WG2 #113bis-e, Electronic meeting, April 12th – 20th, 2021

# Annex A: Draft LS to RAN4 on Tprocessing for SCG activation

3GPP TSG-RAN WG2 #113bis-e	R2-21xxxxx

Electronic meeting, 12th April – 20th April, 2021

Title:	LS on Tprocessing for SCG activation

Release:	Rel-17

Work Item:	LTE_NR_DC_enh2-Core

Source:	RAN WG2

To:	RAN WG4

Cc:

Contact Person:

#### Name:	Stefan Wager

E-mail Address:	stefan.wager@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- Overall Description:

RAN2 has discussed the SCG activation delay and would like to confirm the following understanding with RAN4 regarding the physical layer processing Tprocessing:

- Tprocessing for SCG addition currently consists of

  - 20ms for loading software and

  - 20ms for initializing data structures and buffers.

- For SCG activation,

  - For the software loading it should be realistic to assume that it can be performed during SCG addition, and that the UE can keep the software loaded for the deactivated SCG. There would then be no need to reload the software during SCG activation, assuming SCG stays in the same frequency range.

  - For the initialization of data structures and buffers, it should be possible for UEs to keep the memory initialized for deactivated SCG, assuming the cell or frequency is not changed.

  - In summary, if there is no mobility for the deactivated SCG, it should be possible to assume Tprocessing = 0ms for SCG activation.

- Actions:

To RAN4:	RAN2 respectfully asks RAN4 to confirm the above RAN2 understanding.

- Date of Next TSG-RAN WG2 Meetings:

- TSG-RAN WG2 Meeting #114	2021-05-19 to 2021-05-27		E-Meeting

- TSG-RAN WG2 Meeting #115	2021-08-16 to 2021-08-27 (TBC)	E-Meeting

# Annex B: Draft LS to RAN3 on SN initiated SCG activation

3GPP TSG-RAN WG2 #113bis-e	R2-21xxxxx

Electronic meeting, 12th April – 20th April, 2021

Title:	LS on SN initiated SCG activation

Release:	Rel-17

Work Item:	LTE_NR_DC_enh2-Core

Source:	RAN WG2

To:	RAN WG3

Cc:

Contact Person:

#### Name:	Stefan Wager

E-mail Address:	stefan.wager@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- Overall Description:

RAN2 has discussed the procedures for SCG activation and concluded that for the case of SN initiated SCG activation it is beneficial to support SCG activation through the SgNB initiated SgNB Modification procedure in 36.423 for EN-DC and S-NG-RAN node initiated S-NG-RAN node Modification procedure in 38.423 for other MR-DC options, in order to avoid an extra handshake between MN and SN in case the SN wants to make a change in the SCG configuration upon SCG activation.

- Actions:

To RAN3:	RAN2 respectfully asks RAN3 to add the necessary fields for SCG activation to the SN initiated SN modification procedures in 36.423 and 38.423.

- Date of Next TSG-RAN WG2 Meetings:

- TSG-RAN WG2 Meeting #114	2021-05-19 to 2021-05-27		E-Meeting

- TSG-RAN WG2 Meeting #115	2021-08-16 to 2021-08-27 (TBC)	E-Meeting