3GPP TSG-RAN WG2 Meeting #113bis electronic	R2-2104019

Online, April 12 – April 20, 2021

Source:	CATT

Title:	Analysis on slice based RACH configuration

Agenda Item:	8.8.3

Document for:	Discussion and Decision

Introduction

In RAN#91-e meeting, the WID on enhancement of RAN Slicing for NR was agreed [1]. One of the objectives is:

- Support slice based RACH configuration, specify mechanisms and signalling including, for Mobile Originating cases [RAN2] - Configure separated PRACH configuration (e.g., transmission occasions of time-frequency domain and preambles) for slice or slice group - Configure RACH parameters prioritization (e.g., scalingFactorBI and powerRampingStepHighPriority) for slice or slice group - Determine how this works with existing functionality, which may include how to perform RACH type selection (e.g., 2-step and 4-step), support of RACH fall-back cases, handling of simultaneous configuration with similar functions such as legacy RA prioritization (e.g., MPS and MCS UEs).

In this contribution, we firstly identify the issues of PRACH configuration for slice or slice group. Then, how to configure RACH parameters prioritization is illustrated. Finally, how slice-based RACH configuration works with current functionality is discussed.

Discussion

- Configuration of PRACH corresponding to slice/slice group

From PRACH resource perspective, preamble and transmission occasions of time-frequency domain both can be used to isolate the PRACH resource per slice or slice group. For preamble solution, the total number of preamble is 64 and it is relatively few for the slice based PRACH isolation. Also some preambles are reserved for other purpose, e.g. CFRA, on demand SI and etc. So this issue will impact the configuration flexibility for slice-based RA. Several slices should use the same preamble due to lack of the preambles. But if the slice group concept is introduced, the impaction will be decreased. For the solution of transmission occasions of time-frequency domain, there is no any issue on the capacity and the flexibility of slice-based PRACH isolation. Therefore, we propose that:

Proposal 1: Support to configure the slice-based PRACH resources in both preamble and transmission occasions of time-frequency domain.

Since the supported slice number in one cell is large, if we apply the solutions per slice, there may be some issues, for example, the PRACH resource may be fragmented if isolate the PRACH rsource per slice. Therefore, the concept of slice group is identified in study phase for slice-based RACH configuration.

Meanwhile, if the slice configuration associated to RACH parameters is exposed in SIB, the security issues should be avoided. This is not expected.

Generally we may have two solutions for addressing this issue. Solution 1 is that defines the slice group and associated the PRACH to slice group. Solution 2 is that uses the existing UAC mechanism and associated the PRACH to access category.

Solution 1: Associate PRACH to slice/slice group info, while the slice group information is negotiated via NAS between UE and CN. In this solution, slice/slice group info is configured explicitly.

Regarding solution 1, some companies raise the concerns that it may have impacts on SA2 and CT1, which brings complexities to the specification work if negotiation between UE and CN is needed. We can send liaison to SA2 and CT1 to check the impacts.

Observation 1: Inform the UE about the slice group information may impact SA2 and CT1 specification

Solution 2: Associate the access category to PRACH resources and the association is sent to UE in SIB1. Since the mapping between Access category and S-NSSAI can be configured, so then the UE will know the relationship between PRACH resources and S-NSSAI.

In this solution, the mapping between access category and S-NSSAI are preconfigured via NAS. UE selects category based on the arrived slice info. Afterward, UE chooses the corresponding PRACH resource according to the access category. But if the UE performs initial access to the network, i.e. there is no configuration of mapping between category and S-NSSAI, UE can’t select the proper RA resources.

This solution may solve slice ID exposed security issue but it cannot avoid the PARCH resource fragmented

Observation 2: The UAC mechanism only solve the security issue

Proposal 2: Define the slice group for the slice-based RACH resources configuration

- Configuration of RACH prioritization parameters

In order to facilitate the UE fast access to the network, the network can configure parameters to prioritize RA especially for some procedures, e.g. BFR and HO. Besides, in Rel-16, it was also agreed to define higher RA priority for some access identities.

And it was also agreed to apply RACH prioritization parameters, i.e ra-Prioritization to slice or slice group. The parameters in IE ra-Prioritization, i.e. powerRampingStepHighPriority and scalingFactorBI can be reused for difference slice. Also the slice group concept can be used for the parameter configuration.

Proposal 3: The parameters powerRampingStepHighPriority and scalingFactorBI are reused for slice-based RACH prioritization.

- RA type selection

In our understanding, 2-step RA is beneficial on latency reduction. So it can be used to optimize fast access to the network for some slices, e.g. URLLC. In current specification, UE performs RA type selection by checking the RSRP threshold. One enhancement is to define new procedure for RA type selection. For example, network links the RACH resources of 2-step to delay sensitive slice/slice groups while RACH resource of 4-step to non-delay sensitive slice/slice groups. When traffic arrives at UE, it chooses the RACH resources according to the association. If the corresponding RACH resource is mapped to 2-step RA, UE selects 2-step RA. Otherwise, UE selects 4-steps. In this manner, network can restrict the UEs which initiate 2-step RA and reduce the collision rate in 2-step RA.

However, if the RSRP of the downlink pathloss reference is below msgA-RSRP-Threshold, even 2-step RA is selected, there is high possibility that UE fails in this 2-step RA attempt. This aggravates latency consumption conversely. Therefore, RSRP threshold is still necessary to guarantee the reliability.

When the number that UE fails 2-step RA exceeds msgA-TransMax’, UE can fallback to 4-step RA. But whether msgA-TransMax’ is the same value as msgA-TransMax of normal 2-step RACH can be further studied in stage3.

Proposal 4: Slice/slice group info can be associated with RA type resources to facilitate fast access of some delay sensitive traffic and be used for RA type selection.

- Handling of simultaneous configuration between RACH prioritization parameters for slice and similar functions

In Rel-16, it was agreed to introduce PRACH prioritization parameters for MPS and MCS. The UE will prioritize RA procedures for some Access Identities if configured. This procedure allows UE to fast access to network for these Access Identities. However, when slice based RACH prioritization parameters and RA procedures Access Identities are both configured, UE has the problem on how to select the RACH parameters. Regarding the solutions, the RA prioritization parameters for MPS and MCS should be higher priority than that of the slice-based RA prioritization. But there may be some corner cases where the MPS/MCS is in lower priority. Therefore, we suggest that one configurable solution which includes overwrite indication is applied to handle simultaneous configuration between RACH prioritization parameters for slice and similar functions.

Proposal 5: It is configurable whether RA prioritization parameters for MPS and MCS override the slice-based RA prioritization.

Conclusion

This contribution analyzed common issues of slice based RACH configuration. We conclude the following proposals:

Observation 1: Inform the UE about the slice group information may impact SA2 and CT1 specification

Observation 2: The UAC mechanism only solve the security issue

Proposal 1: Support to configure the slice-based PRACH resources in both preamble and transmission occasions of time-frequency domain.

Proposal 2: Define the slice group for the slice-based RACH resources configuration

Proposal 3: The parameters powerRampingStepHighPriority and scalingFactorBI are reused for slice-based RACH prioritization.

Proposal 4: Slice/slice group info can be associated with RA type resources to facilitate fast access of some delay sensitive traffic and be used for RA type selection.

Proposal 5: It is configurable whether RA prioritization parameters for MPS and MCS override the slice-based RA prioritization.

Reference

- RP-210912, New WID on enhancement of RAN Slicing for NR, CMCC, 3GPP TSG RAN Meeting #91e;

- 38.331, Radio Resource Control;