3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2104209

# Electronic Meeting 12- 20 April, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300> | CR | Draft | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage-2 CR for 2-step release with redirection without anchor change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2020-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Resume request responded with Release with Redirect, with UE context relocation, has been introduced in RAN2#113e while it is still not clear how the resume request responded with Release with Redirect, without UE context relocation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced new section with sequence chart for Resume request responded with Release with Redirect, without UE context relocation. Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: RRC Resume and Release with Redirect Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there are no inter-operability problems. 2. If the UE is implemented according to the CR and the network is not, there are no inter-operability problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Resume request responded with Release with Redirect, without UE context relocation will not be covered in Stage 2 specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS ... CR... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

#### 9.2.2.x	Resume request responded with Release with Redirect, without UE context relocation

The following figure describes a UE triggered NAS procedure responded by the network with a release with redirect, without UE context relocation.

Figure 9.2.2.X-1: Resume request responded with Release with Redirect, without UE Context relocation

1.	The UE resumes from RRC_INACTIVE, providing the I-RNTI allocated by the last serving gNB and appropriate cause value, e.g., RAN notification area update.

2.	The gNB, if able to resolve the gNB identity contained in the I-RNTI, requests the last serving gNB to provide UE Context, providing the cause value received in step 1.

3.	The last serving gNB stores received information to be used in the next resume attempt (e.g. C-RNTI and PCI related to the resumption cell), and responds to the gNB with the RETRIEVE UE CONTEXT FAILURE message including an encapsulated RRCRelease message. The RRCRelease message includes Suspend Indication and redirection information (the frequency layer the UE performs cell selection upon entering RRC_INACTIVE).

4.	The gNB forwards the RRCRelease message to the UE.

NOTE1:	Upon receiving the release with redirect, the higher layers trigger a pending procedure so the UE tries to resume again after cell selection.

End of change