3GPP TSG-RAN WG2 Meeting #113bis-e	R2- 2104303

Electronic meeting, April 12th - 20th, 2021

Agenda Item:	10.3

Source: 		Session Chair (Ericsson)

Title: 			Report eMTC breakout session

Document for:	Approval

General

Recording of voice or video at meetings is not used in 3GPP. This applies also to this e-Meeting. At this e-Meeting, no specific actions are taken to prevent the recording of web conferences. Companies that have concerns related to recordings, if any, may express those by email in the main meeting organizational thread [AT113bis-e][000]

Please see the following Tdocs for e-meeting guidance:

R2-2102600	Agenda for RAN2#113bis-e	Chairman	agenda

R2-2100351	3GPP TSG RAN WG2 Handbook (01/2021)	ETSI MCC	discussion

Time Schedule 
Please refer to the latest schedule in the RAN2 inbox on the public 3GPP servers.

Access Tools

HTTP Upload Tool:

ETSI IT has created a facility in Inbox and Inbox/Drafts folders on the public 3GPP servers to allow delegates to upload their documents using a web browser (however Internet Explorer is not yet supported). Open your browser and navigate to your chosen folder – for example,

https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_113bis-e/Inbox

Click the green button to log in using your EOL account. A panel will appear in the upper part of the screen and documents may be dragged and dropped onto this landing pad; this causes them to be uploaded to the folder.

Secure FTP:

Those e-delegates who prefer to use FTP-like access to our e-meeting Inbox & Draft folders but are concerned by their usernames and passwords being sent unencrypted over the internet, ETSI IT has fitted the server with FTPS (SSL) so delegates can connect from their favourite FTP client using the address: ftps.3gpp.org. Please enter your username and password when prompted.

Organizational

- Incoming LSs are noted by default. Contact companies should flag LSs that need to be replied from this meeting.

- Legacy topics will be treated by email only unless indicated explicitly. Please see the list of offline email discussions below.

- Rel-16 (draft) CRs and text proposals will be handled as part of the email discussion on the corresponding CR(s) or the ASN.1 review email discussion if associated with a RIL#.

- All organizational emails and notes will be shared over the following email discussion throughout both meeting weeks:

[AT113bis-e][400][eMTC/NB-IoT] Organizational Emre’s session

Scope:

- Share plans for the e-meeting and make announcements

- Share status of email discussions

- Share meeting minutes and agreements for review and endorsement

Deadline: Tuesday, April 20th 18:00 UTC

Status: Closed

List and Status of Offline Email Discussions

NOTE: The official kick off date for these email discussions are Monday April 12th at 7:00 UTC. The rapporteurs can share them on the reflector earlier, however companies are not required to participate before the official kick off date. The deadlines refer to the deadline for providing company comments unless stated otherwise.

- [AT113bis-e][401][eMTC R16] Paging DRX cycle (ZTE)

Status: Closed

Scope: Check whether the intention is agreeable and there is sufficient support
in principle; collect initial comments.

Updated scope after online discussion:

- how to capture the intention in Proposal 1, i.e., to conclude on the wording

  - check whether the change is needed also for Rel-15 and for the WI with the code: LTE_5GCN_connect-Core

- how to capture the intention in Proposal 2 for PNB calculation, i.e., to conclude on the wording

  - check whether a similar change is needed for i_s and wg calculation

  - check whether the change is needed also for Rel-15 for i_s calculation

- check whether RAN2 agrees with the intention to send a LS to RAN3 to indicate that ng-eNB(s), other than the anchor ng-eNB, in the RAN paging area need to know about the UE eDRX cycle value, UE specific DRX cycle value and the RAN paging cycle value to calculate, e.g., PNB, PF, i_s and wg.

  - if agreeable a draft LS

Intended outcome: Report in R2-2104388

Deadline: Tuesday, April 20th 8:00 UTC

- [AT113bis-e][402][eMTC R16] Timing of neighbor cell RSS-based measurements (Qualcomm)

Status: Closed

Scope: Check whether there is a need to reply the LS in RAN2 and draft a
potential LS reply assuming that there is sufficient support in principle, i.e.,
so that a draft reply LS would be available if RAN2 agrees to send a reply LS.

Updated scope after online discussion:

Draft an LS reply based on the comments received

Intended outcome: Draft LS reply in R2-2104389

Deadline: Tuesday, April 20th 8:00 UTC

- [AT113bis-e][403][eMTC R16] RSS based RSRQ (Huawei)

Status: Closed

Scope: Check whether RSRQ measurements should be defined for RSS,
collect initial comments and draft an LS reply.

Updated scope after online discussion:

Draft an LS reply based on the comments received

Intended outcome: Draft LS reply in R2-2104390

Deadline: Tuesday, April 20th 8:00 UTC

# 4	EUTRA corrections Rel-15 and earlier

See Appendix A for reference to Work items, work item codes and WIDs.

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.1	NB-IoT corrections Rel-15 and earlier

Documents in this agenda item will be handled in a break out session. Common NB-IoT/eMTC parts treated jointly with 4.2.

## 4.2	eMTC corrections Rel-15 and earlier

Documents in this agenda item will be handled in a break out session. Common NB-IoT/eMTC parts treated jointly with 4.1. No web conference is planned for this agenda item.

# 7	Rel-16 EUTRA Work Items

Essential corrections

## 7.2    Additional MTC enhancements for LTE

(LTE_eMTC5-Core; LTE_eMTC5-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed:  June 20; WID: RP192875;)

Documents in this agenda item will be handled in a break out session.

Some sub-items in 7.2 and 7.3 may be treated jointly.

### 7.2.1     General and Stage-2 corrections

Including incoming LSs

R2-2102651	LS on timing of neighbor cell RSS-based measurements (R4-2103657; contact: Qualcomm)	RAN4	LS in	Rel-16	LTE_eMTC5-Core	To:RAN1, RAN2	Cc:-

Noted

R2-2102653	LS related to RSS based RSRQ for LTE-MTC (R4-2103728; contact: Huawei)	RAN4	LS in	Rel-16	LTE_eMTC5-Core	To:RAN2	Cc:RAN1

Noted

### 7.2.2     Connection to 5GC corrections

Connection to 5GC for MTC and NB-IoT is treated jointly under this AI.

R2-2103361	Discussion on correction for paging DRX cycle determination	ZTE Corporation, Sanechips	discussion	LTE_eMTC5-Core	Late

R2-2104239	draft LS to RAN3 to clarify paging DRX cycle	ZTE Corporation, Sanechips	LS out	Rel-16	LTE_eMTC5-Core	To:RAN3	Late

R2-2104246	Correction on paging DRX cycle	ZTE Corporation, Sanechips	CR	Rel-16	36.304	16.3.0	0825	-	F	LTE_eMTC5-Core	Late

- [AT113bis-e][401][eMTC R16] Paging DRX cycle (ZTE)

Status: Started

Scope: Check whether the intention is agreeable and there is sufficient support
in principle; collect initial comments.

Intended outcome: Report in R2-2104385

Deadline: Thursday 2021-04-15 10:00 UTC

R2-2104385	Report of [AT113bis-e][401][eMTC R16] Paging DRX cycle (ZTE)	ZTE Corporation, Sanechips		discussion	LTE_eMTC5-Core

Proposal 1: The editorial change of moving the condition “if allocated by upper layers” from the back of parameter the default paging cycle to the back of parameter the UE specific paging cycle can be agreed.

Proposal 1a: This editorial change is also needed for R15 and under the WID LTE_5GCN_connect-Core.

- QC supports the proposal but thinks that RRC Inactive is broken from Rel-15 which may lead to non-overlapped POs.

- HW thinks it was intentional that it applies to all, not only to the default paging cycle.

RAN2 agrees with the intention, but exact wording needs further discussion.

Proposal 2: For UE in RRC_INACTIVE, to separately describe DRX cycle T determination for the following three cases: eDRX cycle is not configured, eDRX cycle with 512 radio frames is configured, eDRX cycle is configured and the value is not 512 radio frames.

RAN2 agrees with the intention, but exact wording needs further discussion.

Proposal 2a: For each of above cases, to separately describe DRX cycle T determination for PF calculation and for PNB, i_s and wg calculation. The legacy description is still applicable to DRX cycle T determination for PF calculation. But for PNB, i_s and wg calculation, the DRX cycle T determination should follow RRC_IDLE mode rule.

Proposal 3: For R15 UE in RRC_INACTIVE, to separately describe DRX cycle T determination for PF calculation and for i_s calculation. The legacy description is still applicable to DRX cycle T determination for PF calculation. But for i_s calculation, the DRX cycle T determination should follow RRC_IDLE mode rule.

Proposal 4: Send LS to RAN3 to indicate that other ng-eNB(s) (except the anchor ng-eNB) in the RAN paging area needs the information about the UE eDRX cycle, possible UE specific DRX cycle and the used RAN paging cycle. If not, other ng-eNB(s) cannot calculate the correct value for PNB, PF, i_s and wg and paging would be failed.

Continue the discussion in offline [401] with the scope below:

- how to capture the intention in Proposal 1, i.e., to conclude on the wording

  - check whether the change is needed also for Rel-15 and for the WI with the code: LTE_5GCN_connect-Core

- how to capture the intention in Proposal 2 for PNB calculation, i.e., to conclude on the wording

  - check whether a similar change is needed for i_s and wg calculation

  - check whether the change is needed also for Rel-15 for i_s calculation

- check whether RAN2 agrees with the intention to send a LS to RAN3 to indicate that ng-eNB(s), other than the anchor ng-eNB, in the RAN paging area need to know about the UE eDRX cycle value, UE specific DRX cycle value and the RAN paging cycle value to calculate, e.g., PNB, PF, i_s and wg.

  - if agreeable a draft LS

The outcome can be provided in R2-2104388. The deadline is extended to Tuesday, April 20th 8:00 UTC

- [AT113bis-e][401][eMTC R16] Paging DRX cycle (ZTE)

Status: Started

Scope: Check whether the intention is agreeable and there is sufficient support
in principle; collect initial comments.

Updated scope after online discussion:

- how to capture the intention in Proposal 1, i.e., to conclude on the wording

  - check whether the change is needed also for Rel-15 and for the WI with the code: LTE_5GCN_connect-Core

- how to capture the intention in Proposal 2 for PNB calculation, i.e., to conclude on the wording

  - check whether a similar change is needed for i_s and wg calculation

  - check whether the change is needed also for Rel-15 for i_s calculation

- check whether RAN2 agrees with the intention to send a LS to RAN3 to indicate that ng-eNB(s), other than the anchor ng-eNB, in the RAN paging area need to know about the UE eDRX cycle value, UE specific DRX cycle value and the RAN paging cycle value to calculate, e.g., PNB, PF, i_s and wg.

  - if agreeable a draft LS

Intended outcome: Report in R2-2104388

Deadline: Tuesday, April 20th 8:00 UTC

R2-2104388	Suggestions for agreeable stage-2 proposals	ZTE Corporation, Sanechips		discussion	LTE_eMTC5-Core

Proposal 1: The case that extended DRX value of 512 radio frames is configured by upper layers according to 7.3 (both in RRC_IDLE and RRC_INACTIVE) is a special case of eDRX and it should be handled accordingly in RRC_INACTIVE.

- QC thinks this is not correctly captured in the specifications. RAN paging cycle can be 1024 radio frames, but there is no PTW for this case.

- Huawei thinks there is PTW in idle mode for 1024 radio frames, but not for 512 radio frames. Ericsson agrees.

- Huawei wonders if we can confirm that this proposal is for RRC Inactive case when eDRX value is 512 radio frames.

Working assumption: The case that extended DRX value of 512 radio frames is configured by upper layers should be handled in RRC_INACTIVE.

Proposal 2: RAN2 discuss the impacts when RAN paging cycle is absent in air interface configuration. And further discuss whether this parameter needs to be always present by implementation if issue is identified.

Proposal 3: The CR in [R2-2104246] is postponed to next meeting, with consideration on whether R15 CR to eLTE/NR is needed.

Proposal 4: One-week discussion for LS to RAN3.

Postponed

### 7.2.3     Other corrections

Including corrections related to Mobile-terminated early data transmission (MT-EDT), Scheduling multiple DL/UL transport blocks, Quality report in Msg3, MPDCCH performance improvement using CRS, Improvements for non-BL UEs, Stand-alone deployment, Mobility enhancements, coexistence with NR and MTC specific topics. Corrections related to mobile-terminated early data transmission, scheduling multiple DL/UL transport blocks and coexistence with NR are treated jointly for MTC and NB-IoT under this AI.

R2-2103012	Draft reply LS on timing of neighbor cell RSS-based measurements	Qualcomm Incorporated	LS out	Rel-16	LTE_eMTC5-Core	To:RAN4, RAN1

- [AT113bis-e][402][eMTC R16] Timing of neighbor cell RSS-based measurements (Qualcomm)

Status: Started

Scope: Check whether there is a need to reply the LS in RAN2 and draft a
potential LS reply assuming that there is sufficient support in principle, i.e.,
so that a draft reply LS would be available if RAN2 agrees to send a reply LS.

Intended outcome: Report in R2-2104386

Deadline: Thursday 2021-04-15 10:00 UTC

R2-2104386	[AT113bis-e][402][eMTC R16] Timing of neighbor cell RSS-based measurements	Qualcomm Incorporated	discussion	Rel-16	LTE_eMTC5-Core

Proposal 1	RAN2 to discuss whether to respond to this LS from RAN4.

Proposal 2	If RAN2 decides to send a response LS from RAN4, then RAN2 discuss how to respond.

- HW agrees that RAN1 should reply, but there may be some aspects, e.g., reading neighbour cell SFN, that RAN2 can reply. ZTE thinks there is no need to reply in RAN2. QC agrees with HW that it would be good to reply. Sequans agrees with QC and HW.

We aim to reply the LS from RAN2 standpoint.

Continue the discussion in offline [402] to draft an LS reply. The draft LS can be provided in R2-2104389. The deadline is extended to Tuesday, April 20th 8:00 UTC

- [AT113bis-e][402][eMTC R16] Timing of neighbor cell RSS-based measurements (Qualcomm)

Status: Started

Scope: Check whether there is a need to reply the LS in RAN2 and draft a
potential LS reply assuming that there is sufficient support in principle, i.e.,
so that a draft reply LS would be available if RAN2 agrees to send a reply LS.

Updated scope after online discussion:

Draft an LS reply based on the comments received

Intended outcome: Draft LS reply in R2-2104389

Deadline: Tuesday, April 20th 8:00 UTC

R2-2104389	[Draft] Reply LS on timing of neighbor cell RSS-based measurements	Qualcomm Incorporated	LS out	Rel-16	LTE_eMTC5-Core	To:RAN4, RAN1

Replace “provide the follow feedback from RAN2” with “provide the following feedback from RAN2”

The LS is approved in R2-2104391 unseen with the change above.

R2-2103013	Whether to support RSRQ with RSS	Qualcomm Incorporated	discussion	Rel-16	LTE_eMTC5-Core

R2-2103491	RSRQ measurements when RSS is used	Huawei, HiSilicon	discussion	Rel-16	LTE_eMTC5-Core

R2-2104182	Consideration on LS related to RSS based RSRQ for eMTC	ZTE Corporation, Sanechips	discussion	Rel-16	LTE_eMTC5-Core

- [AT113bis-e][403][eMTC R16] RSS based RSRQ (Huawei)

Status: Started

Scope: Check whether RSRQ measurements should be defined for RSS,
collect initial comments and draft an LS reply.

Intended outcome: Report in R2-2104387

Deadline: Thursday 2021-04-15 10:00 UTC

R2-2104387 Offline 403 - RSRQ measurements when RSS is used	Huawei, HiSilicon	discussion	Rel-16	LTE_eMTC5-Core

Proposal 1: Indicate in the LS that option 1 is not preferred from RAN2 perspective.

- HW suggests providing a reasoning in the LS along with the indication that Option 1 is not preferred. QC thinks there is no need to mention why Option 1 is preferred.

- QC thinks it would be better to state that Option 2 is preferred if that is the intention. HW thinks we would need to state a reasoning. Sequans agrees with HW.

RAN2 will send a reply LS indicating that Option 1 is not preferred from RAN2 standpoint since RAN2 thinks it may have an impact on cell (re)-selection performance and behaviour, but it is up to RAN1 and RAN4 to decide.

Continue the discussion in offline [403] to draft an LS reply. The draft LS can be provided in R2-2104390. The deadline is extended to Tuesday, April 20th 8:00 UTC

- [AT113bis-e][403][eMTC R16] RSS based RSRQ (Huawei)

Status: Started

Scope: Check whether RSRQ measurements should be defined for RSS,
collect initial comments and draft an LS reply.

Updated scope after online discussion:

Draft an LS reply based on the comments received

Intended outcome: Draft LS reply in R2-2104390

Deadline: Tuesday, April 20th 8:00 UTC

R2-2104390	[draft] Reply LS related to RSS based RSRQ for LTE-MTC		Huawei, HiSilicon	LS out	Rel-16	LTE_eMTC5-Core	To:RAN4, RAN1

Replace “RA4 to decide” with “RAN4 to decide.”

The LS is approved in R2-2104392 unseen with the change above.

## Summary

### List of comebacks

None

# Agreed CRs

None

# LS out

R2-2104391	Reply LS on timing of neighbor cell RSS-based measurements	Qualcomm Incorporated		LS out	Rel-16	LTE_eMTC5-Core	To:RAN4, RAN1

R2-2104392	[draft] Reply LS related to RSS based RSRQ for LTE-MTC		Huawei, HiSilicon	LS out	Rel-16	LTE_eMTC5-Core	To:RAN4, RAN1

### List of email discussions

None