3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2104451

Online, April 12th - 20th 2021

Source: 	Document Rapporteur (Ericsson)

Title:	RAN2 agreements for Rel-17 NB-IoT and LTE-MTC

Agenda Item:	9.1.1

Document for:	Endorsement

# 1	Introduction

This document lists RAN2 agreements made for Rel-17 “Additional enhancements for NB-IoT and LTE-M” work item (WI code NB_IOTenh4_LTE_eMTC6; WID in RP-201306), until and including the RAN2#113bis-e, April 12th – 20th 2021.

NOTE:	Email agreements for CRs/specific topics are not listed in this document.

# 2	Additional enhancements for NB-IoT and LTE-MTC

## 2.1	Organizational

RAN2#111-e agreements: Will maintain a document similar to the one used in R16 for capturing agreements. The endorsed report can be provided in R2-2008309. [Post111-e][350][NBIOT/eMTC R17] Capture the agreements (Ericsson) - Scope: Capture the agreements. - Intended outcome: endorsed report in R2-2008309 - Deadline: Friday, 2020-09-04 13:00 UTC RAN2#112-e agreements: None (except for this document, see clause 5) RAN2#113-e agreements: None (except for this document, see clause 5) RAN2#113bis-e agreements: None (except for this document, see clause 5)

## 2.2	NB-IoT neighbour cell measurements and corresponding measurement triggering before RLF

RAN2#111-e agreements: Study current RLF procedure to understand the time taken to select the cell for access. Start and end points FFS. Support neighbour cell measurements in RRC_CONNECTED, at least for intra-frequency. - FFS inter-frequency - FFS whether measurements are done on the anchor carrier - FFS how neighbour cell measurement is triggered - FFS how to perform neighbour cell measurements Working assumption: Neighbour cell measurement results are not reported to the network in RRC_CONNECTED. - FFS whether and when other information can be sent RAN2#112-e agreements: Enhancements to the random-access procedure are not considered. The solution includes reduction of the time between declaration of RLF and the start of the random-access procedure (points C and D) FFS whether the solution includes reduction of the time between out-of-sync detection and declaration of RLF (points B and C) RAN2#113-e agreements: Neighbour cells measurement (detection and measurements) are performed only on the anchor carrier. The solution is optional R2-2102165 LS on neighbour cell measurement in NB-IoT RRC_CONNECTED state LS out Rel-17 To: RAN4. NB_IOTenh4_LTE_eMTC6-Core RAN2#113bis-e agreements: None

## 2.3	NB-IoT carrier selection based on the coverage level, and associated carrier specific configuration

RAN2#111-e agreements: Paging carrier selection Improvements based on CE level is considered Paging carrier selection Improvements based on DRX cycle may be considered - whether DRX cycle is considered as part of CE level (Rmax) or can be also considered separately Enhancements for NPRACH Carrier selection carrier may be considered Paging carrier selection Improvements solely based on WUS or GWUS is not considered FFS service based RAN2#112-e agreements: None RAN2#113-e agreements: Select between one of the options: - Option 1: UE selects a paging carrier based on a rule configured by the network - Option 2: NW configures a specific paging carrier Working assumption: For both options, when coverage changes, mechanism that requires UE to report the update of coverage is not introduced. RAN2#113bis-e agreements: None

## 2.4	Other

### NB-IoT 16-QAM for unicast in UL and DL

RAN2#113bis-e agreements: Working assumption: For the UE supporting 16-QAM, the L2 buffer size is 12000 bytes. Working assumption: Support of 16-QAM has separate UE capabilities for DL and UL

### LTE-MTC 14 HARQ processes in DL for HD-FDD Cat M1 UEs

RAN2#113bis-e agreements: 14 HARQ activation is configured by dedicated RRC signalling. Working assumption: No change to current L2 buffer size requirement

### LTE-MTC Max DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A only

RAN2#113bis-e agreements: DL TBS of 1736 bits is configured by dedicated RRC signalling. FFS: Whether to update L2 buffer size requirement

# 3	References: List of approved outgoing Rel-17 LSs

| Tdoc | Meeting | Title | WI | To | Cc |
| --- | --- | --- | --- | --- | --- |
| R2-2102165 | RAN2#113-e | LS on neighbour cell measurement in NB-IoT RRC_CONNECTED state | NB_IOTenh4_LTE_eMTC6-Core | RAN4 |  |

# 4	References: Reports from the breakout session

| Meeting | Date | Title | Tdoc |
| --- | --- | --- | --- |
| RAN2#111-e, Online | 17-28 August 2020 | Report NB-IoT breakout session | R2-2008127 |
| RAN2#112-e, Online | 2-13 November 2020 | Report NB-IoT breakout session | R2-2010707 |
| RAN2#113-e, Online | Jan 25 - Feb 5 2021 | Report NB-IoT breakout session | R2-2101957 |
| RAN2#113bis-e, Online | 12 - 20 April 2021 | Report NB-IoT breakout session | R2-2104307 |

# 5	Change history

| Tdoc | Title | Comment |
| --- | --- | --- |
| R2-2008309 | RAN2 agreements for Rel-17 additional enhancements for NB-IoT & LTE-MTC | Post RAN2#111-e |
| R2-2010911 | RAN2 agreements for Rel-17 additional enhancements for NB-IoT & LTE-MTC | Post RAN2#112-e |
| R2-2102164 | RAN2 agreements for Rel-17 additional enhancements for NB-IoT & LTE-MTC | Post RAN2#113-e |
| R2-2104451 | RAN2 agreements for Rel-17 additional enhancements for NB-IoT & LTE-MTC | Post RAN2#113bis-e |