3GPP TSG-RAN WG2 Meeting #113-bis-e	R2-2104843

Online, April 12th – April 20th 2021

Source:	vivo

Title:       Discussion on methodologies for network-assisted and UE-assisted integrity

Agenda Item:	8.11.5

Document for:	Discussion and Decision

- Introduction

At the RAN#91e meeting, the study item on NR Positioning Enhancements RP-210903[1] was approved. From RAN2’s perspective, the SI includes the following objective:

- Specify the signalling, and procedures to support GNSS positioning integrity determination, including [RAN2, RAN3]: - The assistance information that will be used to support integrity determination - The information that will be used to provide the positioning integrity KPIs and integrity results - Support of integrity for UE-based and UE-assisted A-GNSS positioning. Note: This objective is applicable to NR and E-UTRA.

In this contribution, we will discuss the methodologies for integrity in GNSS positioning methods.

- Discussion

Integrity discussion is summarized in R2-2104497[2]:

The contribution proposals have been sorted and structured into the following categories: - Liaising with external working groups/organization - QoS requirement related to integrity - Integrity for Automotive Use Case - Considerations on the information transferred for supporting GNSS positioning integrity - GNSS Feared Events - GNSS Assistance Information - Integrity KPIs - Integrity Results - Framework and Mechanisms for supporting GNSS positioning integrity - Guiding framework on integrity concepts - Selection of positioning integrity methods - Timer-related integrity event triggering at UE - Recovery from integrity failure condition - Integration of multiple positioning methods for integrity - Signalling and procedures for positioning integrity - Feasibility of reusing LPP procedures - Signalling to determine the positioning integrity capability - Signalling to deliver the KPIs and integrity results - Signalling to deliver the integrity assistance information to the UE/LMF - Signalling to deliver the integrity information related to the GNSS positioning measurements from the UE to the LMF - Overall Signalling Procedures

These’re many feared events identified by the email discussion, some can be received by 3rd location server like GNSS related feared events and some can be identified by UE itself. like UE feared events. To better organized these feared events and using them for positioning integrity methods, categories must be applied for analysis.

Similar with positioning measurement, positioning integrity can be divided into two parts: network-assisted integrity and UE-assisted integrity. Network-assisted integrity is UE-based integrity, where UE application or service request high requirement for human safety. UE-assisted integrity is UE provided integrity assisted data and integrity calculated by LMF or location server.

Observation 1: feared event can be categorized for network-assisted and UE-assisted integrity.

Different with positioning measurement, positioning integrity has no relationship with measurement methods. For GNSS, integrity can be validated by UE or by LMF. A-GNSS can support both UE-assisted and network-assisted integrity. Even for RAT-dependent positioning methods, they are also not limited one method to network-assisted or UE-assisted.

Observation 2: Positioning methods are decoupled with network-assisted and UE-assisted integrity.

For network-assisted integrity, assisted-data should be transmitted from LMF to UE. And for UE-assisted integrity assisted-data should be transmitted from UE to LMF. Whether choose network-assisted or UE-assisted integrity is based on UE and network capability and service requirement. LMF who has both the capability of UE and network and has connection interface with 3rd part location server can better decide it. Because positioning methods are decoupled with network-assisted and UE-assisted integrity. When in the future we support RAT-depended positioning methods, the structure which LMF decides also works.

Proposal 1: LMF decides whether to choose network-assisted or UE-assisted integrity.

The integrity functions can be integrated into UE or network as an integrity capability. If UE has integrity function where LMF not(legacy network, enhanced UE), then network-assisted integrity should be chosen. If network has integrity function where UE not(legacy UE, enhanced network), then UE-assisted integrity should be chosen.

Table 1 integrity methods choice

| Methods chosen for integrity | Legacy network | Enhanced network |
| --- | --- | --- |
| Legacy UE. | No integrity service provided. | UE-assisted integrity |
| Enhanced UE | Network-assisted integrity | Both |

For this purpose, integrity capability must be transmitted as a kind of assisted data and when and how to transmit it can be considered in WI.

Proposal 2: Integrity capability should be studied in WI.

During the email discussion the following signalling are identified for consideration in the WI:

- Signaling to determine the positioning integrity capability

- Signaling to the deliver the KPIs and integrity results

- Signaling to deliver the integrity assistance information to the UE

- Signaling to deliver the integrity information related to the GNSS positioning measurements from the UE to the LMF

Therefore, these messages can be considered for integrity in WI as well:

- RequestCapabilities

- ProvideCapabilities

- RequestAssistanceData

- ProvideAssistanceData

- RequestLocationInformation

- ProvideLocationInformation

Where integrity capability can be transmitted by LPP capability and feared events can be transmitted by LPP assistance data and integrity result can be studied in location information.

Observation 3: It is feasible that Integrity information can be transmitted by existed signal modification without architecture change and new interface introduced.

One example is described in the Figure-1 below for network-assisted integrity.

Figure-1 Integrity assistant data transmit by LPP

Step 1.  If we choose the GNSS related positioning method, the GNSS system needs to provide assistant data to LMF.

Step 2.  If the positioning method needs gNB assistant, gNB needs to provide assistant data to LMF.

Step 3.  If LMF decides to choose to provide assistant data to UE by dedicated signaling, it sends assistance data to UE via LPP.

Step 4.  If LMF decides to provide assistant data to UE by broadcast, it sends assistant data to gNB.

Step 5.  gNB broadcasts assistant data to UE.

Step 6.  UE uses the assistant data to calculate PL.

Proposal 3: To study whether integrity information can be transmitted by existed signal modification without architecture change and new interface introduced.

- Conclusion

Based on the discussion, we observe and propose the following:

Observation 1: feared event can be categorized for network-assisted and UE-assisted integrity.

Observation 2: Positioning methods are decoupled with network-assisted and UE-assisted integrity.

Proposal 1: LMF decides whether to choose network-assisted or UE-assisted integrity.

Proposal 2: Integrity capability should be studied in WI.

Observation 3: Integrity information can be transmitted by existed signal modification without architecture change and new interface introduced.

Proposal 3: To study whether integrity information can be transmitted by existed signal modification without architecture change and new interface introduced.

- Reference

- RP-210903-(rev.fr.RP-210897)-clean

- [AT113-e][601][POS] Integrity text proposal (Swift)

- TR 38.875, “Study on NR Positioning Enhancements”, v2.0.0(2021-03)

- Annex

In TS38.857[3]

- 9.4 	Positioning Integrity Methods

- 9.4.1		RAT-Independent

- 9.4.1.1		A-GNSS Positioning Integrity Methods

Similar with positioning measurement, positioning integrity can be divided into two parts: network-assisted integrity and UE-assisted integrity. Network-assisted integrity is UE-based integrity, where UE application or service request high requirement for human safety. UE-assisted integrity is UE provided integrity assisted data and integrity calculated by LMF or location server.

For network-assisted integrity, assisted-data should be transmitted from LMF to UE. And for UE-assisted integrity assisted-data should be transmitted from UE to LMF. Whether choose network-assisted or UE-assisted integrity is based on UE and network capability and service requirement. LMF who has both the capability of UE and network and has connection interface with 3rd part location server can better decide it.

The integrity functions can be integrated into UE or network as an integrity capability. If UE has integrity function where LMF not(legacy network, enhanced UE), then network-assisted integrity should be chosen. If network has integrity function where UE not(legacy UE, enhanced network), then UE-assisted integrity should be chosen.

Table 1 integrity methods choice

| Methods to choose for integrity | Legacy network | Enhanced network |
| --- | --- | --- |
| Legacy UE. | No integrity service provided. | UE-assisted integrity |
| Enhanced UE | Network-assisted integrity | Both |

Integrity capability is FFS.

9.4.1.1.1 Signaling considerations

These messages can be considered for integrity in WI.

- RequestCapabilities

- ProvideCapabilities

- RequestAssistanceData

- ProvideAssistanceData

- RequestLocationInformation

- ProvideLocationInformation