3GPP TSG-RAN WG2 Meeting #114-e	R2-2104918

Electronic, 19th - 27th May, 2021

Source: 			Qualcomm Incorporated

Title: 				NR-DC cell grouping UE capability signalling

Document for: 	Decision

Agenda Item: 	6.5.2

# Introduction

In [1], RAN4 expressed their concerns on RAN2 technically endorsed CRs introducing Cell Grouping UE capability for NR-DC [2][3]. RAN2 therefore is required to reconsider their endorsement of those CRs.

In this document we propose to adopt signalling solution which is based on “PUCCH grouping” UE capability signalling.

# Discussion

  - PUCCH grouping

UE capability signalling for PUCCH grouping was introduced as follows.

twoPUCCH-Grp-ConfigurationsList-r16 SEQUENCE (SIZE (1..maxTwoPUCCH-Grp-ConfigList-r16)) OF TwoPUCCH-Grp-Configurations-r16 TwoPUCCH-Grp-Configurations-r16 ::= SEQUENCE { pucch-PrimaryGroupMapping-r16 TwoPUCCH-Grp-ConfigParams-r16, pucch-SecondaryGroupMapping-r16 TwoPUCCH-Grp-ConfigParams-r16 } TwoPUCCH-Grp-ConfigParams-r16 ::= SEQUENCE { pucch-GroupMapping-r16 PUCCH-Grp-CarrierTypes-r16, pucch-TX-r16 PUCCH-Grp-CarrierTypes-r16 } PUCCH-Grp-CarrierTypes-r16 ::= SEQUENCE { fr1-NonSharedTDD-r16 ENUMERATED {supported} OPTIONAL, fr1-SharedTDD-r16 ENUMERATED {supported} OPTIONAL, fr1-NonSharedFDD-r16 ENUMERATED {supported} OPTIONAL, fr2-r16 ENUMERATED {supported OPTIONAL } NOTE: twoPUCCH-Grp-ConfigurationsList-r16 is signalled per band combination.

RAN1 previously recommended reusing the PUCCH grouping signalling for NR-DC cell grouping [4]. Indeed, the signalling structure is highly reusable for NR-DC cell grouping when the following logical replacements are assumed.

- PUCCH primary group is replaced by MCG.

- PUCCH secondary group is replaced by SCG.

- PUCCH TX placement is replaced by spCell placement.

Proposal 1:	Reuse the PUCCH grouping signalling for NR-DC cell grouping with the following logical replacements.

- PUCCH primary group is replaced by MCG.

- PUCCH secondary group is replaced by SCG.

- PUCCH TX placement is replaced by spCell placement.

  - spCell placement

It should be noted that the current specification already defines a UE capability for spCell placement. i.e. spCellPlacement (ASN.1 type definition in CarrierAggregationVariant).

#### 4.2.7.13 CarrierAggregationVariant Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. fr1fdd-FR1TDD-CA-SpCellOnFR1FDD Indicates whether the UE supports an FR1 FDD SpCell (and possibly SCells) when configured with an FR1 TDD SCell.; UE; No; No; No. fr1fdd-FR1TDD-CA-SpCellOnFR1TDD Indicates whether the UE supports an FR1 TDD SpCell (and possibly SCells) when configured with an FR1 FDD SCell.; UE; No; No; No. fr1fdd-FR1TDD-FR2TDD-CA-SpCellOnFR1FDD Indicates whether the UE supports an FR1 FDD SpCell (and possibly SCells) when configured with an FR1 TDD SCell and an FR2 TDD SCell.; UE; No; No; No. fr1fdd-FR1TDD-FR2TDD-CA-SpCellOnFR1TDD Indicates whether the UE supports an FR1 TDD SpCell (and possibly SCells) when configured with an FR1 FDD SCell and an FR2 TDD SCell.; UE; No; No; No. fr1fdd-FR1TDD-FR2TDD-CA-SpCellOnFR2TDD Indicates whether the UE supports an FR2 TDD SpCell (and possibly SCells) when configured with an FR1 FDD SCell and an FR1 TDD SCell.; UE; No; No; No. fr1fdd-FR2TDD-CA-SpCellOnFR1FDD Indicates whether the UE supports an FR1 FDD SpCell (and possibly SCells) when configured with an FR2 TDD SCell.; UE; No; No; No. fr1fdd-FR2TDD-CA-SpCellOnFR2TDD Indicates whether the UE supports an FR2 TDD SpCell (and possibly SCells) when configured with an FR1 FDD SCell.; UE; No; No; No. fr1tdd-FR2TDD-CA-SpCellOnFR1TDD Indicates whether the UE supports an FR1 TDD SpCell (and possibly SCells) when configured with an FR2 TDD SCell.; UE; No; No; No. fr1tdd-FR2TDD-CA-SpCellOnFR2TDD Indicates whether the UE supports an FR2 TDD SpCell (and possibly SCells) when configured with an FR1 TDD SCell.; UE; No; No; No.

Unfortunately, the carrier types used in CarrierAggregationVariant is different from those used in PUCCH grouping. We propose to use the carrier types from PUCCH grouping to retain consistency with carrier types used for cell grouping. It can be specified that the UE capability parameter spCellPlacement is disregarded by the network for NR-DC band combinations where the new NR-DC cell grouping UE capability is provided.

Proposal 2:	Specify that the UE capability parameter spCellPlacement is disregarded by the network for NR-DC band combinations where the new NR-DC cell grouping UE capability is provided.

  - Sync vs Async

RAN2 agreed previously agreed to introduce NR-DC cell grouping signalling separately for synchronous and asynchronous NR-DC, which is reflected in the technically endorsed CRs [2][3]. We propose to keep the same principle.

Proposal 3:	Introduce NR-DC cell grouping signalling separately for synchronous and asynchronous NR-DC.

# Conclusion

In this document, we made the following proposals on NR-DC cell grouping UE capability signalling.

Proposal 1:	Reuse the PUCCH grouping signalling for NR-DC cell grouping with the following logical replacements.

- PUCCH primary group is replaced by MCG.

- PUCCH secondary group is replaced by SCG.

- PUCCH TX placement is replaced by spCell placement.

Proposal 2:	Specify that the UE capability parameter spCellPlacement is disregarded by the network for NR-DC band combinations where the new NR-DC cell grouping UE capability is provided.

Proposal 3:	Introduce NR-DC cell grouping signalling separately for synchronous and asynchronous NR-DC.

# References

[1]		R2-2104723 	Reply LS on Introduction of Cell Grouping UE capability for NR-DC (R4-2105333)		RAN4

[2]		R2-2102210	Release-16 CR to 38.331 on Introduction of Cell Grouping UE capability for NR-DC	 Qualcomm Incorporated

[3]		R2-2102211	Release-16 CR to 38.306 on Introduction of Cell Grouping UE capability for NR-DC	 Qualcomm Incorporated

[4]		R1-2009570		LS reply on cell-grouping UE capability for synchronous NR-DC	RAN1 (To RAN2)