3GPP TSG RAN WG2 #114-e                                                         R2-2104925

e-Meeting, May 19th – 27th, 2021

Source: 	Intel Corporation

Title: 	Support of angle calculation enhancement for DL-AoD

Agenda item:	8.11.1

Document for: 	Discussion and Decision

# Introduction

RAN1 sent LS [1] to RAN2, and ask RAN2 to consider the option of angle report from gNB to LMF for UE-A DL-AoD in Rel-17.

Agreement: Regarding support of angle calculation enhancement for DL-AoD: - Support gNB providing the beam/antenna information to the LMF. - The gNB beam/antenna information can be provided to the UE for UE-based DL-AoD - FFS: the details of contents of the beam/antenna information - FFS: the details of how to provide the beam/antenna information. - Note: The antenna information is related to reducing the overhead of beam information - Send an LS to RAN2/RAN3 regarding the option of angle report from gNB to LMF for UE-A DL-AoD requesting them to consider this option in Rel-17.

In this contribution, we discuss how to support it from RAN2 perspective.

Discussion

To improve the DL-AoD calculation, RAN1 agreed to take antenna information into account when calculating DL-AoD results. Two options were on the table in RAN1:

Alt.1-1 Providing the gNB beam/antenna information to the LMF (for UE-A) or to the UE (for UE-B).

Alt.1-2: DL-AoD estimate report from gNB to LMF.

Alt 1-2 is applied for UE assisted DL-AoD. The main difference between two options is whether beam/antenna information should be shared with LMF. If not, then the gNB shall provide the angle report to the LMF.

To our understanding, considering that beam/antenna system information may be a proprietary design option, the sharing of its characteristic may not be always possible or welcomed, therefore the option where DL-AoD calculated directly at gNB should be supported. Beside above consideration, this option can provide much more freedom for implementation of DL-AoD estimation and design of the antenna system and TX beamforming algorithms at gNB side that can be completely transparent to UE/LMF. In addition, Alt.1-2 does not require extra specification efforts in RAN1 working group.  In addition, in terms of signaling details for beam/antenna information sharing to LMF/UE it may be quite challenging to cover all possible implementation options of gNB antenna system and beamforming implementation.

Based on offline discussion [2], RAN1 agreed to send LS to RAN2/3 and ask us to consider this option in Rel-17.

From RAN2 perspective, to support Alt 1-2:

- UE receives DL-PRS resources, performs measurements (existing LPP procedure)

- UE reports measurement results to LMF (existing LPP procedure)

- LMF sends the measurement results to gNB (new NRPPa procedure)

- gNB estimates DL-AoD and reports it to LMF (new NRPPa procedure)

- LMF computes the coordinates

- To support the UE-Assisted DL-AoD solution, where the gNB/TRP beam/antenna assistance information is not shared with the LMF/UE, the following procedure can be used:

  - UE receives DL-PRS transmission and performs measurements on DL PRS resources

  - UE reports measurements to LMF

  - LMF sends UE measurement results to gNB (new NRPPa procedure)

  - gNB estimates DL-AoD and reports it to LMF (new NRPPa procedure)

  - LMF computes the coordinates based on provided DL-AoD estimates

Note: UE measurements details are under RAN1 discussion.

To support DL-AoD, the PRS may be transmitted from multiple TRPs, and may be from different gNBs.

Scenario 1: all DL PRS are from the same gNB (may contain multiple TRPs)

Scenario 1 is typical scenario for industry IOT scenario. To calculate DL-AoD, the LMF only needs to forward the UE measurement results for DL-AoD to the corresponding gNB based on cell ID information.

Scenario 2: all DL PRS are from different gNBs (may contain multiple TRPs)

For scenario 2, the LMF needs to forward the UE measurement results for DL-AoD to the corresponding gNBs based on cell ID information, and gets the angle report from multiple gNBs.

- If the UE measurement results for DL-AoD are from different gNBs, the LMF can forward the UE measurement results for DL-AoD estimates to the corresponding gNBs based on cell ID information and get the DL-AoD angle report from multiple gNBs.

Conclusions

In this contribution, we discuss the support of angle calculation enhancement for DL-AoD, and have following list of proposals:

- To support the UE-Assisted DL-AoD solution, where the gNB/TRP beam/antenna assistance information is not shared with the LMF/UE, the following procedure can be used:

  - UE receives DL-PRS transmission and performs measurements on DL PRS resources

  - UE reports measurements to LMF

  - LMF sends UE measurement results to gNB (new NRPPa procedure)

  - gNB estimates DL-AoD and reports it to LMF (new NRPPa procedure)

  - LMF computes the coordinates based on provided DL-AoD estimates

Note: UE measurements details are under RAN1 discussion.

- If the UE measurement results for DL-AoD are from different gNBs, the LMF can forward the UE measurement results for DL-AoD estimates to the corresponding gNBs based on cell ID information, and get the DL-AoD angle report from multiple gNBs.

References

[1] R1-2104089 LS on DL-AoD angle calculation enhancement, RAN1

[2] R1-2104047	FL summary #2 for AI 8.5.3 Accuracy improvements for DL-AoD positioning solutions	Moderator (Ericsson)