3GPP TSG-RAN WG2 Meeting #114 electronic	R2-2105329

# Online, 19th – 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.314 | CR | 0015 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the range of PER and the description of D2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2021-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The concept of D2.1 (over-the-air packet delay) given in the Table 4.2.1.2.2-1 defines the end time of D2.1 as ‘the point in time it takes to successfully receive a transport block’, which is not aligned with Table 4.2.1.2.2-2 for the following reason: - Transport block (TB) is a concept used for PHY sublayer, but D2.1 is the delay between MAC@transmitter and RLC@receiver, instead of PHY@receiver, the use of TB introduces ambiguity as it is unclear what the ‘it’ refers to; It is suggested to modify the definition given in Table 4.2.1.2.2-1 in accordance with Table 4.2.1.2.2-2. - According to the latest TS 23.501 V17.0.0 (2021-03), the range of PER vary from 10-8 to 10-2, but the current description on the range of PER is 10-6 to 10-2 #### TS 23.501 5.7.4 Standardized 5QI to QoS characteristics mapping Table 5.7.4-1: Standardized 5QI to QoS characteristics mapping (part) 5QI Value; Resource Type; Default Priority Level; Packet Delay Budget (NOTE 3); Packet Error Rate; Default Maximum Data Burst Volume (NOTE 2); Default Averaging Window; Example Services. 73; 56; 300 ms (NOTE 11, NOTE 13, NOTE 15); 10-8; N/A; 2000 ms; "Live" Uplink Streaming (e.g. TS 26.238 [76]).. |
| Summary of change: |  | - Modify the definition for D2.1 in Table 4.2.1.2.2-1 - from ‘This measurement provides the average (arithmetic mean) time it takes to successfully receive a transport block from the time of UL transmission indicated in scheduling grant.’ - to ‘This measurement provides the average (arithmetic mean) time it takes from the time of UL transmission indicated in scheduling grant to the time that the MAC SDUs are successfully sent to RLC.’ - Revise the range of PER from 10-8 to 10-2, Impact analysis Impacted functionality: SON and MDT Inter-operability: - If UE implements according to the CR and the network does not, there is no impact foreseen. - If the network implements according to the CR and the UE does not, there is no impact foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The contents between different specifications are not aligned, which cause further confusions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.2.2 Average over-the-air interface packet delay in the UL per DRB per UE 4.2.1.5.1 Packet Uu Loss Rate in the DL per DRB per UE |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

4.2.1.2	Packet delay

4.2.1.2.2	Average over-the-air interface packet delay in the UL per DRB per UE

The objective of this measurement is to measure air interface UL packet delay for OAM performance observability or for QoS verification of MDT or for the QoS monitoring as defined in TS 23.501 [4].

Protocol Layer: MAC

Table 4.2.1.2.2-1: Definition for Average over-the-air packet delay in the UL per DRB per UE

| Definition | Average over-the-air packet delay in the UL per DRB per UE. This measurement is applicable for EN-DC and SA. This measurement refers to packet delay for DRBs. This measurement provides the average (arithmetic mean) time it takes from the time of UL transmission indicated in scheduling grant to the time that the MAC SDUs are successfully sent to RLC. Detailed Definition: M(T,drbid)=∀itSucc(i,drbid)-tSched(i,drbid)I(T), where explanations can be found in the table 4.2.1.2.2-2 below. |
| --- | --- |

Table 4.2.1.2.2-2: Parameter description for Average over-the-air packet delay in the UL per DRB per UE

| M(T,drbid) | Over-the-air packet delay in the UL per DRB per UE, averaged during time period T. Unit: 0.1 ms. |
| --- | --- |
| tSched(i,drbid) | The point in time when the UL MAC SDU i is scheduled in MAC layer as per the scheduling grant provided. |
| tSucc(i,drbid) | The point in time when the UL MAC SDU i is successfully sent to RLC. |
| i | A UL MAC SDU that arrives at the MAC during time period T. |
| I(T) | Total number of UL MAC SDUs i. |
| T | Time Period during which the measurement is performed |
| drbid | The identity of the measured DRB. |

NEXT CHANGE

4.2.1.5	Packet Loss Rate

4.2.1.5.1	Packet Uu Loss Rate in the DL per DRB per UE

The objective of this measurement is to measure packets that are lost at Uu transmission, for OAM performance observability.

Protocol Layer: RLC

Table 4.2.1.5.1-1: Definition for Packet Uu Loss Rate in the DL per DRB per UE

| Definition | Uu Packet Loss Rate in the DL per DRB per UE. One packet corresponds to one RLC SDU. The measurement is done separately per DRB. Detailed Definition: M(T,drbid)=Dloss(T,drbid)*1000000N(T,drbid)+Dloss(T,drbid), where explanations can be found in the table 4.2.1.5.1-2 below. |
| --- | --- |

NOTE 1:	Packet loss is expected to be upper bounded by the PER (packet error rate, as defined in TS 23.501 [4]) of the DRB which takes values between 10-8 and 10-2. The statistical accuracy of an individual packet loss rate measurement result is dependent on how many packets have been received, and thus the time for the measurement.

NOTE 2:	The granularity for Packet loss rate measurement is per DRB per UE, as defined in TS 28.552 [2].

Table 4.2.1.5.1-2: Parameter description for Packet Uu Loss Rate in the DL per DRB per UE

| M(T,drbid) | Packet Loss Rate in the DL per DRB per UE. Unit: number of lost packets per transmitted packets per DRB * 106, Integer. |
| --- | --- |
| Dloss(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, for which at least a part has been transmitted over the air but not positively acknowledged, and it was decided during time period T that no more transmission attempts will be done. If transmission of a packet might continue in another cell, it shall not be included in this count. |
| N(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, which has been transmitted over the air and positively acknowledged during time period T. |
| T | Time Period during which the measurement is performed, Unit: minutes. |
| drbid | The identity of the measured DRB. |

END OF CHANGES