3GPP TSG-RAN WG2 Meeting #114-e                                  R2-2105421

Online, May 19 – May 27, 2021

Agenda item:	6.1.4.1.2

Source:	Samsung

Title:	Discussion on CGI reporting for NPN-only cell

Document for:	Discussion & Decision

# 1.	Introduction

In the previous meeting, RAN2 discussed CGI reporting for NPN-only cell but it was postponed as below.

R2-2103169	Clarification on NPN related CGI report	Huawei, CMCC, China Telecom, China Unicom, HiSilicon	CR	Rel-16	38.331	16.4.1	2501	-	F	NG_RAN_PRN-Core

-	[020] Rap: It’s suggested to have a further discussion on NPN related CGI reporting in the next RAN2 meeting by taking into account companies’ comments (including avoiding impacts to non-NPN-capable UEs, limiting the impacts to NPN-only cells with presence of npn-IdentityInfoList, etc.)

[020] Postponed

This contribution discusses the way forward.

# 2.	Discussion

In [1][2], RAN3 agreed that in case of NPN-only cell, the PLMN identities and associated information contained in the PLMN-IdentityInfoList IE are not included in the Xn and F1 interfaces to set the Broadcast PLMN Identity Info List appropriately.

Observation 1: RAN3 agreed that in case of NPN-only cell, the PLMN identities and associated information contained in the PLMN-IdentityInfoList IE are not included in the Xn and F1 interfaces to set the Broadcast PLMN Identity Info List appropriately.

To address above, [3] proposed that all the UEs report the plmn-IdentityInfoList only when the cellReservedForOtherUse is not set to true as part of CGI reporting procedure. However, if nr-CGI-Reporting-NPN is not supported by UE, UE is not required to detect whether the concerned cell is NPN-only cell or not as part of CGI reporting procedure.

Observation 2: UE not supporting nr-CGI-Reporting-NPN is not required to detect whether the concerned cell is NPN-only cell or not as part of CGI reporting procedure.

Thus, as same as R15 UE behaviors, R16 UE not supporting nr-CGI-Reporting-NPN should not selectively report the obtained PLMN-IdentityInfoList IE from the indicated NR cell depending on the value of cellReservedForOtherUse IE.

Proposal 1: Confirm that UE not supporting nr-CGI-Reporting-NPN shall report the obtained PLMN-IdentityInfoList IE from the indicated NR cell as part of CGI reporting procedure irrespective of the value of cellReservedForOtherUse IE.

If nr-CGI-Reporting-NPN is supported by UE and the concerned NR cell broadcasts NPN-IdentityInfoList IE in SIB1, UE can determine that the concerned NR cell is NPN-only cell by detecting that the cellReservedForOtherUse IE is set to true. In case of NPN-only cell scenario, we think it would be more logical and cleaner to report the value of cellReservedForOtherUse IE to indicate the cell is NPN-only cell.

Proposal 2: UE supporting nr-CGI-Reporting-NPN reports the cellReservedForOtherUse IE as part of CGI reporting procedure if the concerned cell is NPN-only cell.

If Proposal 2 is agreed, we think that the feature of reporting the cellReservedForOtherUse IE is conditionally mandatory for UEs supporting nr-CGI-Reporting-NPN. Otherwise, UE may not even implement this feature so that the issue still remains i.e. gNB is not able to determine whether the concerned cell is NPN-only cell or not. RAN2 can further discuss whether additional capability bit is needed or not.

Proposal 3: The feature of reporting the cellReservedForOtherUse IE as part of CGI reporting procedure is conditionally mandatory for UEs supporting nr-CGI-Reporting-NPN. RAN2 to discuss whether additional capability bit is needed or not.

The draft TP for Proposal 2 is provided in Appendix.

# 3.	Conclusion

In section 2, we made the following observations:

Observation 1: RAN3 agreed that in case of NPN-only cell, the PLMN identities and associated information contained in the PLMN-IdentityInfoList IE are not included in the Xn and F1 interfaces to set the Broadcast PLMN Identity Info List appropriately.

Observation 2: UE not supporting nr-CGI-Reporting-NPN is not required to detect whether the concerned cell is NPN-only cell or not as part of CGI reporting procedure.

Based on the above, RAN2 is requested to discuss and if possible agree on the following proposal:

Proposal 1: Confirm that UE not supporting nr-CGI-Reporting-NPN shall report the obtained PLMN-IdentityInfoList IE from the indicated NR cell as part of CGI reporting procedure irrespective of the value of cellReservedForOtherUse IE.

Proposal 2: UE supporting nr-CGI-Reporting-NPN reports the cellReservedForOtherUse IE as part of CGI reporting procedure if the concerned cell is NPN-only cell.

Proposal 3: The feature of reporting the cellReservedForOtherUse IE as part of CGI reporting procedure is conditionally mandatory for UEs supporting nr-CGI-Reporting-NPN. RAN2 to discuss whether additional capability bit is needed or not.

# 4.	References

[1] R3-211129	Correction of NPN related Cell Information,	Huawei, China Telecom

[2] R3-211130	Correction on NPN related Cell Information,	Huawei, China Telecom

[3] R2-2103169	Clarification on NPN related CGI report	Huawei, CMCC, China Telecom, China Unicom, HiSilicon

# 5.	Appendix

5.5.5	Measurement reporting

5.5.5.1	General

Figure 5.5.5.1-1: Measurement reporting

[Unrelated parts omitted]

3>	if the cell indicated by cellForWhichToReportCGI is an NR cell:

4>	if plmn-IdentityInfoList of the cgi-Info for the concerned cell has been obtained:

5>	include the plmn-IdentityInfoList including plmn-IdentityList, trackingAreaCode (if available), ranac (if available), cellIdentity and cellReservedForOperatorUse for each entry of the plmn-IdentityInfoList;

5>	include frequencyBandList if available;

4>	if nr-CGI-Reporting-NPN is supported by the UE and npn-IdentityInfoList of the cgi-Info for the concerned cell has been obtained:

5>	include the npn-IdentityInfoList including npn-IdentityList, trackingAreaCode, ranac (if available), cellIdentity and cellReservedForOperatorUse for each entry of the npn-IdentityInfoList;

5>	include the cellReservedForOtherUse if that is set to true;

4>	else if MIB indicates the SIB1 is not broadcast:

5>	include the noSIB1 including the ssb-SubcarrierOffset and pdcch-ConfigSIB1 obtained from MIB of the concerned cell;

3>	if the cell indicated by cellForWhichToReportCGI is an E-UTRA cell:

4>	if all mandatory fields of the cgi-Info-EPC for the concerned cell have been obtained:

5>	include in the cgi-Info-EPC the fields broadcasted in E-UTRA SystemInformationBlockType1 associated to EPC;

4>	if the UE is E-UTRA/5GC capable and all mandatory fields of the cgi-Info-5GC for the concerned cell have been obtained:

5>	include in the cgi-Info-5GC the fields broadcasted in E-UTRA SystemInformationBlockType1 associated to 5GC;

4>	if the mandatory present fields of the cgi-Info for the cell indicated by the cellForWhichToReportCGI in the associated measObject have been obtained:

5>	include the freqBandIndicator;

5>	if the cell broadcasts the multiBandInfoList, include the multiBandInfoList;

5>	if the cell broadcasts the freqBandIndicatorPriority, include the freqBandIndicatorPriority;