3GPP TSG-RAN WG2 Meeting #114 electronic	                                    R2-2105631

Online, May 19 – May 27, 2021

Agenda item:		8.8.2

Source:			Asia Pacific Telecom, FGI

Title:		Cell (re)selection for RAN slicing

Document for:	 	Discussion and decision

- Introduction

In RAN2#113bis e-meeting [1], the cell reselection related topics in RAN slicing WI [2] are prioritized:

- 1: With regard the main solution for prioritisation for slice based cell reselection, the following topics to be the initial focus for discussion: Details of slice availability in terms of Slice grouping and frequency priority information for broadcast and RRC Release message, usage of “intended slice” (FFS whether we use this term in specification), UE prioritisation of slice when there is more than one intended slice and how UE determines frequency priority for inter-frequency cell reselection based on these. - 2: Following topics are only considered after some progress on the main solution for prioritisation for slice based cell reselection: which SIB(s) to carry slice availability, whether an LS to SA3 is needed (if SST/SD is agreed for slice info), whether SIB segmentation/on-demand is required (if new SIB is defined). - 3: Other topics that have some support and could be discussed further depending on companies providing more details on the motivation and level of support: slice based reselection for MO, different RSRP/RSRQ thresholds for inter and intra-frequency slice based cell reselection, need for Validity area in RRC Release

In this contribution, we would share our views on the green highlighted issues.

- Discussion

Slice related information for slice-based cell reselection

During the RAN slicing SI phase, it is concluded that providing the slice related information for cell reselection in the system information or RRCRelease message can address four issues as below.

- Issue 1: The UE is unaware of the slices supported on different cells or frequencies, which prevents UE from (re)select to the cell or frequency supporting the intended slice.

- Issue 2: Dedicated priorities would not be available to the UE prior to first RRC connection establishment and only remain valid before T320 expires upon entering IDLE mode. In addition, dedicated priorities are discarded each time when UE entering CONNECTED mode and need to be configured again before UE leaving CONNECTED mode.

- Issue 3: Operator may require different frequency priority configurations for the specific slice in different areas, however the dedicated priority always overwrites the broadcast priorities if configured.

- Issue 4: If the serving cell is unable to support the requested slices, the serving cell may need to perform handover to a cell supporting the requested slices or release the RRC connection. That may increase control plane signalling overhead as well as long control plane latency for the UE to access the network.

Furthermore, broadcasting slice related cell reselection information in SIB is beneficial. Similar slice related cell reselection information as that in SIB can be added in RRCRelease message as well. However, the detailed slice related cell reselection information in SIB and in RRCRelease message are not yet decided. During RAN slicing SI phase, the conclusion shows that the possible slice related cell reselection information in system information can include the supported slice information of the current cell and neighboring cells and cell reselection priority per slice.

The supported slice information of the current cell and neighboring cells can make the UE aware of the slices supported on different cell. In this way, the UE can be prevented from reselecting a cell not supporting the UE’s intended slice, which would be power-efficient and time-efficient. Thus, we propose that the supported slice information of the current cell and neighboring cells can be provided by the current cell. The current cell can be a serving cell of the UE when the UE is in RRC_CONNECTED state. Otherwise, the current cell can be a camped cell of the UE when the UE is in RRC_IDLE and RRC_INACTIVE state. When the UE is in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED state, the UE can receive the supported slice information of the current cell and neighboring cells via the system information. When the current cell intends to transition the UE in RRC_CONNECTED state to RRC_IDLE or RRC_INACTIVE state, the UE can receive the supported slice information of the current cell and neighboring cells via the RRCRelease message.

Proposal 1: The supported slice information of the current cell and neighboring cells can be provided by the current cell by the system information and RRCRelease message.

Due to the payload size concerns on SIB1 and the ease of maintenance, the cell reselection related system information block (e.g., SIB2, SIB3, SIB4, SIB5) and a new slice-specific system information block can be applied for the current cell to broadcast the supported slice information of the current cell and neighboring cells. According to the classification of SIB2, SIB3 and SIB4 [4], SIB2 can include the supported slice information of the current cell, SIB3 can include the supported slice information of the intra-frequency neighboring cells, and SIB4 can include the supported slice information of the inter-frequency neighboring cells. Alternatively, the current cell may broadcast a new slice-specific system information block including the supported slice information of the current cell and neighboring cells. Thus, we propose that RAN2 can further down-select which SIB to include the supported slice information of current cell and neighboring cells.

Proposal 2: RAN2 can down-select which SIB (e.g., SIB2/3/4, a new slice-specific SIB) to include the supported slice information of current cell and neighboring cell.

- SIB2 can include the supported slice information of the current cell, SIB3 can include the supported slice information of the intra-frequency neighboring cells, and SIB4 can include the supported slice information of the inter-frequency neighboring cells.

- A new slice-specific system information block can include the supported slice information of the current cell and neighboring cells.

Cell reselection rule

According to the conclusion in the RAN slicing SI [3], it is RAN2’s common understanding that the UE’s intended slices are derived based on the information the Access Stratum (AS) layer of the UE receives from the Non-Access Stratum (NAS) layer of the UE. For the initial registration and requesting new S-NSSAI(s), the intended slices are Requested S-NSSAI(s) (Single Network Slice Selection Assistance Information). For the idle-mode mobility, the intended slices are Allowed S-NSSAI(s). After receiving the supported slice information of the current cell and neighboring cells, the UE can further determine whether its Requested S-NSSAI(s) and Allowed S-NSSAI(s) are supported by the current cell and neighboring cells.

The cell reselection criteria do not consider whether the current cell and neighboring cells support the UE’s Requested S-NSSAI(s) and Allowed S-NSSAI(s). Currently, the UE determines the ranks of the current cell and neighboring cells mainly based on the measured signal strength, i.e., the RSRP (Reference Signal Received Power) measurement quantity. The UE would select the highest-ranked cell if specific rules are satisfied. Thus, during the slice-based cell reselection, whether the highest-ranked cell supports the UE’s intended slices should be considered.

Proposal 3: During the slice-based cell reselection, the UE should consider both the measured signal strength and the UE’s intended slices to determine the ranking of a cell.

Conclusion

Based on the above, we have the following proposals.

Proposal 1: The supported slice information of the current cell and neighboring cells can be provided by the current cell by the system information and RRCRelease message.

Proposal 2: RAN2 can down-select which SIB (e.g., SIB2/3/4, a new slice-specific SIB) to include the supported slice information of current cell and neighboring cell.

- SIB2 can include the supported slice information of the current cell, SIB3 can include the supported slice information of the intra-frequency neighboring cells, and SIB4 can include the supported slice information of the inter-frequency neighboring cells.

- A new slice-specific system information block can include the supported slice information of the current cell and neighboring cells.

Proposal 3: During the slice-based cell reselection, the UE should consider both the measured signal strength and the UE’s intended slices to determine the ranking of a cell.

Reference

[1] RAN2#113bis e-Meeting Chairman’s note.

[2] RP-210872 New WID on enhancement of RAN Slicing for NR.

[3] TR 38.832 v1.0.0. 2021-03. Study on enhancement of Radio Access Network (RAN) slicing (Release 17).

[4] TS 38.331 v16.3.1 2021-01. Radio Resource Control (RRC) protocol specification (Release 16).