3GPP TSG-RAN WG2 Meeting #114 electronic	                                    R2-2105632

Online, May 19 – May 27, 2021

Agenda item:		8.16.2

Source:			Asia Pacific Telecom, FGI

Title:		Cell (re)selection for Rel-17 NPN enhancements

Document for:	 	Discussion and decision

- Introduction

In RAN2#113bis e-Meeting, RAN2 WG reaches the agreements on “Support SNPN with subscription or credentials by a separate entity” in Rel-17 eNPN WI [1] as below [2]:

- Use the term "Credentials Holder (CH)" in future RAN2 discussions for the external entity providing subscription or credential for SNPNs. - Use the term "Group IDs for Network Selection (GINs)" in future RAN2 discussions for the service provider Group IDs. - The following assumptions in last meeting are confirmed as agreements, - The new indicator that "access using credentials from a separate entity is supported" is broadcasted in SIB1. - The new indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" is broadcasted in SIB1. - GIDs are broadcasted per SNPN in network sharing scenarios. - RAN2 to revise the previous agreement as following: - In the UE, AS reports broadcast Group IDs per SNPN to NAS. - To supporting SNPN with subscription or credentials by a separate entity, R2 assumes that there is no impact on cell (re)selection (e.g. no need to change suitable cell criteria).

In this contribution, to support the SNPN with subscription or credentials by a separate entity, we would share our views on the cross-layer indication, suitable cell criteria and cell barring rule.

- Discussion

Cross-Layer indication in cell (re)selection

To support SNPN along with subscription / credentials owned by an entity separate from the SNPN, the UE would receive a new indicator “access using credentials from a separate entity is supported” per SNPN and another new indicator “whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN” per SNPN from a cell supporting SNPN. The two new indicators are broadcast in SIB1. The AS of the UE would report the two indicators to the NAS of the UE.

Based on the agreement in RAN2#113 and RAN2#113bis, the AS of the UE would report (1) the indicator indicating whether access using credentials from a separate entity is supported, (2) the indicator indicating whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN, and broadcast Group IDs per SNPN, to the NAS of the UE.

It is the NAS of the UE that would determine whether the UE is explicitly configured to select the SNPN. Thus, there are two outcomes from the perspectives of the AS of the UE: (1) the UE would consider the cell as a candidate cell for cell (re)selection, and (2) the UE would not consider the cell as a candidate cell for cell (re)selection. Thus, the AS of the UE should receive an indicator from the NAS of the UE for the two outcomes.

We propose that the AS of the UE can receive an indicator from the NAS of the UE indicating whether the cell can be a candidate cell for cell (re)selection (e.g., based on the information reported by AS of the UE to NAS of the UE). In addition to the indicator, the NAS of the UE can further provide an SNPN identity associated to the indicator, so that the AS of the UE would select a cell operated by the SNPN identified by the received SNPN identity. This approach is beneficial because the signal exchange between the AS of the UE and the NAS of the UE can be kept in minimum. The AS of the UE only acquires the information whether a cell can be a candidate cell for cell (re)selection, or whether a cell operated by a certain SNPN identity can be a candidate cell for cell (re)selection.

Proposal 1: After the AS of the UE reports to the NAS of the UE about the broadcast Group IDs per SNPN and two indicators: (1) access using credentials from a separate entity is supported, and (2) whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN, the AS of the UE receives an indicator, from the NAS of the UE, indicating whether the cell broadcasting the two indicators and Group IDs can be a candidate cell for cell (re)selection.

Suitable cell criteria

Though the agreement in RAN2#113bis e-Meeting [2] stated “To supporting SNPN with subscription or credentials by a separate entity, R2 assumes that there is no impact on cell (re)selection (e.g. no need to change suitable cell criteria).”, we have different views from the agreement. With the introduction of accessing a cell operated by an SNPN while the UE has subscription or credentials by a credential holder (CH), the UE should still be able to camp on a cell operated by an SNPN different from the credential holder. That is, the UE needs to regard the cell as a suitable cell and camp on the cell.

During cell (re)selection, the UE determine whether a cell is suitable to camp on based on the suitable cell criteria. However, the legacy suitable cell criteria do not consider the cases that the UE may access a cell using a credential from a separate entity and the UE may be allowed to register an SNPN that the UE is not explicitly configured to select the SNPN. Thus, we propose the suitable cell criteria need further study to enable the feature in Rel-17 eNPN.

To support Rel-17 eNPN features, when the UE operates in SNPN Access Mode, the UE may determine whether the cell is suitable based on the two indicators: (1) access using credentials from a separate entity is supported and (2) whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN. For example, the UE operating in SNPN Access Mode can consider a cell as a suitable cell when the cell is neither part of the selected SNPN nor the registered SNPN of the UE, but the cell supports access using credentials from a separate entity. For another example, the UE operating in SNPN Access Mode can consider a cell as a suitable cell when the cell is neither part of the selected SNPN nor the registered SNPN of the UE, but the SNPN operating the cell allows registration attempts from UEs that are not explicitly configured to select the network. More criteria catering to Rel-17 eNPN features can be further investigated.

Proposal 2: The UE may determine whether a cell can be a suitable cell based on the conditions including:

- For a UE operating in SNPN Access Mode, the conditions can include: -

  - Either the cell is part of either the selected SNPN or the registered SNPN of the UE or SNPN of the Equivalent SNPN list; or

  - The cell is neither part of the selected SNPN nor the registered SNPN of the UE, but the cell supports access using credentials from a separate entity; or

  - The cell is neither part of the selected SNPN nor the registered SNPN of the UE, but the cell indicates that the SNPN operating the cell allows registration attempts from UEs that are not explicitly configured to select the network.

Cell barring rules

During cell (re)selection, the UE can bar a cell if the cell cannot meet specific criteria. Thus, when the two indicators: (1) access using credentials from a separate entity is supported and (2) whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN, are considered as factors that the cell is not suitable for the UE to select, the UE can further bar the cell for a certain time.

Possible scenarios can include:

- The cell does not support access using credentials from a separated entity, and the UE has a credential from a separate entity;

- The SNPN does not allow registration attempts from UEs that are not explicitly configured to select the SNPN, and the UE is not explicitly configured to select the SNPN.

Thus, when the two scenarios occur, the UE may bar the cell broadcasting the indicators not supporting the UE’s requirement for a certain time (e.g., a maximum of 300 seconds).

Proposal 3: During cell (re)selection, the UE can bar the cell for a certain time when the two conditions occur: (1) the cell does not support access using credentials from a separated entity and the UE has a credential from a separate entity, and (2) the SNPN does not allow registration attempts from UEs that are not explicitly configured to select the SNPN, and the UE is not explicitly configured to select the SNPN.

Conclusion

Based on the above, we have the following proposals.

Proposal 1: After the AS of the UE reports to the NAS of the UE about the broadcast Group IDs per SNPN and two indicators: (1) access using credentials from a separate entity is supported, and (2) whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN, the AS of the UE receives an indicator, from the NAS of the UE, indicating whether the cell broadcasting the two indicators and Group IDs can be a candidate cell for cell (re)selection.

Proposal 2: The UE may determine whether a cell can be a suitable cell based on the conditions including:

- For a UE operating in SNPN Access Mode, the conditions can include: -

  - Either the cell is part of either the selected SNPN or the registered SNPN of the UE or SNPN of the Equivalent SNPN list; or

  - The cell is neither part of the selected SNPN nor the registered SNPN of the UE, but the cell supports access using credentials from a separate entity; or

  - The cell is neither part of the selected SNPN nor the registered SNPN of the UE, but the cell indicates that the SNPN operating the cell allows registration attempts from UEs that are not explicitly configured to select the network.

Proposal 3: During cell (re)selection, the UE can bar the cell for a certain time when the two conditions occur: (1) the cell does not support access using credentials from a separated entity and the UE has a credential from a separate entity, and (2) the SNPN does not allow registration attempts from UEs that are not explicitly configured to select the SNPN, and the UE is not explicitly configured to select the SNPN.

Reference

[1] RP-202363 Revised WID: Enhancement of Private Network Support for NG-RAN.

[2] RAN2#113bis e-Meeting Chairman Notes.