3GPP TSG-RAN2 Meeting #114-e 	R2-2105644

# E-Meeting, 19 – 27 May 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1813 | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indication of unsupported capabilities for IMS video |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, LTE_5GCN_connect-Core |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Capture proposal 1 in R2-2105629. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add notes to clarify: - If the UE does not include a capability bit for IMS voice, the UE does not support the corresponding function for IMS video. - If the UE includes a capability bit for IMS voice and supports IMS video, whether the UE supports the corresponding function for IMS video is unspecified. Impact analysis Impacted 5G architecture options: E-UTRA/5GC, (NG)EN-DC Impacted functionality: IMS video over NR PDCP with MCG bearer IMS video over NR PDCP with SCG bearer IMS video over FR1/FR2 IMS video voer EUTRA/5GC Inter-operability: There is no inter-operability issue foreseen as described in R2-2105629. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE cannot indicate that the UE does not support the AS capabilities for IMS video. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1A, 4.3.34.13, 4.3.34.14, 4.3.36.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.3.1A	NR PDCP Parameters

NR PDCP capabilities: the definition of rohc-Profiles-r15, rohc-ContextMaxSessions-r15, rohc-ProfilesUL-Only-r15, rohc-ContextContinue-r15, outOfOrderDelivery-r15 and sn-SizeLo-r15 are the same as supportedROHC-Profiles, maxNumberROHC-ContextSessions, uplinkOnlyROHC-Profiles, continueROHC-Context, outOfOrderDelivery and shortSN defined in TS 38.306 [32].

ims-VoiceOverNR-PDCP-MCG-Bearer-r15 indicates whether the UE supports IMS voice over NR PDCP with only MCG RLC bearer.

ims-VoiceOverNR-PDCP-SCG-Bearer-r15 indicates whether the UE supports IMS voice over NR PDCP with only SCG RLC bearer when configured with EN-DC. If the UE does not include this field, the UE does not support IMS video over NR PDCP with only SCG RLC bearer when configured with EN-DC.

ims-VoNR-PDCP-SCG-NGENDC-r15 indicates whether the UE supports IMS voice over NR PDCP with only SCG RLC bearer when configured with NGEN-DC.

NOTE 1:	In this release of specification, IMS voice over split bearer is not supported for (NG)EN-DC.

NOTE 2:	If the UE does not include ims-VoiceOverNR-PDCP-MCG-Bearer-r15, the UE does not support IMS video over NR PDCP with only MCG RLC bearer. If the UE includes this field and supports IMS video, whether the UE supports IMS video over NR PDCP with only MCG RLC bearer is unspecified.

NOTE 3:	If the UE does not include ims-VoiceOverNR-PDCP-SCG-Bearer-r15, the UE does not support IMS video over NR PDCP with only SCG RLC bearer when configured with EN-DC. If the UE includes this field and supports IMS video, whether the UE supports IMS video over NR PDCP with only SCG RLC bearer is unspecified.

NOTE 4:	If the UE does not include ims-VoNR-PDCP-SCG-NGENDC-r15, the UE does not support IMS video over NR PDCP with only SCG RLC bearer when configured with NGEN-DC.

#### 4.3.34.13	ims-VoiceOverNR-FR1-r15

This field indicates whether the UE supports IMS voice over NR FR1.

NOTE:	If the UE does not include this field, the UE does not support IMS video over NR FR1. If the UE includes this field and supports IMS video, whether the UE supports IMS video over NR FR1 is unspecified.

#### 4.3.34.14	ims-VoiceOverNR-FR2-r15

This field indicates whether the UE supports IMS voice over NR FR2.

NOTE:	If the UE does not include this field, the UE does not support IMS video over NR FR2. If the UE includes this field and supports IMS video, whether the UE supports IMS video over NR FR2 is unspecified.

#### 4.3.36.6	IMS-VoiceOverMCG-BearerEUTRA-5GC-r15

This field indicates whether the UE supports IMS voice over NR PDCP for MCG bearer for E-UTRA/5GC. It is mandated to the IMS voice capable UE if the UE supports eutra-5GC-r15.  If the UE does not include this field, the UE does not support IMS video over NR PDCP for MCG bearer for E-UTRA/5GC.

NOTE:	If the UE does not include this field, the UE does not support IMS video over NR PDCP for MCG bearer for E-UTRA/5GC. If the UE includes this field and supports IMS video, whether the UE supports IMS video over NR PDCP for MCG bearer for E-UTRA/5GC is unspecified.