3GPP TSG-RAN WG2 Meeting #114-e  	      R2-2105645

E-meeting, 19 – 27 May 2021

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2637 | rev | - | Current version: | 16.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving ambiguity in use of BAP routing ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is at present only one use of a BAP routing ID in the Rel-16 NR RRC spec – it appears in the configuration of the default UL BAP routing ID of a node, which is then used during the bootstrapping phase of the node. However the BAP-routingID IE may be perceived as contradictory and misleading: - It makes forward compatibility difficult by specifying a very narrow use for this IE, by specifying that “The IE BAP-RoutingID is used for IAB-node to configure the BAP Routing ID.” It is true that this is the current use – however the very fact that we have a separate IE for the BAP routing ID means it could be used for other purposes in future. - It then paradoxically defines the BAP address and path ID in very general terms. For instance, it talks about the BAP address being something used in the BAP header. While this is true, the current sole use of the BAP address in the RRC spec has nothing to do with a BAP header of a packet – it is a parameter of the node. It also defines the address as that of an IAB-node or an IAB-donor-DU, whereas for the current use only IAB-donor-DU is applicable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the BAP routing ID in suitably general terms and add a reference to the BAP spec, thus facilitating future uses of the IE. Then specify the constraints for the current use. Remove some unnecessary and potentially confusing descriptors such as ‘used in the BAP header’. Impact analysis Impacted functionality: Use of BAP routing ID IE Inter-operability: If only the network is implemented according to the CR, no interoperability problems are foreseen. If only the IAB-MT is implemented according to the CR, no interoperability problems are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Difficult to add new uses of BAP routing ID to the spec. Incorrect information given on the BAP address and path ID contained in the BAP routing ID when used for its sole current purpose. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 Radio resource control information elements |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

#### –	BAP-RoutingID

The IE BAP-RoutingID contains a BAP Routing ID as specified in TS 38.340 [47].

BAP-RoutingID information element

-- ASN1START

-- TAG-BAPROUTINGID-START

BAP-RoutingID-r16::=        SEQUENCE{

bap-Address-r16              BIT STRING (SIZE (10)),

bap-PathId-r16               BIT STRING (SIZE (10))

}

-- TAG-BAPROUTINGID-STOP

-- ASN1STOP

BAP-RoutingID field descriptions

bap-Address In general, the ID of a destination IAB-node or IAB-donor-DU. When BAP-RoutingID is used to configure defaultUL-BAP-RoutingID, the bap-Address is always the ID of a destination IAB-donor-DU.

bap-PathId In general, the ID of a path to a destination IAB-node or IAB-donor-DU. When BAP-RoutingID is used to configure defaultUL-BAP-RoutingID, the bap-PathId is always the ID of a path to the destination IAB-donor-DU.