3GPP TSG-RAN WG2 Meeting #114 electronic   						    	R2-2105731

E-Meeting, 19th May – 27th May, 2021

Source: 	Xiaomi Communications

Title:  	Enhanced TCI State Indication for UE-specific PDCCH MAC CE

Agenda Item:	8.17.1

Document for:	Discussion and Decision

# Introduction

According to the RAN1 LS on the support of SFN-based PDCCH, RAN1 asks RAN2 to define a MAC CE based on the following RAN1 agreements:

Agreement - Support MAC CE activation of two TCI states for PDCCH - FFS other details Agreement - Introduce enhanced MAC CE signaling for PDCCH activating two TCI states for SFN-based PDCCH transmission - The corresponding MAC CE includes at least the following fields - Serving cell ID - CORESET ID - Two TCI state IDs - FFS whether for CA scenario additionally support RRC configured set of the serving cells which can be addressed by a single MAC CE - FFS whether or not enhanced MAC CE signaling is applicable to a CORESET configured with CORESETPoolindex

In this contributions, we provide the text proposal for the design of the “Enhanced TCI State Indication for UE-specific PDCCH MAC CE”.

# Discussion

## Enhanced TCI State Indication for UE-specific PDCCH MAC CE

According to the current MAC specification [2], the “TCI State Indication for UE-specific PDCCH MAC CE” is used to indicate one TCI state for a specific CORESET. The CORESET ID and the TCI state ID is unique per cell. Then we could have the following two ways to indicate two TCI states for a CORESET (i.e. the SFN-based PDCCH):

- Option 1 (2 legacy MAC CEs): Reuse the legacy TCI State Indication for UE-specific PDCCH MAC CE. The Two TCI State Indication for UE-specific PDCCH MAC CE(s) included in the same MAC PDU indicates that two TCI states are used for a CORESET.

- Option 2 (One legacy MAC CE + one new MAC CE): Define a new MAC CE (i.e. using a new LCID) indicating one extra TCI state for a CORESET. Then another TCI state for the same CORESET is indicated by the legacy TCI State Indication for UE-specific PDCCH MAC CE.

- Option 3 (One new MAC CE)

  - Option 3.1: Define a new MAC CE indicating two TCI states for a CORESET

  - Option 3.2: Define a new MAC CE indicating either one or two TCI states for a CORESET

For Option 1, RAN2 does not need to define an extra MAC CE to indicate two TCI states for a CORESET, but requires the gNB to send the two MAC CEs in the same MAC PDU. Option 2 can use the legacy MAC CE and a new MAC CE to indicate the two TCI states of a CORESET. Compared with Option 3, both Option 1 and Option 2 would require more signalling overheads. For Option 3.2, the legacy MAC CE does not need to be sent when the new MAC CE is used, as the new MAC CE can be used for either one or two TCI states for a CORESET. However as the UE does not know which MAC CE is sent by the gNB, then the UE would still need to support both the legacy and the new MAC CE(s). Then the extra flexibility/complexity of allowing the indication of either one or two TCI states for a CORESET in one MAC CE seems not necessary. Thus, Option 3.1 is preferred.

Proposal 1: A new MAC CE is defined to indicate two TCI states for a CORESET.

When the legacy “TCI State Indication for UE-specific PDCCH MAC CE” is indicated after the reception of the “Enhanced TCI State Indication for UE-specific PDCCH MAC CE”, the UE should fall back to the legacy operation, namely only one TCI state used for the PDCCH. Then we do not have to define extra UE behaviours regarding the cooperation (e.g. using the legacy MAC CE to update one of the two TCI states indicated in the new MAC CE) between the two MAC CEs.

Proposal 2: When the legacy “TCI State Indication for UE-specific PDCCH MAC CE” is indicated after the reception of the “Enhanced TCI State Indication for UE-specific PDCCH MAC CE”, the UE falls back to one TCI state for the PDCCH

According to the current MAC specification [2], there are three types of LCIDs which can be used for the MAC CE, namely 6-bits LCID, one-octet eLCID and two octet eLCID. As only 12 code points are reserved for the 6-bits LCID and the new MAC CE is not expected to be sent frequently, we consider that the one-octet eLCID can be used for the new MAC CE.

Proposal 3: One value of one-octet eLCID is used for the new MAC CE.

Regarding the maximum number of TCI states which can be configured for the CORESET, the RRC specification supports two different configurations for the maximum numbers of the TCI states of a CORESET:

- Up-to 64 TCI states can be configured for a CORESET (i.e. tci-StatesPDCCH-ToAddList) other than CORESET zero.

- The first 64 TCI states (i.e. tci-StatesToAddModList in the PDSCH-Config) of PDSCH can be applied for CORESET zero.

As such RAN1 should clarify which CORESET can be indicated with two TCI states and provide the maximum number of the TCI states which can be configured for the CORESET.

Proposal 4: RAN2 is kindly requested to ask the following clarification questions to RAN1:

- Q1: Which CORESET can be indicated with two TCI states?

- Q2: How many TCI states (i.e. maximum number) can be configured for the CORESET indicated with two TCI states?

According to the analysis given above, a text proposal giving the design of the new MAC CE is provided in the Annex.

Proposal 5: RAN2 is kindly requested to consider the text proposal given in the Annex as the baseline for the Enhanced TCI State Indication for UE-specific PDCCH MAC CE

# Conclusions

According to the analysis given above, we have the following observations and proposals:

Proposal 1: A new MAC CE is defined to indicate two TCI states for a CORESET.

Proposal 2: When the legacy “TCI State Indication for UE-specific PDCCH MAC CE” is indicated after the reception of the “Enhanced TCI State Indication for UE-specific PDCCH MAC CE”, the UE falls back to one TCI state for the PDCCH

Proposal 3: One value of one-octet eLCID is used for the new MAC CE.

Proposal 4: RAN2 is kindly requested to ask the following clarification questions to RAN1:

- Q1: Which CORESET can be indicated with two TCI states?

- Q2: How many TCI states (i.e. maximum number) can be configured for the CORESET indicated with two TCI states?

Proposal 5: RAN2 is kindly requested to consider the text proposal given in the Annex as the baseline for the Enhanced TCI State Indication for UE-specific PDCCH MAC CE

# Reference

[1] R1-2104064, LSin, from RAN1, “LS on TCI states indication for PDCCH”.

[2] 3GPP TS 38.321, “Medium Access Control (MAC) protocol specification”.

# Annex A: Enhanced TCI State Indication for UE-specific PDCCH MAC CE

----------------------------------------------------Start of Change--------------------------------------------------------

### 5.18.1	General

This clause specifies the requirements upon reception of the following MAC CEs:

-	SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE;

-	Aperiodic CSI Trigger State Subselection MAC CE;

-	TCI States Activation/Deactivation for UE-specific PDSCH MAC CE;

-	TCI State Indication for UE-specific PDCCH MAC CE;

-	SP CSI reporting on PUCCH Activation/Deactivation MAC CE;

-	SP SRS Activation/Deactivation MAC CE;

-	PUCCH spatial relation Activation/Deactivation MAC CE;

-	Enhanced PUCCH spatial relation Activation/Deactivation MAC CE;

-	SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE;

-	Recommended Bit Rate MAC CE;

-	Enhanced SP/AP SRS Spatial Relation Indication MAC CE;

-	SRS Pathloss Reference RS Update MAC CE;

-	PUSCH Pathloss Reference RS Update MAC CE;

-	Serving Cell set based SRS Spatial Relation Indication MAC CE;

-	SP Positioning SRS Activation/Deactivation MAC CE;

-	Timing Delta MAC CE;

-	Guard Symbols MAC CEs;

-	Enhanced TCI State Indication for UE-specific PDCCH MAC CE.

----------------------------------------------------Next change--------------------------------------------------------

### 5.18.5	Indication of TCI state for UE-specific PDCCH

The network may indicate a TCI state for PDCCH reception for a CORESET of a Serving Cell or a set of Serving Cells configured in simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 by sending the TCI State Indication for UE-specific PDCCH MAC CE described in clause 6.1.3.15. The network may also indicate two TCI states for PDCCH reception for a CORESET of a Serving Cell by sending the Enhanced TCI State Indication for UE-specific PDCCH MAC CE described in clause 6.1.3.x.

The MAC entity shall:

1>	if the MAC entity receives a TCI State Indication for UE-specific PDCCH MAC CE on a Serving Cell:

2>	indicate to lower layers the information regarding the TCI State Indication for UE-specific PDCCH MAC CE.

1>	if the MAC entity receives an Enhanced TCI State Indication for UE-specific PDCCH MAC CE on a Serving Cell:

2>	indicate to lower layers the information regarding the Enhanced TCI State Indication for UE-specific PDCCH MAC CE.

----------------------------------------------------Next change--------------------------------------------------------

#### 6.1.3.x	Enhanced TCI State Indication for UE-specific PDCCH MAC CE

The Enhanced TCI State Indication for UE-specific PDCCH MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1. It has a fixed size of 16 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

Editor’s NOTE: FFS whether the MAC CE can be applied to a set of serving cells.

-	CORESET ID: This field indicates a Control Resource Set identified with ControlResourceSetId as specified in TS 38.331 [5], for which the TCI State is being indicated. In case the value of the field is 0, the field refers to the Control Resource Set configured by controlResourceSetZero as specified in TS 38.331 [5]. The length of the field is 4 bits;

Editor’s NOTE: FFS whether the MAC CE can be applied to CORESET zero.

-	TCI State ID: This field indicates the TCI state identified by TCI-StateId as specified in TS 38.331 [5] applicable to the Control Resource Set identified by CORESET ID field. If the field of CORESET ID is set to 0, this field indicates a TCI-StateId for a TCI state of the first 64 TCI-states configured by tci-StatesToAddModList and tci-StatesToReleaseList in the PDSCH-Config in the active BWP. If the field of CORESET ID is set to the other value than 0, this field indicates a TCI-StateId configured by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList in the controlResourceSet identified by the indicated CORESET ID. The length of the field is 7 bits.

Editor’s NOTE: FFS how many TCI state can be configured for the CORESET.

Figure 6.1.3.x-1: Enhanced TCI State Indication for UE-specific PDCCH MAC CE

----------------------------------------------------Next change--------------------------------------------------------

### 6.2.1	MAC subheader for DL-SCH and UL-SCH

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1 and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The LCID field size is 6 bits. If the LCID field is set to 34, one additional octet is present in the MAC subheader containing the eLCID field and follow the octet containing LCID field. If the LCID field is set to 33, two additional octets are present in the MAC subheader containing the eLCID field and these two additional octets follow the octet containing LCID field;

-	eLCID: The extended Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE as described in tables 6.2.1-1a, 6.2.1-1b, 6.2.1-2a and 6.2.1-2b for the DL-SCH and UL-SCH respectively. The size of the eLCID field is either 8 bits or 16 bits.

NOTE:	The extended Logical Channel ID space using two-octet eLCID and the relevant MAC subheader format is used, only when configured, on the NR backhaul links between IAB nodes or between IAB node and IAB Donor.

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	R: Reserved bit, set to 0.

The MAC subheader is octet aligned.

Table 6.2.1-1 Values of LCID for DL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH |
| 1–32 | Identity of the logical channel |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one-octet eLCID field) |
| 35–46 | Reserved |
| 47 | Recommended bit rate |
| 48 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 49 | PUCCH spatial relation Activation/Deactivation |
| 50 | SP SRS Activation/Deactivation |
| 51 | SP CSI reporting on PUCCH Activation/Deactivation |
| 52 | TCI State Indication for UE-specific PDCCH |
| 53 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 54 | Aperiodic CSI Trigger State Subselection |
| 55 | SP CSI-RS/CSI-IM Resource Set Activation/Deactivation |
| 56 | Duplication Activation/Deactivation |
| 57 | SCell Activation/Deactivation (four octets) |
| 58 | SCell Activation/Deactivation (one octet) |
| 59 | Long DRX Command |
| 60 | DRX Command |
| 61 | Timing Advance Command |
| 62 | UE Contention Resolution Identity |
| 63 | Padding |

Table 6.2.1-1a Values of two-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to (216 – 1) | 320 to (216 + 319) | Identity of the logical channel |

Table 6.2.1-1b Values of one-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 243 | 64 to 307 | Reserved |
| 244 | 308 | Enhanced TCI State Indication for UE-specific PDCCH |
| 245 | 309 | Serving Cell Set based SRS Spatial Relation Indication |
| 246 | 310 | PUSCH Pathloss Reference RS Update |
| 247 | 311 | SRS Pathloss Reference RS Update |
| 248 | 312 | Enhanced SP/AP SRS Spatial Relation Indication |
| 249 | 313 | Enhanced PUCCH Spatial Relation Activation/Deactivation |
| 250 | 314 | Enhanced TCI States Activation/Deactivation for UE-specific PDSCH |
| 251 | 315 | Duplication RLC Activation/Deactivation |
| 252 | 316 | Absolute Timing Advance Command |
| 253 | 317 | SP Positioning SRS Activation/Deactivation |
| 254 | 318 | Provided Guard Symbols |
| 255 | 319 | Timing Delta |

Table 6.2.1-2 Values of LCID for UL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) |
| 1–32 | Identity of the logical channel |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one-octet eLCID field) |
| 35–44 | Reserved |
| 45 | Truncated Sidelink BSR |
| 46 | Sidelink BSR |
| 47 | Reserved |
| 48 | LBT failure (four octets) |
| 49 | LBT failure (one octet) |
| 50 | BFR (one octet Ci) |
| 51 | Truncated BFR (one octet Ci) |
| 52 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) |
| 53 | Recommended bit rate query |
| 54 | Multiple Entry PHR (four octets Ci) |
| 55 | Configured Grant Confirmation |
| 56 | Multiple Entry PHR (one octet Ci) |
| 57 | Single Entry PHR |
| 58 | C-RNTI |
| 59 | Short Truncated BSR |
| 60 | Long Truncated BSR |
| 61 | Short BSR |
| 62 | Long BSR |
| 63 | Padding |

Table 6.2.1-2a Values of two-octet eLCID for UL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to (216 – 1) | 320 to (216 + 319) | Identity of the logical channel |

Table 6.2.1-2b Values of one-octet eLCID for UL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 249 | 64 to 313 | Reserved |
| 250 | 314 | BFR (four octets Ci) |
| 251 | 315 | Truncated BFR (four octets Ci) |
| 252 | 316 | Multiple Entry Configured Grant Confirmation |
| 253 | 317 | Sidelink Configured Grant Confirmation |
| 254 | 318 | Desired Guard Symbols |
| 255 | 319 | Pre-emptive BSR |

----------------------------------------------------End of Change--------------------------------------------------------