3GPP TSG-RAN WG2 #114e	Tdoc R2-2105761

Electronic meeting, 2021-05-19 - 2021-05-27

Agenda Item:	5.3.2

Source:	Ericsson

Title:	Change of PDU Session ID

Document for:	Discussion, Decision

# 1	Introduction

The topic of changing PDU session ID was discussed in RAN#113bis. The contribution [1] wanted to clarify that the PDU session ID could be changed only be releasing and adding the corresponding DRBs. A discussion was held concluding to postpone the issue [2]. This contribution argues it is known that the PDU session ID cannot be changed and thus no clarification in RAN specifications is needed.

# 2	Discussion

The original contribution argues it is unclear whether the field pdu-Session of SDAP-Config can be changed and suggests adding a clarification that this value cannot be changed, unless the DRBs associated with the SDAP-config are released first.

From 23.501 it is clear the PDU session ID cannot be changed in a running session:

Table 5.6.1-1: Attributes of a PDU Session

| PDU Session attribute | May be modified later during the lifetime of the PDU Session | Notes |
| --- | --- | --- |
| S-NSSAI of the HPLMN | No | (Note 1) (Note 2) |
| S-NSSAI of the Serving PLMN | Yes | (Note 1) (Note 2) (Note 4) |
| DNN (Data Network Name) | No | (Note 1) (Note 2) |
| PDU Session Type | No | (Note 1) |
| SSC mode | No | (Note 2) The semantics of Service and Session Continuity mode is defined in clause 5.6.9.2 |
| PDU Session Id | No |  |
| User Plane Security Enforcement information | No | (Note 3) |
| Multi-access PDU Connectivity Service | No | Indicates if the PDU Session provides multi-access PDU Connectivity Service or not. |
| NOTE 1: If it is not provided by the UE, the network determines the parameter based on default information received in user subscription. Subscription to different DNN(s) and S-NSSAI(s) may correspond to different default SSC modes and different default PDU Session Types NOTE 2: S-NSSAI(s) and DNN are used by AMF to select the SMF(s) to handle a new session. Refer to clause 6.3.2. NOTE 3: User Plane Security Enforcement information is defined in clause 5.10.3. NOTE 4: The S-NSSAI value of the Serving PLMN associated to a PDU Session can change whenever the UE moves to a different PLMN, while keeping that PDU Session. |  |  |

Why would the NW change the field pdu-session in SDAP-config? As shown, the PDU session ID cannot be modified in the lifetime of the PDU session. In our understanding, the only reason would be to re-assign an existing SDAP configuration (and associated DRBs) from one PDU session to another. In that case it would be more efficient from a signalling perspective if the PDU session ID could be changed instead of releasing the whole configuration and setting up a new one. However, efficient signaling setup of multiple PDU sessions is not something RAN2 has discussed, nor do we find it useful. We do not see a big need of efficient setup of multiple PDU sessions.

As such, we agree with the intention with the CR, we agree that the network cannot change the value of pdu-session. However, we think adding this to specification is a case of unnecessary over-specification. We have not seen such behaviour in the field either.

RAN2 confirms the network cannot change the value of pdu-session in SDAP-config. No need to capture this in specification.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 confirms the network cannot change the value of pdu-session in SDAP-config. No need to capture this in specification.

# 4	References

R2-2104293, "Clarification on the change of PDU session ID", Samsung, RAN2#113bis-e, 12 – 20 April 2021

R2-2104534, "Report of [AT113bis-e][004][NR15] PDCP SDAP", LG Electronics (Rapporteur), RAN2#113bis-e, 12 – 20 April 2021