3GPP TSG-RAN WG2 Meeting #114-e	R2-2105805

Online, 19 – 27 May 2021

Agenda item:	 8.13.2.1

Source:		Lenovo, Motorola Mobility

Title:		SON Enhancements for DAPS Handover

Document for:		Discussion and Decision

1	Introduction

In RAN2#112e [1], SON for DAPS HO was discussed and agreements were achieved as following:

Agreements:

In case of successive failures associated to DAPS, the UE stores and reports both failure related information (FFS the details of the information). The successive failure referred above, includes the following scenarios:

UE declares RLF on the source cell while performing the DAPS towards the target cell and declares HOF towards the target cell.

FFS:	For the case of failed DAPS handover to the target cell but successful fallback to source, no further information is needed in the legacy FailureInformation message.

Agreements:

At least the following cells’ related cell and beam measurements are included in the UE report associated to DAPS failure (try to reuse existing information):

a.	Source cell of the DAPS

b.	Target cell of the DAPS

In RAN2#113e [2], further agreements were achieved:

Following DAPS HO scenarios are considered:

a.	Failed DAPS handover to the target cell but successfully fallback to source

b.	UE declares RLF on the source cell before successfully DAPS handover towards target cell

In RAN2#113 bis-e [3], more were achieved:

Agreements:

1	Include in the RLF report for DAPS HO, the following measurements (reuse the legacy mechanism and IEs):

a.	Measurements of neighbour cells when HOF or RLF occurs

2	RAN2 to agree the intention of the following timers:

a.	Time elapsed since DAPS HO execution until RLF occurs in source cell before fallback

b.	Time elapsed since DAPS HO execution until RLF occurs in source cell after fallback

c.	The elapsed time between the execution of DAPS and RLF in target cell

FFS if for the above timers the existing timers can be reused.

3	Include in the RLF report for DAPS HO the following information:

a.	RLF-cause of the RLF occurred in the source cell while performing a DAPS HO

b.	Explicit indicator for DAPS handover failure

4	At least the following triggering conditions are applied for generating an HO Success Report in the case that the HO succeeds:

a.	The UE logs the HO success report if, while doing HO, T310 value exceeds a threshold

b.	The UE logs the HO success report if, while doing HO, T312 value exceeds a threshold

c.	The UE logs the HO success report if, while doing HO, T304 exceeds a threshold

d.	In case of DAPS, if the UE gets an RLF in the source while doing DAPS.

Agreements:

1	RAN2 to focus on the following scenarios for HO Success Report:

a.	Scenario 1 (ordinary HO): 1a, 1b

b.	Scenario 2 (CHO): 2a, 2b

c.	Scenario 3 (DAPS): 3a

2	RAN2 for further discuss whether the following scenarios should be considered under the RLF report or under the HO success report:

a.	Scenario 2c

b.	Scenario 3b

3	The following radio related measurements are as part of the successful HO report:

a.	Latest radio measurement results of the candidate target cells in the case of conditional HO. FFS best cell(s) should be included in.

b.	Flag to indicate RLF issues in source cell during DAPS HO

4	The following time-related measurements are as part of the successful HO report:

a.	Time elapsed between the CHO execution towards the target cell and the corresponding latest CHO configuration received for the selected target cell

5	Location information is included as part of the successful HO report.

In this paper, we would further discuss the details of MRO for DAPS HO.

2	Discussion

2.1 DAPS HO failure

In DAPS handover, source link connection is maintained after receiving handover command associated with DAPS and until releasing the source cell after successful random access to the target gNB. There may be several failure cases happened during DAPS handover procedure.

Case 1: RLF on source link after T304 stops

TS38.300 has specified that:

The UE declares Radio Link Failure (RLF) when one of the following criteria are met:

-	Expiry of a radio problem timer started after indication of radio problems from the physical layer (if radio problems are recovered before the timer is expired, the UE stops the timer); or

-	Expiry of a timer started upon triggering a measurement report for a measurement identity for which the timer has been configured while another radio problem timer is running; or

-	Random access procedure failure; or

-	RLC failure; or

-	Detection of consistent uplink LBT failures for operation with shared spectrum channel access as described in 5.6.1; or

-	For IAB-MT, the reception of BH RLF indication received from its parent node.

In R16 eMob WI, it has agreed that after successful RACH towards the target cell, the UE would not perform RLM for source cell, but it can still perform HARQ/ARQ retransmission and ROHC feedback transmission to the source gNB, i.e. there is a possibility that the UE can declare source RLF once RLC failure occurs.

Therefore, after successful RACH towards the target cell and before receiving the RRCReconfiguration message including source release indication, the UE may declare RLF at the source gNB due to RLC failure. We should also take this case into consideration for SON.

Observation 1: Before receiving the RRCReconfiguration message including source release indication, the UE may declare source RLF due to RLC failure after successful RACH towards the target cell.

For the case that source RLF occurs after successful DAPS HO and before source link is released, the state of source link e.g. RLF or RLC retransmission times in the source link, should be included in rlf-report.

Proposal 1: The state of source link after successful RACH should be included in the RLF-Report.

Case 2: Source link is available upon T304 expiry

In DAPS HO, the UE falls back to the source link if the source link is still available when T304 expires. The UE is expected to report the FailureInformation message including the DAPS-failure indication to source gNB after falling back to the source link. The rlf-report is not needed since the DAPS failure information has been reported using FailureInformation message. In addition, since source link is maintained during DAPS handover, the source node can identify the problem when it receives the DAPS failure information from UE. Therefore, no further information is needed in the legacy FailureInformation message for DAPS failure.

Observation 2: The DAPS-failure indication included in the FailureInformation message has been reported to the source gNB after falling back to the source gNB.

Observation 3: RLF-Report is not needed for the case that DAPS HO fails but UE falls back to the source link.

Proposal 2: No further information is needed in the legacy FailureInformation message for the case that DAPS HO fails but UE falls back to the source link.

Case 3: Source RLF occurs before HOF or an RLF occurring shortly after successful RACH in the target cell

For any failure case existed in DAPS HO, UE can report failure related information report. Since different handover type has different mobility related parameters, it is necessary to distinguish DAPS handover from normal handover. In the legacy rlf-report, there is no DAPS specific information included, and the existing connectionFailureType only includes ‘hof’ and ‘rlf’. Especially for the case that the UE performs re-establishment upon an RLF occurs in the target cell shortly after the successful RACH even though the source link is always available, it is difficult to be identified if legacy rlf-report is totally re-used without any update. There are two potential options as below:

- Option 1: Introduce new connection failure type, e.g. besides ‘hof’ and ‘rlf’, the connectionFailureType can be extended to include ‘daps-hof’ and ‘daps-rlf’;

- Option 2: Re-use current connection failure type and add corresponding indication.

For example, for the case that UE initiates the reestablishment procedure when DAPS HO fails (T304 expiry) and source link is not available, in option 1, we can include the new defined ‘daps-hof’ in the rlf-report to show the handover failure type; in option 2, we can re-use current ‘hof’ together with a new added indication which indicates source RLF in the rlf-report to show the handover failure type. Compared with these two options, since last RAN2 meeting has agreed that an explicit indicator for DAPS handover failure is included in the RLF-Report, Option 2 is the better way, and  Option 2 is also backward compatibility.

Proposal 3: The existing connection failure type i.e. ‘RLF’ or ‘HOF’ can be reused for SON for DAPS HO since additional indication is added to indicate DAPS failure.

RAN2#112e meeting has agreed that “In case of successive failures associated to DAPS, the UE stores and reports both failure related information”, the potential options are listed as below for the UE to store and report information for the two successive failures:

- Option 1: Re-use the existing rlf-report with extensions to cover all the two successive failures related information.

- Option 2: Since the legacy entry rlf-report in the RLF Report can only cover the information for the latest failure, introduce a new entry in the same one RLF Report for the first failure.

- Option 3: Use Two separate RLF Reports, one containing IEs related to the first failure, the other one containing IEs related to the second failure.

In Option 1, separate IEs within the existing rlf-report can be reused to represent the second failure, and the first failure can be represented by reusing as much as possible existing IEs. Compared with the other two options, Option 1 has less spec impact, and we can make the final decision when the state for DAPS failure related parameters to be reported are stable.

Proposal 4: Re-use the existing rlf-report with extensions to cover all the two successive failures related information.

2.2. Successful DAPS HO

Similar to successful handover report for normal handover, we can also study MRO procedure for successful DAPS handover, and the main cases are as below:

Case a: Source RLF occurs when T304 is running, but the UE finally completes DAPS handover successfully;

Case b: There is no source RLF and the UE finally completes DAPS handover successfully.

After successfully accessing the target cell during DAPS handover, UE stops the timer T304 and transmits the RRCReconfigurationComplete to target cell. There are two states of source link when T304 stops upon successful random access, i.e. source link fails when T304 is running as illustrated in case a, or source link is available when T304 stops. Therefore, UE can report the state of source link for MRO purpose in the case that UE successfully completes DAPS handover.

Proposal 5: The state of source link can be reported in the successful handover report.

In last RAN2 meeting [3], the scenario that UE successfully performed a DAPS HO towards the target cell with no RLF in the source cell was agreed for HO Success Report for DAPS HO procedure, and some triggering conditions for HO Success Report were agreed as below:

a.	The UE logs the HO success report if, while doing HO, T310 value exceeds a threshold

b.	The UE logs the HO success report if, while doing HO, T312 value exceeds a threshold

c.	The UE logs the HO success report if, while doing HO, T304 exceeds a threshold

d.	In case of DAPS, if the UE gets an RLF in the source while doing DAPS.

Besides T310 or T312 for radio link of the source cell, the UE may also monitor target link after the UE successfully handovers to the target cell but finally no RLF occurs in the target cell, e.g. the UE starts T310 or T312 for the target cell, or the UE detects the consecutive "out-of-sync" indications from target cell. Therefore, the conditions e.g. RLF does not happen in target cell but T310 in target cell is started within a period after successful DAPS handover, RLF does not happen in target cell but T312 in target cell is started within a period after successful DAPS handover, or RLF does not happen in target cell but the number of consecutive "out-of-sync" indications from target cell is greater than one threshold should also be considered.

Proposal 6: At least the following triggering conditions are applied for generating an HO Success Report for the case that DAPS HO succeeds:

- RLF does not happen in target cell but T310 in target cell is started within a period after successful handover;

- RLF does not happen in target cell but T312 in target cell is started within a period after successful handover;

- RLF does not happen in target cell but the number of consecutive "out-of-sync" indications from target cell is greater than one threshold.

Based on the triggering conditions, additional information should be reported in the HO successful report for DAPS:

- T310 value in target cell;

- T312 value in target cell;

- The number of consecutive "out-of-sync" indications from target cell;

Proposal 7: At least the following information can be reported in the HO Success Report for the case that DAPS HO succeeds:

- T310 value in target cell;

- T312 value in target cell;

- The number of consecutive "out-of-sync" indications from target cell.

3	Conclusion

In this contribution, the further issues for MRO on DAPS handover are discussed. We have the following observations and proposals:

Observation 1: Before receiving the RRCReconfiguration message including source release indication, the UE may declare source RLF due to RLC failure after successful RACH towards the target cell.

Observation 2: The DAPS-failure indication included in the FailureInformation message has been reported to the source gNB after falling back to the source gNB.

Observation 3: RLF-Report is not needed for the case that DAPS HO fails but UE falls back to the source link.

Proposal 1: The state of source link after successful RACH should be included in the RLF-Report.

Proposal 2: No further information is needed in the legacy FailureInformation message for the case that DAPS HO fails but UE falls back to the source link.

Proposal 3: The existing connection failure type i.e. ‘RLF’ or ‘HOF’ can be reused for SON for DAPS HO since additional indication is added to indicate DAPS failure.

Proposal 4: Re-use the existing rlf-report with extensions to cover all the two successive failures related information.

Proposal 5: The state of source link can be reported in the successful handover report.

Proposal 6: At least the following triggering conditions are applied for generating an HO Success Report for the case that DAPS HO succeeds:

- RLF does not happen in target cell but T310 in target cell is started within a period after successful handover;

- RLF does not happen in target cell but T312 in target cell is started within a period after successful handover;

- RLF does not happen in target cell but the number of consecutive "out-of-sync" indications from target cell is greater than one threshold.

Proposal 7: At least the following information can be reported in the HO Success Report for the case that DAPS HO succeeds:

- T310 value in target cell;

- T312 value in target cell;

- The number of consecutive "out-of-sync" indications from target cell.

Reference

[1] Chairman notes for RAN2#112e meeting

[2] Chairman notes for RAN2#113e meeting

[3] Chairman notes for RAN2#113bis-e meeting