3GPP TSG-RAN WG2 Meeting #114 electronic                                       R2-2105828

Online, May 19 – 27, 2021                                     Revisiosn of R2-2103394

Agenda item:		9.1.2

Source:		Lenovo, Motorola Mobility

Title:		       Neighbor cell measurements triggering before RLF

Document for:		Discussion and Decision

# Introduction

Based on WI RP-201306 as in [1], the following content is proposed in the WI objective:

- Specify signaling for neighbor cell measurements and corresponding measurement triggering before RLF, to reduce the time taken to RRC reestablishment to another cell, without defining specific gaps. [NB-IoT] [RAN2, RAN4].

In RAN2#111e and RAN2#112e meeting, the following agreements were made as in [2]:

RAN2#112e Agreements: - Enhancements to the random access procedure are not considered. - The solution includes reduction of the time between declaration of RLF and the start of the random access procedure (points C and D) - FFS whether the solution includes reduction of the time between out-of-sync detection and declaration of RLF (points B and C) RAN2#111e Agreements: - Study current RLF procedure to understand the time taken to select the cell for access. Start and end points FFS. - Support neighbour cell measurements in RRC_CONNECTED, at least for intra-frequency. - FFS inter-frequency - FFS whether measurements are done on the anchor carrier - FFS how neighbour cell measurement is triggered - FFS how to perform neighbour cell measurements - Working assumption: Neighbour cell measurement results are not reported to the network in RRC_CONNECTED. - FFS whether and when other information can be sent

In this contribution, it mainly focuses on the neighbour cell measurement triggering condition before RLF to reduce the time taken to RRC reestablishment to another cell, and the assistant information reporting to reduce the time taken to RRC reestablishment to another cell.

# Discussion

For legacy NB-IOT UE, considering UE power saving, there is no measurement configuration and measurement report procedure for UE in connected mode, the NB-IOT UE will trigger the RRC connection re-establishment procedure if it moves to another cell as in figure.1. However, in some scenario, the cell selection to another cell based on RLF and re-establishment procedure may be not time efficient, because the cell selection will be triggered at the reference point C where the T310 is expired as in figure.1.

Figure.1 The procedure of cell selection in RLF and Re-establishment procedure

In order to reduce the time taken to RRC reestablishment to another cell procedure, for UE in connected mode, the time to perform the cell selection as shown in figure.1 should be reduced to find a suitable cell.

Considering the neighbor cell measurement to take place before RLF is beneficial to reduce the time taken to RRC reestablishment to another cell as in WID, as in [3], the following options has been discussed to trigger the neighbour cell measurement before the RLF procedure to assist the cell selection procedure to find suitable frequency or cell.

  - Option1, the neighbour cell measurement could be triggered when the serving cell channel quality is lower than a threshold. The definition that the serving cell channel quality is lower than a threshold could be various, it could be as follows:

    - The definition based on s-Measure,

    - The definition based on the formulation including the offset and Hys used in measurement configuration.

  - Option2，the neighbour cell measurement could be triggered after n number of consecutive "out-of-sync" indications for Pcell is detected.

- Option 3: combination of option1 and option2; multiple triggers (e.g., a configured threshold of RSRP/RSRQ, T310) are applied, the neighbour cell measurement would be triggered whichever the configured threshold of RSRP/RSRQ is met or T310 starts.

For option2, we clarify that the number of consecutive "out-of-sync" indications for PCell could be independent from or less than the one for radio link failure detection which is used to trigger the Timer T310. Then the trigger for neighbor cell measurement will be early than the start of T310, then enough number of gap could be given to neighbor cell measurement.

Proposal1: The neighbor cell measurement could be triggered based on the below two options:

- Option1, the neighbour cell measurement could be trigger when the serving cell channel quality is lower than a threshold.

- Option2, the neighbour cell measurement could be triggered after n number of consecutive "out-of-sync" indications for Pcell is detected.

In order to perform the neighbour cell measurement for NB-IOT UE in connected mode, since no measurement configuration could be transmitted to connected NB-IOT UE, the neighbour cell measurement could follow the parameter such as the frequency priority information configured in SIB for cell reselection. Then UE will choose the target cell with high channel quality and frequency priority to send RRC reestablishment request, it is UE implementation which cell is selected as the target cell.

Proposal2: The neighbour cell measurement could follow the parameter such as the frequency priority information configured in SIB for cell reselection, it is UE implementation which cell is selected as the target cell.

In legacy specification, the target cell needs to retrieve the UE context from source cell after receiving the re-establishment request from UE. In order to reduce the latency and ensure service continuity, some assistant information can be sent to eNB, then eNB could send its UE context to several neighbour eNB. The assistant information including the candidate neighbour cell information could be reported to eNB, then eNB will transmit the UE context to the neighbour cells.

Proposal3: The assistant information including the candidate neighbour cell information could be reported to help eNB deliver the UE context to several neighbor eNB.

# Conclusion

In this contribution, the issues on neighbour cell measurement triggering condition and the signalling for neighbour cell measurement are discussed, we have the following proposals:

Proposal1: The neighbor cell measurement could be triggered based on the below two options:

- Option1, the neighbour cell measurement could be trigger when the serving cell channel quality is lower than a threshold.

- Option2, the neighbour cell measurement could be triggered after n number of consecutive "out-of-sync" indications for Pcell is detected.

Proposal2: The neighbour cell measurement could follow the parameter such as the frequency priority information configured in SIB for cell reselection, it is UE implementation which cell is selected as the target cell.

Proposal3: The assistant information including the candidate neighbour cell information could be reported to help eNB deliver the UE context to several neighbor eNB.

# References

RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#88e, June 29 - July 3, 2020.

R2-2100001, “Report of 3GPP TSG RAN2#112-e meeting, Online,”, MediaTek Inc, 2 - 13 November, 2020.

“Report of email discussion [351] NB-IoT RLF measurements”, Huawei, HiSilicon, RAN2#114e, May 19 – 27, 2021.