3GPP TSG-RAN WG2 #114-e	R2-2105970

Electronic meeting, 2021-05-19 – 2021-05-27

Agenda Item:	8.11.5

Source:	Ericsson

Title:	On GNSS Integrity

Document for:	Discussion, Decision

# 1	Introduction

In RAN#91e, the WI for positioning (3GPP, March 16-26, 2021) has been updated with the following objectives [1]:

- Specify the signalling, and procedures to support GNSS positioning integrity determination, including [RAN2, RAN3]: - The assistance information that will be used to support integrity determination - The information that will be used to provide the positioning integrity KPIs and integrity results - Support of integrity for UE-based and UE-assisted A-GNSS positioning. Note: This objective is applicable to NR and E-UTRA.

In this paper, we discuss the GNSS integrity agenda items.

# 2 	LS discussions

## 2.1	RTCM SC 104 and 134

As part of 3gpp Rel-15 Positioning work item, 3gpp inherited the GNSS RTK OSR messages from RTCM standard. The 3gpp Rel-16 SSR based however was taken from the QZSS standard. Currently RTCM Special Committee (SC) 104 is working on defining SSR and SC134 is involved in defining the integrity for GNSS based High accuracy application. It is worth to co-ordinate with both RTCM SC groups 104 and 134 to enable a harmonized solution rather than fragmented or differing solution between RTCM and 3gpp. 3gpp can liase with RTCM SC 104 with the work that was done in Rel-16 for QZSS SSR. We can check with RTCM if they can adopt the same. With RTCM SC 134, we can liase and check the progress on Integrity work and express our interest that we would like to have a harmonized solution.

RAN2 to liase with RTCM SC 104 and RTCM SC 134 as specified in as specified in R2-2105973.

## 2.2	Integrity and QoS

In simple form, Integrity provides the estimation of PL < AL or whether PL > AL. When PL > AL; the QoS need for positioning may not be met as UE would have a positioning error beyond allowed upper limit. The AL and time to alert value would depend upon positioning applications QoS need. If the QoS requirement is stringent the AL and time to alert also needs to be stringent whereas if QoS requirement is relaxed, then these Integrity parameters/thresholds can also be relaxed. Similar to how QoS framework has different classification; the integrity level can be determined based on primarily QoS.

In Fig. 1, we try to explain a simple integrity level classification in an example with four different levels, of high, medium, low and no integrity support for both UE and the network. The network and a UE may support the operation at all or a subset of levels, which may also be a part of their respective capabilities.

Fig. 1 A simple example of integrity level classification

- No integrity: It can mean that the system has no means to assess the integrity level of the positioning estimation. As there is no systematic way, there is no way to justify the reliability and/or timeliness (actuality) of the obtained position estimation from the UE or the network.

- Low integrity: It can mean that the integrity KPIs and thresholds are defined; however, the AL and PL are set with large offset such that the system rarely has any issue with unavailability or misleading operation. The position error can also be quite high while both the network and the UE are not alerted about it.

- Medium integrity: It can mean that the integrity KPIs and thresholds are defined, and the AL and PL are set such that sometimes the system may provide failure errors due to unavailability of proper position estimation, or notifying on the potential of misleading information, etc.

- High integrity: It can mean that the integrity KPIs and thresholds are defined, and the AL and PL are set such tight that unless the positioning error is below some small amount, the system would not accept the performance and there is a need to repeat the measurement or add extra positioning technique to improve the position estimation. So as long as the system reports a position estimation, it is quite highly guaranteed that it is a very reliable value.

Observation 2	By a proper integrity level classification, it is possible to set a more clear definition for the UE and the network to assess the received integrity KPIs and estimations and to set the threshold values for differing use-cases and requirements.

RAN2 to agree on defining integrity level classification for integrity support. The UE and the network may report their supported levels in the signalling with associated QoS.

QoS signalling from AMF to LMF is defined in TS 29.572 which CT4 is responsible group. Thus, RAN2 need to liase with CT4 to define the Integrity KPI signalling.

However, our view is that SA2 should take the responsibility as this may also impact stage 2 description. For example, below the Integrity KPI may also influence (needs to be added as shown in highlight) TS 23.273:

A proper access type shall be determined to assure that the positioning result can fulfil the requested QoS, QoS with associated Integrity requirements and operator policy

We agree that the requirements are as such specified by SA1 group. Hence, the integrity requirements should also come from SA1. SA1 have been discussing the Integrity requirements.

RAN2 to liase with SA1, SA2 and CT4 to provide signalling of Integrity based upon associated QoS as specified in R2-2105973.

Another aspect that needs to be discussed is that although the RAN2 WID is limited to GNSS Integrity. In order to be forward compatible, it would benefit if a generic requirement is specified by SA1 and captured in SA2 stage 2 and CT4 stage 3 specification.

RAN2 to allow a generic Integrity description to be captured by SA1, SA2 and CT4 specification.

# 3	Positioning Integrity Methods and Signalling

## 3.1	General approach to integrity introduction

Positioning with integrity is studied for GNSS positioning in the technical report [3], but integrity aspects for RAT-dependent positioning can be studied as part of the work item or subsequent study and work items. Therefore, it is reasonable to keep such development in mind when discussing procedures and messages, as well a new and extensions of existing information elements. Much of the integrity discussion is generic and not restricted to GNSS, so an ambition to introduce changes with RAT-dependent extensions in mind, and well as to identify common parts seems adequate.

Many integrity aspects for GNSS and other positioning methods are expected to be similar and certain parts can be in common.

Design signalling, procedures and information element additions/extensions with both GNSS and other positioning methods in mind.

## 3.2	On integrity procedures

Integrity is seemingly a new concept within positioning. Therefore, one may opt for new procedures, messages and information elements. However, there is also an option to extend existing procedures, messages and information elements to accommodate positioning.

For example, basic integrity is already supported for GNSS via the GNSS-RealTimeIntegrity and several measurements and assistance data attributes come with uncertainties and quality indicators. Therefore, it seems to be natural to start investigating to what extent the existing procedure and structures can be extended to accommodate integrity

Existing procedures, messages and information elements already includes integrity components and extensions to accommodate integrity seems plausible.

As baseline, use existing procedures, messages and information elements with extensions to accommodate positioning integrity.

## 3.3	On integrity modes

In the study item [3], two integrity modes where identified:

Two modes of integrity result reporting are also identified below for consideration in the WI:

-	Mode 1 of Integrity Result Reporting : PL Reporting

The integrity computing entity calculates the PL, based on the measurement, assistance information and TIR. Then, the calculated PL is directly reported to where the LCS client resides (Network or UE). Hence, the integrity computing entity does not judge whether the positioning system is still available, it simply provides whatever PL value it has obtained. It is left to the LCS client itself to determine if the positioning system is still available based on the reported PL.

-	Mode 2 of Integrity Result Reporting : Integrity Event Flagging

The integrity computing entity calculates the PL, based on the measurement, assistance information and TIR. Then, the integrity computing entity further compares the calculated PL with the given AL to determine if the positioning system is still available to offer trustable position estimation. Thus, the integrity computing entity may only have to report a binary flag (0 and 1) to indicate whether the positioning system is available or not.  Thus, in this case the LCS client can be directly informed about the system availability, without conducting further evaluation by itself.

In order to be generic and adhere to different use cases, it seems limited to make a decision about only one of these modes. Which mode that is appropriate depends on device capability, use case aspects, latency, software architecture, etc. Therefore, for flexibility and adaptability to use cases, it is natural to support both modes in Rel. 17

Both modes 1 and 2 for integrity result reporting have merits and applies to different scenarios

Support both mode 1 and 2 for integrity result reporting.

# 4	Quality indicators for integrity assessment

The focus of the integrity assistance data signaling discussion is on feared event characterization as well as feared even flags. The feared event characterization includes different error distribution aspects to support computations of protection levels

## 4.1	OSR quality indicators

For OSR, the assistance data has been leveraged by the RTCM MSM message definitions and it is natural that the RTCM MSM are used as input to the location server, which translates into 3GPP LPP.

The RTCM MSM messages are generically represented by the IE GNSS-RTK-Observations-r15 with optional fields to enable representation of all the MSM types. The RTCM MSM types are listed in Table 1 below.

Table 1. MSM types, satellite signal attributes, from table 3.5-71 [2]

| MSM Type | Message Name |
| --- | --- |
| MSM1 | Compact GNSS Pseudoranges |
| MSM2 | Compact GNSS PhaseRanges |
| MSM3 | Compact GNSS Pseudoranges and PhaseRanges |
| MSM4 | Full GNSS Pseudoranges and PhaseRanges plus Carrier to Noise Ratio |
| MSM5 | Full GNSS Pseudoranges, PhaseRanges, PhaseRangeRate and Carrier to Noise Ratio |
| MSM6 | Full GNSS Pseudoranges and PhaseRanges plus Carrier to Noise Ratio (high resolution) |
| MSM7 | Full GNSS Pseudoranges, PhaseRanges, PhaseRangeRate and Carrier to Noise Ratio (high resolution) |

Table 2 further lists the attribute resolution per MSM types in terms of standard or high resolution.

Table 2. MSM satellite signal attribute resolutions, standard or high.

| Attribute | MSM1 | MSM2 | MSM3 | MSM4 | MSM5 | MSM6 | MSM7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GNSS signal fine Pseudoranges | Std |  | Std | Std | Std | High | High |
| GNSS signal fine Phaserange data |  | Std | Std | Std | Std | High | High |
| GNSS Phaserange Lock Time Indicator |  | Std | Std | Std | Std | High | High |
| Half-cycle ambiguity indicator |  | - | - | - | - | - | - |
| GNSS signal Carrier-to-Noise Ratio |  |  |  | Std | Std | High | High |
| GNSS signal fine Phaserange Rates |  |  |  |  | Std |  | High |

The MSM6 and MSM7 provides observations in high resolution, while the other MSM types provide observations in standard resolution. For the IE GNSS-RTK-Observations-r15, all the attributes are represented according to the high resolution.

-- ASN1START

GNSS-RTK-Observations-r15 ::= SEQUENCE {

epochTime-r15							GNSS-SystemTime,

gnss-ObservationList-r15				GNSS-ObservationList-r15,

...

}

GNSS-ObservationList-r15 ::= SEQUENCE (SIZE(1..64)) OF GNSS-RTK-SatelliteDataElement-r15

GNSS-RTK-SatelliteDataElement-r15 ::= SEQUENCE{

svID-r15								SV-ID,

integer-ms-r15							INTEGER (0..254)				OPTIONAL,	-- Need ON

rough-range-r15							INTEGER (0..1023),

rough-phase-range-rate-r15				INTEGER (-8192..8191)			OPTIONAL,	-- Need ON

gnss-rtk-SatelliteSignalDataList-r15	GNSS-RTK-SatelliteSignalDataList-r15,

...

}

GNSS-RTK-SatelliteSignalDataList-r15 ::= SEQUENCE (SIZE(1..24)) OF

GNSS-RTK-SatelliteSignalDataElement-r15

GNSS-RTK-SatelliteSignalDataElement-r15 ::= SEQUENCE {

gnss-SignalID-r15					GNSS-SignalID,

fine-PseudoRange-r15				INTEGER (-524288..524287),

fine-PhaseRange-r15					INTEGER (-8388608..8388607),

lockTimeIndicator-r15				INTEGER (0..1023),

halfCycleAmbiguityIndicator-r15		BIT STRING (SIZE (1)),

carrier-to-noise-ratio-r15			INTEGER (0..1023)					OPTIONAL,	-- Need ON

fine-PhaseRangeRate-r15				INTEGER (-16384..16383)				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

For a UE, especially based on a legacy positioning engine defined for RTCM MSM, one possible handling of the 3GPP LPP IE is to translate that into MSM and forward to the positioning engine. In this case, it is not possible for the UE to determine if the IE originates from MSM4 or MSM6, or from MSM5 or MSM7. Furthermore, the device may use the standard/high resolution information to assess the observation uncertainty in addition to the RTK residuals information.

It is not possible to determine if an IE GNSS-RTK-Observations-r15 originates from MSM4 or MSM6, of from MSM5 or MSM7.

Devices that internally translates the obtained GNSS-RTK-Observations-r15 into corresponding MSM cannot correctly do that.

A simple indication can solve this issue – by indicating an optional indication about what resolution that the information originates from.

Option 1, Indicate optionally that a standard resolution has been used. If omitted, the assumption is that a high resolution has been used.

-- ASN1START

GNSS-RTK-Observations-r15 ::= SEQUENCE {

epochTime-r15							GNSS-SystemTime,

gnss-ObservationList-r15				GNSS-ObservationList-r15,

...,

[[

gnss-ObservationStdRes-r15			NULL							OPTIONAL

]]

}

GNSS-ObservationList-r15 ::= SEQUENCE (SIZE(1..64)) OF GNSS-RTK-SatelliteDataElement-r15

GNSS-RTK-SatelliteDataElement-r15 ::= SEQUENCE{

svID-r15								SV-ID,

integer-ms-r15							INTEGER (0..254)				OPTIONAL,	-- Need ON

rough-range-r15							INTEGER (0..1023),

rough-phase-range-rate-r15				INTEGER (-8192..8191)			OPTIONAL,	-- Need ON

gnss-rtk-SatelliteSignalDataList-r15	GNSS-RTK-SatelliteSignalDataList-r15,

...

}

GNSS-RTK-SatelliteSignalDataList-r15 ::= SEQUENCE (SIZE(1..24)) OF

GNSS-RTK-SatelliteSignalDataElement-r15

GNSS-RTK-SatelliteSignalDataElement-r15 ::= SEQUENCE {

gnss-SignalID-r15					GNSS-SignalID,

fine-PseudoRange-r15				INTEGER (-524288..524287),

fine-PhaseRange-r15					INTEGER (-8388608..8388607),

lockTimeIndicator-r15				INTEGER (0..1023),

halfCycleAmbiguityIndicator-r15		BIT STRING (SIZE (1)),

carrier-to-noise-ratio-r15			INTEGER (0..1023)					OPTIONAL,	-- Need ON

fine-PhaseRangeRate-r15				INTEGER (-16384..16383)				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

Option 2, Indicate optionally that a high resolution has been used. If omitted, the assumption is that a standard resolution has been used.

-- ASN1START

GNSS-RTK-Observations-r15 ::= SEQUENCE {

epochTime-r15							GNSS-SystemTime,

gnss-ObservationList-r15				GNSS-ObservationList-r15,

...,

[[

gnss-ObservationHighRes-r15			NULL							OPTIONAL

]]

}

GNSS-ObservationList-r15 ::= SEQUENCE (SIZE(1..64)) OF GNSS-RTK-SatelliteDataElement-r15

GNSS-RTK-SatelliteDataElement-r15 ::= SEQUENCE{

svID-r15								SV-ID,

integer-ms-r15							INTEGER (0..254)				OPTIONAL,	-- Need ON

rough-range-r15							INTEGER (0..1023),

rough-phase-range-rate-r15				INTEGER (-8192..8191)			OPTIONAL,	-- Need ON

gnss-rtk-SatelliteSignalDataList-r15	GNSS-RTK-SatelliteSignalDataList-r15,

...

}

GNSS-RTK-SatelliteSignalDataList-r15 ::= SEQUENCE (SIZE(1..24)) OF

GNSS-RTK-SatelliteSignalDataElement-r15

GNSS-RTK-SatelliteSignalDataElement-r15 ::= SEQUENCE {

gnss-SignalID-r15					GNSS-SignalID,

fine-PseudoRange-r15				INTEGER (-524288..524287),

fine-PhaseRange-r15					INTEGER (-8388608..8388607),

lockTimeIndicator-r15				INTEGER (0..1023),

halfCycleAmbiguityIndicator-r15		BIT STRING (SIZE (1)),

carrier-to-noise-ratio-r15			INTEGER (0..1023)					OPTIONAL,	-- Need ON

fine-PhaseRangeRate-r15				INTEGER (-16384..16383)				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

Add an optional indicator in the IE GNSS-RTK-Observations-r15 for the attribute resolution of the origin MSM message.

## 4.2	SSR Quality indicators

For SSR, there is already an introduced SSR User Range Accuracy (URA) information element. While this could be considered sufficient to assess the SSR quality, it seems valuable to be able to include quality indicators that are specific to the different specific information elements, at least as an option.

In more details:

- SSR Orbit corrections are provided as radial, along track and cross track corrections, with an optional velocity component in each of these dimensions. Corresponding uncertainty extensions per field is a natural extension

- SSR clock corrections are provided as a parameterized polynomial where an uncertainty per parameter is natural, but also an uncertainty representing the correction as a whole

- SSR code bias and phase bias are naturally extended with an uncertainty per field

- SSR atmospheric delay models already come with quality indicators for the STEC and gridded corrections

RAN2 to discuss whether the existing SSR quality indicators are sufficient for protection level assessments as part of integrity procedures.

# 5	Data Integrity

For reliable communication, it is common to provide checksum. RAN2 have previously added checksum for example in PDCP for Uplink data compression as example below TS 36.323.

An example of UDC Checksum calculation

The current UDC compression/decompression buffer has the following binary values for example:

Header <1,1,0,0,0,1,0,1,0,0,1,1,1,1,1,1,0,0,0,1,1,0,0,1,0,1,0,1,0,0,0,1, ……, 0,1,1,1,1,1,0,1,1,0,0,0,1,0,1,0,1,0,0,1,1,1,1,1,1,0,0,1,1,1,0,0> Tail

The sum of the first 4 bytes and the last 4 bytes can be calculated:

1100+0101+0011+1111+0001+1001+0101+0001+0111+1101+1000+1010+1001+1111+1001+1100 = 10000110;

And checksum value will be one's complement of the right-most 4 bits (i.e. 4 LSB) of the above sum. Hence checksum is 1001.

The CRC/checksum as shown above should not be too many bits otherwise it incurs overhead.

RAN2 to discuss the need of checksum in LPP layer for improved reliability and if need be a simple checksum with maximum one octet be defined for carrying Integrity related information

# Conclusion

In the previous sections we made the following observations:

Observation 1	Many integrity aspects for GNSS and other positioning methods are expected to be similar and certain parts can be in common.

Observation 2	Existing procedures, messages and information elements already includes integrity components and extensions to accommodate integrity seems plausible.

Observation 3	Both modes 1 and 2 for integrity result reporting have merits and applies to different scenarios

Observation 4	It is not possible to determine if an IE GNSS-RTK-Observations-r15 originates from MSM4 or MSM6, of from MSM5 or MSM7.

Observation 5	Devices that internally translates the obtained GNSS-RTK-Observations-r15 into corresponding MSM cannot correctly do that.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to liase with RTCM SC 104 and RTCM SC 134 as specified in as specified in R2-2105973.

Proposal 2	RAN2 to agree on defining integrity level classification for integrity support. The UE and the network may report their supported levels in the signalling with associated QoS.

Proposal 3	RAN2 to liase with SA1, SA2 and CT4 to provide signalling of Integrity based upon associated QoS as specified in R2-2105973.

Proposal 4	RAN2 to allow a generic Integrity description to be captured by SA1, SA2 and CT4 specification.

Proposal 5	Design signalling, procedures and information element additions/extensions with both GNSS and other positioning methods in mind.

Proposal 6	As baseline, use existing procedures, messages and information elements with extensions to accommodate positioning integrity.

Proposal 7	Support both mode 1 and 2 for integrity result reporting.

Proposal 8	Add an optional indicator in the IE GNSS-RTK-Observations-r15 for the attribute resolution of the origin MSM message.

Proposal 9	RAN2 to discuss whether the existing SSR quality indicators are sufficient for protection level assessments as part of integrity procedures.

Proposal 10	RAN2 to discuss the need of checksum in LPP layer for improved reliability and if need be a simple checksum with maximum one octet be defined for carrying Integrity related information

# References

RP-210903 , “Revised WID on NR Positioning Enhancements“, March 16 - 26, 2021

RTCM SC 104 v3.x

3GPP TR 38.857, “Positioning enhancement

R2-2105973, draft LS to different groups, Ericsson