3GPP TSG-RAN2 Meeting # 114-e	R2-2105997

# Electronic Meeting,  19th May - 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1816 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On WLAN-BT configuration in reportConfigInterRAT in LTE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2021-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Durng RAN2#113bis meeting, the following agreement was made. R2-2104002 Discussion on the location reporting in inter-RAT measurement for immediate MDT Huawei, HiSilicon discussion Rel-16 NR_SON_MDT-Core => The changes are agreed and will be merged to the big CR. This CR ws agreed for NR and this CR enables the configuration of the location information (includeCommonLocationInfo, includeBT-Meas, includeWLAN-Meas, includeSensor-Meas) related IEs in the reportConfigInterRAT of TS 38.331. In the LTE specificaiton, the UE can be configured with includeLocationInfo in the reportConfigInterRAT. However, the UE cannot be configured with includeBT-Meas or includeWLAN-Meas. In LTE, immMeasBT-r15 and immMeasWLAN-r15 are defined for UE capabilities. With the previous changes, it may force the legacy UE to support BT/WLAN measurements for Inter-RAT cases. So it is proposed to introduce separate UE capabilities so that a Rel-16 UE supporting immMeasBT-r15/immMeasWLAN-r15 can decide whether to also support new changes or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New capabilities (immMeasBTInterRAT-r15, immMeasWLANInterRAT-r15) related to the inclusion of includeBT-Meas and includeWLAN-Meas in the reportConfigInterRAT are introduced. Impact analysis Impacted functionality: Immediate MDT report Inter-operability analysis: No inter-operability issues foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE will not support the inclusion of WLAN and BT measurement information inclusion in LTE inter-RAT measurement reports. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331 CR 4674 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/* Start of change */

### 4.3.13	UE-based network performance measurement parameters

#### 4.3.13.1	loggedMeasurementsIdle

This parameter defines whether the UE supports logged measurements in RRC_IDLE upon request from the network. A UE that supports logged measurements in RRC_IDLE shall also support a minimum of 64kB memory for log storage.

#### 4.3.13.2	standaloneGNSS-Location

This parameter defines whether the UE is equipped with a standalone GNSS receiver that may be used to provide detailed location information in RRC measurement report and logged measurements in RRC_IDLE.

#### 4.3.13.3	Void

#### 4.3.13.4	loggedMBSFNMeasurements-r12

This parameter defines whether the UE supports logged MBSFN measurement in RRC_IDLE and RRC_CONNECTED upon request from the network. A UE that supports logged MBSFN measurements shall also support a minimum of 64kB memory for log storage. A UE that supports logged MBSFN measurements shall also support logged measurements in RRC_IDLE upon request from the network.

#### 4.3.13.5	locationReport-r14

This parameter defines whether the UE supports reporting of its geographical location information to eNB.

#### 4.3.13.6	loggedMeasBT-r15

This parameter indicates whether the UE supports Bluetooth measurements in RRC_IDLE mode.

#### 4.3.13.7	loggedMeasWLAN-r15

This parameter indicates whether the UE supports WLAN measurements in RRC_IDLE mode.

#### 4.3.13.8	immMeasBT-r15

This parameter indicates whether the UE supports Bluetooth measurements in RRC_CONNECTED mode.

#### 4.3.13.9	immMeasWLAN-r15

This parameter indicates whether the UE supports WLAN measurements in RRC_CONNECTED mode.

#### 4.3.13.10	ul-PDCP-AvgDelay-r16

This parameter indicates whether the UE supports UL PDCP Packet Average Delay measurement (as specified in TS 38.314 [41]) and reporting in RRC_CONNECTED state.

#### 4.3.13.11	immMeasBTInterRAT-r16

This parameter indicates whether the UE supports Bluetooth measurements in RRC_CONNECTED mode in the inter RAT reporting configuration.

#### 4.3.13.12	immMeasWLANInterRAT-r16

This parameter indicates whether the UE supports WLAN measurements in RRC_CONNECTED mode in the inter RAT reporting configuration.

/* End of change */