3GPP TSG-RAN WG2 Meeting #114-e		R2-2106036

Electronic, 19th May – 27th May 2021

Source:	Qualcomm Incorporated

Title:	On logging of on-demand SI information

Agenda Item:	8.13.2.2 2-step RA related SON aspects

Document for:	Discussion and Decision

# Introduction & Background

Rel-17 SON/MDT [1] includes the following RAN2-led objectives in the context of MDT.

- Support of data collection for MDT features for identified use cases, including 2-step RACH optimization and leftovers of Rel-16 SON/MDT WI (MDT enhancements and MDT for MR-DC) [RAN2, RAN3, RAN4]

- Enhancement of logged and immediate MDT (including coexistence with IDC) [RAN2, RAN3]

- Enhancement of reporting e.g. RLF and accessibility measurements, Successful Handover reporting [RAN2, RAN3].

- Specification of MDT for MR-DC [RAN2, RAN3, RAN4]

Furthermore, in RAN2#111-emeeting [2], the following topics are prioritized:

  - Study the support of logged and Immediate MDT in the MR-DC scenario.

In this contribution draft, we consider different aspects of logged measurement enhancements in MR-DC and CA scenarios.

# Discussion

In the RAN2#113-bis-emeeting, we had the following agreements for on-demand SI:

Agreements:

1	UE reports the SIBs that UE actually intends to request.

2	Both Msg1-based and Msg3-based SI request related information are supported.

While RAN2 made the above agreements, it is FFS whether we need to enhance the RA-report for the logging of on-demand SI information.

Proposal 4: It is FFS whether to extend the current RA-report to include the on-demand SI-related information.

Note that in the current procedures, the RA report logs the information related to the successful RACH procedures. Furthermore, the objective of logging the on-demand SI-related information is to optimize the broadcasting and non-broadcasting SIs sets rather than optimizing the RACH procedure. Therefore, we believe that the on-demand SI-related information can be logged in IDLE or INACTIVE states for both successful and failed SI acquisitions.

Observation 1: In the RA reports, UE logs the parameters only for the successful RACH procedures.

Proposal 1: No enhancements to the RA report for on-demand SI need to be considered.

Proposal 2: Enhance the logged measurement configuration to log the on-demand SI-related measurements for both msg1 and msg3 for IDLE and INACTIVE states.

Proposal 3: UE logs the following measurements in logged measurement report for on-demand SI:

- System information that UE intended to request

- Whether on-demand SI is requested using msg 1 or msg 3

# Conclusion

Observation 1: In the RA reports, UE logs the parameters only for the successful RACH procedures.

Proposal 1: No enhancements to the RA report for on-demand SI need to be considered.

Proposal 2: Enhance the logged measurement configuration to log the on-demand SI-related measurements for both msg1 and msg3 for IDLE and INACTIVE states.

Proposal 3: UE logs the following measurements in logged measurement report for on-demand SI:

- System information that UE intended to request

- Whether on-demand SI is requested using msg 1 or msg 3

# Reference

[1] RP-201281, “Revised WID on the enhancement of data collection for SON/MDT in NR and EN-DC”

[2] RAN2#111e meeting report

[3] RAN2-113-emeeting

[4] Post RAN2-113-emeeting [854]

[5] TS 38.331