3GPP TSG-RAN WG2 Meeting #114-e	R2-2106208

E-meeting, 19– 27 May, 2021 													  Revision of R2-2104561

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4633 | rev | 2 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections on F1 over LTE for IAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2021-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For an IAB-node in EN-DC mode, it has been agreed that the F1-C traffic can be transmitted via its NR BH link or via LTE Uu. According to the configuration provided by the IAB-donor-CU, an IAB node in EN-DC mode can use LTE Uu or NR BH or both to transmit the F1-C related messages. In addition, the UL information transfer procedure will be reused to transmit the UL F1-C messages if the path via LTE Uu is configured. According to the description about the initiation of the UL information transfer in section 5.6.2.2 : “an IAB-MT in RRC_CONNECTED initiates the UL information transfer procedure whenever there is a need to transfer F1-C related information.” It reads as if that the UL information transfer procedure will be initiated for any F1-C messages transmission regardless of the IAB-MT configuration, which is incorrect since such initiation is unnecessary if the IAB node uses NR BH link rather than LTE Uu to transmit the F1-C related messages. So the description should be changed to indicate that the F1-C transmission over LTE is just optional, not mandatory. - The need code is used in downlink, rather than the UL messages, so the need code of dedicatedInfoF1c-r16 IE and dedicatedInfoType-r16 IE in the ULInformationTransfer message are not needed, and should be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In section 5.6.2.2, change the IAB specific initiation of UL information transfer procedure to “an IAB-MT in RRC_CONNECTED may initiate the UL information transfer procedure whenever there is a need to transfer F1-C related information.”. - In section 6.2.2, remove the need codes for the dedicatedInfoF1c-r16 IE and dedicatedInfoType-r16 IE in the ULInformationTransfer message. Impact analysis Impacted functionality: UL F1-C message transfer of IAB-node in EN-DC mode. Inter-operability: - If the IAB-MT is implemented according to this CR but the network is not, the network may not parse the ULInformationTransfer message from IAB node correctly. - If the network is implemented according to this CR but the IAB-MT is not, there is no inter-operability issue forseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - An IAB node in EN-DC mode will only transmit the UL F1-C related messages via the LTE link, the NR BH link will not be used even if configured by the donor CU. - The IAB donor CU may not be able to correctly parse an ULInformationTransfer message from an IAB node. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | (rev 0) Contains several proposed changes on top of those in current version (rev 1); many of these were merged with the separate rapporteur CR, and one was rejected. (rev 1) Focus reduced from rev 0 to: 1) clarification of Initiation of UL Information Transfer; and 2) removal of ‘need’ codes from ULInformationTransfer message ASN.1. (rev 2) resubmit the rev 1(R2-2104561) which was agreed in principle to RAN2-114e meeting. |  |  |  |  |  |  |  |  |

Start of Change

5.6.2	UL information transfer

5.6.2.1	General

Figure 5.6.2.1-1: UL information transfer

The purpose of this procedure is to transfer NAS or (tunnelled) non-3GPP dedicated information from the UE to E-UTRAN, or to transfer F1-C related information from IAB-DU to IAB Donor-CU via IAB-MT in RRC_CONNECTED.

5.6.2.2	Initiation

A UE in RRC_CONNECTED initiates the UL information transfer procedure whenever there is a need to transfer NAS, non-3GPP dedicated information, except at RRC connection establishment or resume in which case the NAS information is piggybacked to the RRCConnectionSetupComplete or RRCConnectionResumeComplete message correspondingly. In addition, an IAB-MT in RRC_CONNECTED may initiate the UL information transfer procedure whenever there is a need to transfer F1-C related information. The UE initiates the UL information transfer procedure by sending the ULInformationTransfer message. When CDMA2000 information has to be transferred, the UE shall initiate the procedure only if SRB2 is established. When F1-C related information has to be transferred, the IAB-MT shall initiate the procedure only if SRB2 is established.

Next Change

6.2.2	Message definitions

>>>>>>>>>> unchanged parts are skipped<<<<<<<<<<

–	ULInformationTransfer

The ULInformationTransfer message is used for the uplink transfer of NAS or non-3GPP dedicated information.

Signalling radio bearer: SRB2 or SRB1(only if SRB2 not established yet). If SRB2 is suspended, the UE does not send this message until SRB2 is resumed

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

ULInformationTransfer message

-- ASN1START

ULInformationTransfer ::=			SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

ulInformationTransfer-r8			ULInformationTransfer-r8-IEs,

ulInformationTransfer-r16			ULInformationTransfer-r16-IEs,

spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

ULInformationTransfer-r8-IEs ::=	SEQUENCE {

dedicatedInfoType					CHOICE {

dedicatedInfoNAS					DedicatedInfoNAS,

dedicatedInfoCDMA2000-1XRTT			DedicatedInfoCDMA2000,

dedicatedInfoCDMA2000-HRPD			DedicatedInfoCDMA2000

},

nonCriticalExtension				ULInformationTransfer-v8a0-IEs		OPTIONAL

}

ULInformationTransfer-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

ULInformationTransfer-r16-IEs ::=	SEQUENCE {

dedicatedInfoType-r16				CHOICE {

dedicatedInfoNAS-r16				DedicatedInfoNAS,

dedicatedInfoCDMA2000-1XRTT-r16		DedicatedInfoCDMA2000,

dedicatedInfoCDMA2000-HRPD-r16		DedicatedInfoCDMA2000

}																		OPTIONAL,

dedicatedInfoF1c-r16				DedicatedInfoF1c-r16				OPTIONAL,

nonCriticalExtension				ULInformationTransfer-v8a0-IEs	OPTIONAL

}

-- ASN1STOP

End of Change