3GPP TSG-RAN2 Meeting #114-e	R2-2106214

# Online, 19th May 2021 - 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4679 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add ack-NACK-NumRepetitions for PUR-Config-NB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For NB-IoT UE in RRC_CONNECTED state, ack-NACK-NumRepetitions configured in NPUSCH-ConfigDedicated-NB-r13 would be used. And it describes that if this field is absent and no value was configured via dedicated signalling, the value used for reception of Msg4 is used. But in PUR-Config-NB, ack-NACK-NumRepetitions is not included, and the value used for reception of Msg4 is not suitable for PUR. The reason is, ack-NACK-NumRepetitions-Msg4 is configured per CEL. UE cannot determine which CEL’s value can be used for PUR procedure. Then UE cannot decide the HARQ ack-NACK repetition number to be used for PUR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce ack-NACK-NumRepetitions into PUR-Config-NB. Impact Analysis Impacted functionality: The changes only impacts NB-IoT PUR procedure. Inter-operability: No inter-operability issue is found. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE cannot decide the HARQ ack-NACK repetition number to be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

#### 6.7.3.2	NB-IoT Radio resource control information elements

//Skip the unrelated part//

#### –	PUR-Config-NB

The IE PUR-Config-NB is used to specify PUR configuration.

PUR-Config-NB information element

-- ASN1START

PUR-Config-NB-r16	::=				SEQUENCE {

pur-ConfigID-r16					PUR-ConfigID-NB-r16				OPTIONAL,	--Need OR

pur-TimeAlignmentTimer-r16			INTEGER (1..8)				OPTIONAL,	--Need OR

pur-NRSRP-ChangeThreshold-r16		SetupRelease {PUR-NRSRP-ChangeThreshold-r16}

OPTIONAL,	--Need ON

pur-ImplicitReleaseAfter-r16		ENUMERATED {n2, n4, n8, spare}	OPTIONAL,	--Need OR

pur-RNTI-r16						C-RNTI							OPTIONAL,	--Need ON

pur-ResponseWindowTimer-r16			ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}

OPTIONAL,	--Need ON

pur-StartTimeParameters-r16			SEQUENCE {

periodicityAndOffset-r16			PUR-PeriodicityAndOffset-NB-r16,

startSFN-r16						INTEGER (0..1023),

startSubframe-r16					INTEGER (0..9),

hsfn-LSB-Info-r16					BIT STRING (SIZE(1))

}																	OPTIONAL,	--Need ON

pur-NumOccasions-r16				ENUMERATED {one, infinite},

pur-PhysicalConfig-r16				SEQUENCE {

carrierConfig-r16					CarrierConfigDedicated-NB-r13,

npusch-NumRUsIndex-r16				INTEGER (0..7),

npusch-NumRepetitionsIndex-r16		INTEGER (0..7),

npusch-SubCarrierSetIndex-r16		CHOICE {

khz15								INTEGER (0..18),

khz3dot75							INTEGER (0..47)

},

npusch-MCS-r16						CHOICE {

singleTone							INTEGER (0..10),

multiTone							INTEGER (0..13)

},

p0-UE-NPUSCH-r16					INTEGER (-8..7),

alpha-r16							ENUMERATED {al0, al04, al05, al06,

al07, al08, al09, al1},

npusch-CyclicShift-r16				ENUMERATED {n0, n6},

npdcch-Config-r16					NPDCCH-ConfigDedicated-NB-r13

}	OPTIONAL,	-- Need ON

...,

[[ack-NACK-NumRepetitions-v16xy		ACK-NACK-NumRepetitions-NB-r13	OPTIONAL,	-- Need ON

]]

}

PUR-NRSRP-ChangeThreshold-r16 ::=	SEQUENCE {

increaseThresh-r16						NRSRP-ChangeThresh-NB-r16,

decreaseThresh-r16						NRSRP-ChangeThresh-NB-r16	OPTIONAL	--Need OP

}

NRSRP-ChangeThresh-NB-r16 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1}

-- ASN1STOP

PUR-Config-NB field descriptions

ack-NACK-NumRepetitions Number of repetitions for the ACK NACK resource unit carrying HARQ response to NPDSCH, see TS 36.213 [23], clause 16.4.2.

alpha Parameter: αc(3). See TS 36.213 [23], clause 16.2.1.1.1.

carrierConfig Carrier used for PUR.

hsfn-LSB-Info LSB of the H-SFN corresponding to the last subframe of the first transmission of RRCConnectionRelease message containing pur-Config.

npdcch-Config NPDCCH configuration for PUR.

npusch-CyclicShift Parameter: ncs. See TS 36.211 [21], clause 10.1.4.1.2. Value n0 corresponds to value 0 and value n6 corresponds to value 6.

npusch-MCS Index to tables specified in TS 36.213 [23], Table 16.5.1.2-1 and Table 16.5.1.2-2 for single tone and multi tone respectively, that defines modulation and TBS index for NPUSCH for PUR.

npusch-NumRepetitionsIndex Index to a table specified in TS 36.213 [23], Table 16.5.1.1-3, that defines number of repetitions for NPUSCH for PUR.

npusch-NumRUsIndex Index to a table specified in TS 36.213 [23], Table 16.5.1.1-2, that defines number of resource units for NPUSCH for PUR.

npusch-SubCarrierSetIndex For NPUSCH transmission with subcarrier spacing 3.75 kHz, indicates the subcarrier used for PUR specified in TS 36.213 [23]. For NPUSCH transmission with subcarrier spacing 15 kHz, index to a table specified in TS 36.213 [23], Table 16.5.1.1-1, that defines the set of subcarriers for NPUSCH for PUR.

p0-UE-NPUSCH Parameter: . See TS 36.213 [23], clause 16.2.1.1.1, unit dB.

pur-ImplicitReleaseAfter Number of consecutive PUR occasions that can be skipped before implicit release of PUR configuration. Value n2 corresponds to 2 PUR occasions, value n4 corresponds to 4 PUR occasions, and so on.

pur-NRSRP-ChangeThreshold Threshold(s) of change in serving cell NRSRP in dB for TA validation. Value dB4 corresponds to 4 dB, value dB6 corresponds to 6 dB, and so on. When pur-NRSRP-ChangeThreshold is set to setup, if decreaseThrsh is absent the value of increaseThresh is also used for decreaseThresh.

pur-NumOccasions Number of PUR occasions. Value one corresponds to 1 PUR occasion, and value infinite corresponds to an infinite number of PUR occasions.

pur-PeriodicityAndOffset Indicates the periodicity for the PUR occasions and time offset until the first PUR occasion.

pur-ResponseWindowTimer Duration of the PUR response window in TS 36.321 [6]. Value in PDCCH periods. Value pp2 corresponds to 2 PDDCH periods, pp3 corresponds to 3 PDCCH periods, and so on. The value considered by the UE is: pur-ResponseWindowSize = Min (signaled value x PDCCH period, 10.24s).

pur-TimeAlignmentTimer Value of the time alignment timer for PUR. Value in number of periodicity of PUR.

<End of the change>