3GPP TSG-RAN WG2 Meeting #114 electronic	R2-2106240

Online, May 19 – May 27, 2021

Agenda Item:	8.1.2.3

Source:	CMCC

Title:	Discussion on Mobility with Service Continuity

Document for:	Discussion

Introduction

In RAN2#112e, the discussion on mobility issues has a good progress, and following agreements have been achieved [1]:

R2 aim to support lossless handover for MBS-MBS mobility for service that requires this (TBD which detailed scenario but at least PTP-PTP) In order to support the lossless handover for 5G MBS services, at least DL PDCP SN synchronization and continuity between the source cell and the target cell should be guaranteed by the network side to realize. The design of specific approach to realize this can be involved with WG RAN3. From network side, the source gNB may forward the data to the target gNB and the target gNB will deliver the forwarding data. Meanwhile, the SN STATUS TRANSFER should be extended to cover the PDCP SN for MBS data; Then (TBD after or in parallel) the UE receives the MBS in the target cell by the target cell according to target configuration. From UE side, PDCP status report may be supported as well.

In this contribution, we mainly analyse lossless handover scenarios except for PTP-PTP, and possible enhancements in handover procedure.

Discussion

Lossless HO scenarios

It was agreed to support Lossless HO for MBS in last RAN2#112 meeting, considering there’s high reliability requirements for some services, and it could be supported in PTP-PTP HO scenario naturally, where re-transmission by PTP leg is possible, similar to unicast HO.

As for other HO scenarios, to achieve lossless handover, it’s better to configure MBS as an AM bearer for PDCP status report and PDCP re-establishment/recovery. Based on the current progress, the majority of companies do not support RLC-AM mode for PTM leg. Since the design of PTM transmission could be complex which may introduce heavy standard workload, while no much reliability gains can be achieved with extra transmission delay. Therefore, to achieve lossless HO for high reliability requirement, it’s rational to configure a PTP leg with RLC AM mode in the target node. And, it’s network implementation whether to configure PTM leg in target node.

Observation 1: To achieve lossless HO for high reliability requirement, it’s rational to configure a PTP leg with RLC AM mode in the target node.

Base on the analysis above, besides PTP-PTP, lossless HO could be supported in PTM->PTP, PTM+PTP->PTP, PTM->PTP+PTM and PTM+PTP->PTM+PTP scenarios.

Observation 1: Lossless HO could also be supported in PTM-PTP, PTM+PTP-PTP, PTM-PTP+PTM and PTM+PTP-PTM+PTP besides PTP-PTP.

Proposal 1: RAN2 is kindly asked to agree the above scenarios as lossless HO scenarios.

Handover Procedure enhancement

The basic handover procedure for unicast is defined in [2], consisting of three phases: handover preparation, handover execution and handover completion, as shown in Figure1, and it could be baseline for MBS HO procedure.

Figure 1 unicast handvoer procedure

Handover Preparation

This phase mainly includes measurement configuration and reporting, and the target gNB confirms the source gNB’s handover request. With regard to the measurement configuration and reporting, the legacy unicast measurement mechanism could be used, while for the confirmation of handover request, it’s better to embed UE’s MBS related information into the Handover Request sent to the target gNB, which is helpful for the target gNB’s admission control, transmission mode decision, and multicast session setup, if there’s no multicast session at the target node yet. In LTE MBMS, for handover preparation, the source eNB forwards the MBMS interest of the UE, if available, to the target eNB, so the MBS related information for NR could be on-going multicast services and/or UE’s interest services. Besides, the target gNB could provide the MBS configuration in the Handover Request Acknowledge message, if the target gNB determines to accept the UE.

Observation 2: Current measurement configuration and reporting mechanism could be reused for multicast handover.

Observation 3: The MBS related information, e.g., on-going multicast services and/or UE’s interest services embedded into Handover Request is helpful for the target gNB’s admission control, transmission mode decision and multicast session setup.

Proposal 2: MBS related information, e.g., on-going multicast services and UE’s interest services could be embedded into Handover Request for the target gNB’s better decision and proper actions.

Proposal 3: The target node could provide the MBS configuration in the Handover Request Acknowledge message.

Handover execution and completion

For unicast handover, there is only one PDU session established from CN to gNB. After the handover, UPF will switch the path from source gNB to target gNB, as depicted in Figure 1. And the Service continuity is mainly supported by the SN STATUS TRANSFER message sending from the source gNB to the target gNB.

In the MBS handover scenario, MBS packets are sending to multiple UEs within one gNB, MBS sessions in the target gNB are always existing even there is no handover happening.

In addition, considering the different distances to the source gNB and to the target gNB, the different radio link quality of different gNBs, and difference of gNB scheduling, mismatching of packets delivery status still exists in the target gNB and the source gNB, even approaches to minimize the data loss like DL PDCP SN synchronization, and data forwarding was agreed to introduce in last meeting. This may lead to repeated transmission and waste of resource.

Observation 4: In the MBS handover scenario, considering that MBS packets are sending to multiple UEs within one gNB, MBS sessions in target gNB still be there even though there is no handover happening.

Observation 5: There could be mismatching of packets delivery status in the source gNB and the target gNB, which may lead to repeated transmission and waste of resource.

To solve the problem mentioned above, one potential solution is that the target gNB informs the source gNB its delivery status besides SN Status transfer, which is used for the source gNB transmission status notification, and this could be discussed in RAN3.

Proposal 4: Message to indicate the target gNB’s delivery status to the source gNB could be introduced to avoid repeated transmission and waste of resource.

Mobility between MBS supporting node and non-MBS supporting node

In the previous discussion, RAN2 agreed to focus on MBS-MBS scenario initially, while other scenarios will be discussed later. In SA2, there are conclusions on the other scenarios, like mobility between MBS-supporting node and non-MBS supporting node, including the following two cases:

- Mobility from MBS supporting node to non-MBS supporting node

- Mobility from non-MBS supporting node to MBS supporting node

Mobility from MBS supporting node to non-MBS supporting node

In this case, source gNB could convert MRB to gNB first for mobility  and then initial the legacy unicast handover, which could reuse the legacy HO procedure as much as possible. Also, converting MRB to DRB during the HO procedure could also be considered, which relies on target gNB recognizing MBS context and reconfigure DRB to UE. No matter which way, it’s the RAN node (source gNB in the former solution, and target gNB in the latter solution) play the role of make the decision, and this procedure also involves CN delivery mode or session converting, therefore, we think RAN2 should take this into further discussion.

Observation 6: Mobility from MBS supporting node to non-supporting node mainly depends on RAN node’s decision.

Proposal 5: For the case mobility from MBS supporting node to non-MBS supporting node, MRB could be converted to DRB before or during the HO procedure.

Mobility from non-MBS supporting node to MBS supporting node

For this scenario, the most straightforward way is to trigger legacy unicast handover procedure first, then it’s network’s decision whether or when to convert DRB to MRB. Similar with the case mobility from MBS supporting node to non-MBS supporting node, it’s RAN node to make the decision or at least trigger the RB converting and session converting.

Proposal 6: For the case mobility from non-MBS supporting node to MBS supporting node, the legacy HO procedure could be performed first, and MRB to DRB converting could be performed after HO procedure.

Proposal 7: RAN2 is kindly asked to consider the procedure how RAN node decides or triggers the RB converting.

Besides, SA2 is considering the lossless requirement in mobility between MBS supporting node and non-MBS supporting node [3].

C) Whether the support for lossless handover with data forwarding from source NG-RAN to the target NG-RAN is needed, which needs confirmation by RAN.

When we talk about lossless supporting in MBS-MBS HO scenario, the main factor considered is about the service QoS requirements. Therefore, we think lossless HO should also be considered in mobility between MBS node and non-MBS node.

Proposal 8: Lossless HO should also be considered in mobility between MBS node and non-MBS node.

Conclusions

In this paper, we make analysis on how to achieve lossless in handover, and provide our view on possible lossless handover scenarios. Besides, based on the unicast handover procedure, we discussed the potential enhancement in the MBS handover procedure. We have the following observations and proposals:

Observation 1: Lossless HO could also be supported in PTM-PTP, PTM+PTP-PTP, PTM-PTP+PTM and PTM+PTP-PTM+PTP besides PTP-PTP.

Observation 2: Current measurement configuration and reporting mechanism could be reused for multicast handover.

Observation 3: The MBS related information, e.g., on-going multicast services and/or UE’s interest services embedded into Handover Request is helpful for the target gNB’s admission control, transmission mode decision and multicast session setup.

Observation 4: In the MBS handover scenario, considering that MBS packets are sending to multiple UEs within one gNB, MBS sessions in target gNB still be there even though there is no handover happening.

Observation 5: There could be mismatching of packets delivery status in the source gNB and the target gNB, which may lead to repeated transmission and waste of resource.

Observation 6: Mobility from MBS supporting node to non-supporting node mainly depends on RAN node’s decision.

Proposal 1: RAN2 is kindly asked to agree the above scenarios as lossless HO scenarios.

Proposal 2: MBS related information, e.g., on-going multicast services and UE’s interest services could be embedded into Handover Request for the target gNB’s better decision and proper actions.

Proposal 3: The target node could provide the MBS configuration in the Handover Request Acknowledge message.

Proposal 4: Message to indicate the target gNB’s delivery status to the source gNB could be introduced to avoid transmission and waste of resource.

Proposal 5: For the case mobility from MBS supporting node to non-MBS supporting node, MRB could be converted to DRB before or during the HO procedure.

Proposal 6: For the case mobility from non-MBS supporting node to MBS supporting node, the legacy HO procedure could be performed first, and MRB to DRB converting could be performed after HO procedure.

Proposal 7: RAN2 is kindly asked to consider the procedure how RAN node decides or triggers the RB converting.

Proposal 8: Lossless HO should also be considered in mobility between MBS node and non-MBS node.

References

- RAN2_112-e Chairman notes 2020-11-15_EOM

- TS 38.300 NR and NG-RAN Overall Description; Stage 2 (Release 16)

- TR 23.757 V17.0.0 (2021-03) Study on architectural enhancements for 5G multicast-broadcast services (Release 17)