3GPP TSG-RAN WG2 Meeting #114 electronic	R2-2106344

Online, 19 – 27 May 2021

Agenda Item:	8.7.3.

Source: 	MediaTek Inc.

Title:  	Other remaining issues on (re)selection

Document for:	Discussion and decision

Introduction

As discussed in [1] during RAN2 #113e-bis meeting, RAN2 had lots of agreements [2] related to baseline solution for NR Sidelink relay, most of them are similar to LTE relay re/selection mechanism and can be directly reused. In this tdoc, we address some remaining issues of relay (re)selection.

PC5 measurement to trigger relay reselection

In [2], proposal related to which PC5 measurement to trigger relay reselection is not discussed due to lack of time. RAN2 try to discuss the case when there is no data transmission over PC5 unicast link, how remote UE do PC5 measurement? Two alternatives are on the table:

- Alt-1: Based on only SL-RSRP. In case of no data transmission, remote UE can use keep-alive message or triggered PC5-S/CSI reporting from relay UE to perform SL-RSRP based on its implementation.

- Alt-2: Based on both SL-RSRP and SD-RSRP. If data is available, only SL-RSRP of data. In case of no data transmission, the remote UE triggers reselection based on SD-RSRP.

In our opinion, both options works, it is a matter of taste to decide which alternative is adopted. To make spec simple and clear, we prefer to use SL-RSRP only mechanism. Besides, PC5-S/CSI reporting is also a good idea to keep the channel up to date, it can help the transmission over PC5 unicast with lower BLER and higher spectrum efficiency.

Proposal 1: RAN2 to adopt purely SL-RSRP measurement for relay reselection. If there is no data transmission, remote UE can use keep-alive message or triggered PC5-S/CSI reporting to perform SL-RSRP measurements.

Conclusion

This document promulgated the following proposal:

Proposal 1: RAN2 to adopt purely SL-RSRP measurement for relay reselection. If there is no data transmission, remote UE can use keep-alive message or triggered PC5-S/CSI reporting to perform SL-RSRP measurements.

References

[1]	R2-2104287, Summary of AI 8.7.3 (relay selection/reselection), Qualcomm Incorporated

[2]	R2-2104305, Report from session on positioning and sidelink relay, Session Chair (MediaTek)