3GPP TSG-RAN WG2 Meeting #114 electronic	 R2-2106361

Online Meeting, May 19 – May 27, 2021

Agenda item:	8.1.3 Idle and Inactive mode UEs

Source:	Convida Wireless

Title:	NR MBS Configuration Information

Document for:	Discussion & Decision

# Introduction

The NR_MBS Work Item has as a main goal to enable “general MBS services over 5GS” for a number of use cases, including: public safety and mission critical, transparent IPv4/IPv6 multicast delivery, IPTV, software delivery over wireless, group communications and IoT applications. Each of these use cases would benefit from a mechanism of broadcast/multicast communication over the RAN.

One of the objectives listed in the NR MBS WID [1] includes:

  - Specify required changes to enable the reception of Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, with the aim of keeping maximum commonality between RRC_CONNECTED state and RRC_IDLE/RRC_INACTIVE state for the configuration of PTM reception. [RAN2, RAN1].

In order to enable this reception, the UE must be provided with the MBS configuration information. The following agreements on MBS configuration information for RRC IDLE and RRC_INACTIVE were achieved at RAN2#112-e [2], RAN2#113-e [3], and RAN2 #113bis-e [4]:

- UE receives the MBS configuration (for broadcast/delivery mode 2) by BCCH and/or MCCH (TBD), and this can be received in Idle / Inactive mode. Connected mode FFS (dep on UE cap and where service is provided etc). A notification mechanism is used to announce the change of MBS Control information.

- The two-step based approach (i.e. BCCH and MCCH) as adopted by LTE SC-PTM is reused for the transmission of PTM configuration for NR MBS delivery mode 2.

- Assume it is possible to reuse LTE SC-PTM mechanism for the CONNECTED UEs to receive the PTM configuration for NR MBS delivery mode 2, i.e. broadcast based manner.

- Assume that MCCH change notification mechanism is used to notify the changes of MCCH configuration due to session start for delivery mode 2 of NR MBS (other cases FFS, if any).

- The MCCH transmission window is defined by MCCH repetition period, MCCH window duration and radio frame/slot offset

- The modification period is defined for NR MCCH and NR MCCH contents are only allowed to be modified at each modification period boundary.

- The updated MCCH message should be sent in the same MCCH modification period where the change notification is sent.

- New RNTI is defined for scheduling MCCH.

- It is up to RAN1 to decide about the RNTI and DCI format used for MCCH change notifications.

- FFS whether to support multiple MCCH, e.g. to support different service types.

- RAN2 will discuss and down-select from the following two options for the UE to get aware of session stop/modification:

- Reading MCCH once per each MCCH modification period when receiving an ongoing broadcast session

- DCI used for MCCH notification indicates the change of an ongoing broadcast session

In this contribution, we discuss our views related to the MBS configuration with regard to: single vs multiple MCCH and MCCH change notification.

# Discussion

It is expected that the use cases for NR MBS will be much more diverse that those for eMBMS. In fact, SA2 has already agreed that the 5G QoS model and parameters defined in TS 23.501 [5] clause 5.7, also apply to MBS services. Consequently, we do expect that for different MBS services, the QoS requirements, especially in terms of latency, will be very varied.

Observation 1: NR MBS QoS requirements (especially latency) for different MBS services may be very different

In addition, it is expected that a UE may receive multiple MBS services simultaneously, and these services may have very different QoS requirements. Each of these services will have its own MBS configuration, and the rate of change of this MBS configuration will also depend on the type of MBS service. For example this may depend on the traffic pattern of the service.

Observation 2: UEs are expected to be able to receive multiple MBS services simultaneously. Each of these services may have its own MBS configuration.

RAN2 has already agreed to support Delivery Mode 2 in RRC_CONNECTED. The RRC_CONNECTED UEs may receive the MBS configuration either through dedicated signaling or through the same BCCH + MCCH mechanism used in RRC IDLE and RRC INACTIVE. If there are many RRC CONNECTED UEs receiving the service, using dedicated signaling may be inefficient in terms of DL resource usage. In addition, the BCCH + MCCH approach is already used for LTE eMBMS (for both MBSFN and SC-PTM), and we do not see any significant issue with extending support to NR MBS.

Proposal 1: RAN2 agree that the two-step based approach (i.e. BCCH and MCCH), as adopted by LTE SC-PTM, is reused for the transmission of PTM configuration for NR MBS delivery mode 2, and this can be received in RRC_CONNECTED

At the last two RAN2 meetings (RAN2 #113-e and RAN2 #113bis-e), it was agreed that the BCCH provides the configuration of the MCCH logical channel. The MCCH configured with a modification period and a repetition period. In addition, although there has been no agreement, it is also assumed that the MCCH  provides the MBS configuration information for one or multiple MBS services. These MBS services will have similar QoS requirements and traffic profiles, and may therefore share an MCCH logical channel.  However, not all MBS services will have similar QoS requirements and traffic profiles, and as a result, it may be useful to allow multiple MCCH logical channels, with each MCCH providing MBS configuration information for MBS services that have similar QoS requirements and traffic profiles. A unique MCCH-RNTI may be used to schedule each of the MCCH logical channels.

Proposal 2: RAN2 agree that a cell may be configured with multiple MCCH, each MCCH providing MBS configuration information for one MBS service or a group of MBS services. A unique MCCH-RNTI may be used to schedule each of the MCCH logical channels.

Based on 36.300 [6], for UEs that are not NB-IoT UEs, BL UEs, or UEs in enhanced coverage,  LTE SC-PTM divides changes to SC-MCCH into 2 categories: those signalling an MBMS service Session Start, and those signalling ‘anything’ else. The UE uses a different mechanism for each of these. For signaling a Session Start, the network uses a notification mechanism to announce the changes. The notification is sent in the first subframe in a repetition period where the SC-MCCH can be scheduled. The notification for SC-MCCH change, uses a known SC-N-RNTI (=FFFA). For all other changes, the UE detects changes to SC-MCCH by SC-MCCH monitoring at the modification period.

Observation 3: For LTE SC-PTM, signaling a Session Start is done through a notification mechanism using a SC-N-RNTI, while signaling other MBMS configuration changes is done through SC-MCCH monitoring at the modification periods.

As noted above, it is expected that a cell will support multiple simultaneous MCCH logical channels. A UE will not necessarily be interested in all these MCCH logical channels – rather only those that contain MBS configuration information for MBS services that the UE is interested in. It would be beneficial to the UE, if the notification mechanism provided an indication of which MCCH logical channel is changing and, as a result, needs to be read by the UE. In LTE MBSFN, the notification mechanism relies on an 8-bit bitmap to signal to the UE which of the 8 possible MBSFN area configurations is changing. A similar approach may be used in the NR  MBS notification mechanism, where the 8-bit bitmap is used to signal which of 8 possible MCCH logical channels is changing.

Proposal 3: RAN2 agree that the change notification mechanism provide an indication of which of the MCCH logical channels is changing. This may be provided using an 8-bit bitmap, as used in LTE SC-PTM and LTE MBSFN.

At this point all the triggers that will result in the need to signal an MBS configuration change are still FFS for NR MBS. It is expected that some of these may be delay sensitive and as a result, they likely would also benefit from being signaled through a notification mechanism. Note that this is also in line with the enhancements made in LTE SC-PTM for NB-IoT UEs, BL UEs or UEs in enhanced coverage, where the notification mechanism is used for Session Start as well as to announce changes of SC-MCCH for ongoing services

Proposal 4: RAN2 consider using the notification mechanism to signal a modification in MBS configuration information as well as an MBS service session stop.  In addition, RAN2 also consider additional triggers for sending this notification.

# Conclusion

In this contribution, we discuss the MBS configuration options for RRC IDLE and RRC INACTIVE and make the following observations and conclusions:

Observation 1: NR MBS QoS requirements (especially latency) for different MBS services may be very different

Observation 2: UEs are expected to be able to receive multiple MBS services simultaneously. Each of these services may have its own MBS configuration.

Proposal 1: RAN2 agree that the two-step based approach (i.e. BCCH and MCCH), as adopted by LTE SC-PTM, is reused for the transmission of PTM configuration for NR MBS delivery mode 2, and this can be received in RRC_CONNECTED

Proposal 2: RAN2 agree that a cell may be configured with multiple MCCH, each MCCH providing MBS configuration information for one MBS service or a group of MBS services. A unique MCCH-RNTI may be used to schedule each of the MCCH logical channels.

Observation 3: For LTE SC-PTM, signaling a Session Start is done through a notification mechanism using a SC-N-RNTI, while signaling other MBMS configuration changes is done through SC-MCCH monitoring at the modification periods.

Proposal 3: RAN2 agree that the change notification mechanism provide an indication of which of the MCCH logical channels is changing. This may be provided using an 8-bit bitmap, as used in LTE SC-PTM and LTE MBSFN.

Proposal 4: RAN2 consider using the notification mechanism to signal a modification in MBS configuration information as well as an MBS service session stop.  In addition, RAN2 also consider additional triggers for sending this notification.

# References

- RP-201038, Revised New Work Item on NR Multicast and Broadcast Services

- R2-2100001, Report of 3GPP TSG RAN WG2 meeting #112-e

- R2-2102601, Report of 3GPP TSG RAN WG2 meeting #113-e, Online

- Draft_R2-113bis-e_Meeting_Report

- TS 23.501, System architecture for the 5G System (5GS); Stage 2, V16.7.0

- TS 36.300, Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio  Access Network (E-UTRAN); Overall description; Stage 2, V16.4.0