3GPP TSG-RAN WG2 Meeting #114-e	     R2-2106451(Revision of R2-2105174)

Online, May 19th - May 27th, 2021

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on the common configuration in the dedicated signaling

Agenda item:		5.4.3

Document for: 	   Discussion and Decision

- Introduction

In the last meeting, whether the common configuration in the dedicated signaling shall refer to the UE capability was discussed [1] but postponed. In this paper, we share our understanding on this issue.

- Discussion

In this chapter, we first compare the common configuration in the dedicated signaling with that in the system Information, then discuss the common configuration that only included in the dedicated signaling.

System Information vs Common configuration in the dedicated signaling

As shown in the annex, we can see that, the common configuration in the dedicated signalling includes 2 parts:

- BWP Common configuration for the non-initial BWP

- ServingCellConfigCommon

BWP Common configuration of the non-initial BWP in the dedicated signaling

For the BWP Common configuration for the non-initial BWP, the same signalling structure as the initial BWP in the system information was adopted.

Observation 1: For the BWP Common configuration for the non-initial BWP, the same signalling structure as the initial BWP in the system information was adopted.

Based on the observation 1 and considering that the system information setting will not take the UE capability into consideration, for the BWP Common configuration for the non-initial BWP in the dedicated signalling, there is also no need to take the UE capability into consideration.

Proposal 1: There is no need to take the UE capability into consideration for the BWP Common Configuration in the dedicated signalling.

ServingCellConfigCommon in the dedicated signaling

For the ServingCellConfigCommon, from the annex we can see that except the lte-CRS-ToMatchAround/ rateMatchPatternToAddModList, all of other elements are also included in the system Information.

Observation 2: For the ServingCellConfigCommon, except the lte-CRS-ToMatchAround /rateMatchPatternToAddModList, all of other elements are also included in the system Information.

About the lte-CRS-ToMatchAround/ rateMatchPatternToAddModList, the related UE capabilities are listed as below:

| rateMatchingLTE-CRS Indicates whether the UE supports receiving PDSCH with resource mapping that excludes the REs determined by the higher layer configuration LTE-carrier configuring common RS, as specified in TS 38.214 [12]. | Band | Yes | N/A | N/A |
| --- | --- | --- | --- | --- |
| rateMatchingResrcSetSemi-Static Indicates whether the UE supports receiving PDSCH with resource mapping that excludes the REs corresponding to resource sets configured with RB-symbol level granularity indicated by bitmaps and cTntrolResourceSet (see patternType in RateMatchPattern in TS 38.331[9]) following the semi-static configuration as specified in TS 38.214 [12]. | UE | Yes | No | No |

Now we need to analyze whether the network need to take the above two UE capabilities into consideration when configure lte-CRS-ToMatchAround/ rateMatchPatternToAddModList. Let’s go back to the discussion history of these two elements, which were introduced based on [2]. As analyzed in [2]: the main usage of lte-CRS-ToMatchAround /rateMatchPatternToAddModList is LTE/NR co-existence.

“lte-CRS-ToMatchAround is used for the UE to learn where LTE CRS is present for the rate matching purpose. A practical use case for the LTE CRS rate matching is LTE/NR co-existence where LTE and NR is operated on the same carrier. Likewise, rateMatchPattern is used for the future usage in NR. Given that the main usage is co-existence with LTE and future usage/service in NR, PDSCH rate matching would be needed not only for the dedicated signalling to the UE but also for broadcast signalling delivered on PDSCH, i.e. Other SI (OSI). In that sense, a cell-level PDSCH config. needs to be introduced, which are to be broadcast.”

Note: By checking the Asn.1 review history, we can find that the original intention was to add the lte-CRS-ToMatchAround /rateMatchPatternToAddModList to the system Information, however, considering the signaling overhead of the SIB1, at last these two elements were only included in the common configuration of the dedicated signalling.

Observation 3: The main usage of lte-CRS-ToMatchAround /rateMatchPatternToAddModList is LTE/NR co-existence.

Based on the observation 3, for the UE that doesn’t support rateMatchingLTE-CRS/ rateMatchingResrcSetSemi-Static, even the network doesn’t configure lte-CRS-ToMatchAround /rateMatchPatternToAddModList, it still can’t change the truth that some REs/Symbols have been occupied for the LTE/NR co-existence case. Thus there is no motivation/benefit to match the UE capability when configure lte-CRS-ToMatchAround/ rateMatchPatternToAddModList.

Proposal 2: For the lte-CRS-ToMatchAround/rateMatchPatternToAddModList configuration in the ServingCellConfigCommon, no need to match the UE capability (i.e. rateMatchingLTE-CRS /rateMatchingResrcSetSemi-Static).

Besides the lte-CRS-ToMatchAround/rateMatchPatternToAddModList, the other configurations in the ServingCellConfigCommon are also included in the system information. Obviously, these configuration shall be consistent with that in the system information unless explicitly specified otherwise (e.g.controlResourceSetZero   and searchSpaceZero in the downlinkConfigCommon, which can be configured in ServingCellConfigCommon even if MIB indicates that they are absent.)

Proposal 3: Common configuration in dedicated signalling and in broadcast information should be consistent unless explicitly specified otherwise (e.g.controlResourceSetZero   and searchSpaceZero  in the downlinkConfigCommon,  which can be  configured in ServingCellConfigCommon  even if MIB indicates that they are absent.)

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: For the BWP Common configuration for the non-initial BWP, the same signalling structure as the initial BWP in the system information was adopted.

Proposal 1: There is no need to take the UE capability into consideration for the BWP Common Configuration in the dedicated signalling.

Observation 2: For the ServingCellConfigCommon, except the lte-CRS-ToMatchAround /rateMatchPatternToAddModList, all of other elements are also included in the system Information.

Observation 3: The main usage of lte-CRS-ToMatchAround /rateMatchPatternToAddModList is LTE/NR co-existence.

Proposal 2: For the lte-CRS-ToMatchAround/rateMatchPatternToAddModList configuration in the ServingCellConfigCommon, no need to match the UE capability (i.e. rateMatchingLTE-CRS /rateMatchingResrcSetSemi-Static).

Proposal 3: Common configuration in dedicated signalling and in broadcast information should be consistent unless explicitly specified otherwise (e.g.controlResourceSetZero   and searchSpaceZero  in the downlinkConfigCommon,  which can be  configured in ServingCellConfigCommon  even if MIB indicates that they are absent.)

- Reference

- R2-2103799	Configuration of common fields in dedicated signalling	Ericsson	discussion	Rel-15	NR_newRAT-Core

- R2-1803722 [D313] Necessity of LTE CRS rate matching for Other SI  NTT DOCOMO

- Annex: System Information vs Common configuration in the dedicated signaling

| System Information | Common configuration in the dedicated signaling |  |
| --- | --- | --- |
| ServingCellConfigCommonSIB | ServingCellConfigCommon | BWP common configuration |
|  | physCellId |  |
| downlinkConfigCommonSIB(frequencyInfoDL,BWP-DownlinkCommon bcch-Config pcch-Config ) | downlinkConfigCommon(FrequencyInfoDL,BWP-DownlinkCommon ) | ServingCellConfig->BWP-Downlink->BWP-DownlinkCommon |
| uplinkConfigCommon (FrequencyInfoUL BWP-UplinkCommon timeAlignmentTimerCommon | uplinkConfigCommon (FrequencyInfoUL BWP-UplinkCommon | ServingCellConfig->->BWP-Uplink->BWP-UplinkCommon |
| supplementaryUplink n-TimingAdvanceOffset ssb-PositionsInBurst:inOneGroup/ groupPresence ssb-PeriodicityServingCell tdd-UL-DL-ConfigurationCommon ss-PBCH-BlockPower channelAccessMode-r16: dynamic/semiStatic discoveryBurstWindowLength-r16 highSpeedConfig-r16 MIB:subCarrierSpacingCommon MIB:dmrs-TypeA-Position | supplementaryUplinkConfig n-TimingAdvanceOffset ssb-PositionsInBurst: shortBitmap/ mediumBitmap/longBitmap ssb-periodicityServingCell tdd-UL-DL-ConfigurationCommon ss-PBCH-BlockPower channelAccessMode-r16 : dynamic/ semiStatic discoveryBurstWindowLength-r16 highSpeedConfig-r16 ssbSubcarrierSpacing dmrs-TypeA-Position |  |
|  | lte-CRS-ToMatchAround rateMatchPatternToAddModList rateMatchPatternToReleaseList |  |