3GPP TSG-RAN2#114e		   	 					 				  									R2-2106452

Electronic meeting, May 19 – 27

Source: 			ZTE Corporation

Title: 	SDT RACH resource configuration and coordination with other WIs (REDCAP, Coverage-ext, RAN-Slicing)

Agenda item:	8.18

Document for: Discussion and Decision

# Introduction

In Rel-17, a number of WIs will likely need RACH resource partitioning. As of now, these WIs are:

- SDT

  - To identify SDT cause and provide larger MSG3/MSGA payload size

- RAN slicing

  - For RA resource prioritisation and slice isolation

- REDCAP

  - For network to know the REDCAP UE at MSG1 level

- Coverage extension

  - To identify the coverage extension UE at MSG1 level and to use coverage extension mechanisms from MSG3 onwards

In this contribution we discuss the cross WI and cross WG aspects of configuring separate RACH resources for the above use cases

# Feature combinations

Until Rel-16, MSG1/MSGA preamble indicates the following information:

Rel-16 Info set

- Selected SSB

- CFRA or CBRA cause

- Payload size (preamble group B configured or not)

- Random access type (2-step vs 4-step RA)

The above indication can be conveyed by either the selected Random access Occasion (RO) or the selected preamble or a combination of the RO + preamble.

With Rel-17 the RO or Preamble or RO + Preamble combination will need to indicate the following in addition:

Rel-17 Info set

- SDT cause

  - This is a binary indication (i.e. the access is either for SDT or for non-SDT)

- RAN slicing

  - The selected RAN slice has be indicated

  - The number of RAN slices available may be configured

  - It is not yet clear whether there is a one-to-one mapping between the RAN slices and RACH resource partitioning or not (i.e. can more than one slice use the same RACH partition or does each slice need a different RACH partition)

  - In the worst case, if there are N slices, we need N partitions

- REDCAP

  - Assuming there are no sub-categories for the REDCAP UES, this is also likely to be a binary indication (i.e., the UE either is a REDCAP UE or not)

- Coverage extension

  - Depending on number of coverage extension levels to be indicated, this may be either a binary indication or an indication of more than two levels

Given the above situation, we need a holistic analysis of RACH resource configuration to accommodate all the above use cases.

The first question is whether all the ROs should satisfy all the above use cases. The ROs provide random access resources (i.e. preambles) and are generally evenly spread in time/frequency domain to cater for a required random access latency. Some services may be time critical and may require a more frequent RO opportunity whilst the other services may be tolerant to a sparser resource configuration as these are more delay tolerant. Ideally, if all ROs can accommodate all the above information, then there would be no issue, however, we should keep in mind that there is only a limited preamble space (64 preambles in general) and even in the simplistic example below, it is clear that not all ROs can accommodate all the use cases.

e.g: Number of SSBs (2) x group B configured (2) x 2-step and 4-step RA (2) x SDT allowed (2) x #Slices (2) x REDCAP (2) x Coverage ext (2) = 128!

Observation 1: In order to cater for the RA resources, some services will not be available on some of the ROs

Then, we can look into detail on which combination of the services make sense from logical perspective. It can be assumed that the features until Rel-16 are the baseline and are likely needed to be supported possibly on all ROs (note this doesn’t necessarily mean that they will have to be configured on all ROs, but the signalling should support combination of them on all ROs). With respect to Rel-17 features, this is not yet completely clear. The following table shows some possible considerations that need to be discussed:

| Analysis of feature combinations for Rel-17 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | SDT | Redcap | coverage ext | slice indication |
| SDT | X | Needed Devices such as wearables will benefit from reduced signalling overhead/power saving for small data packets in INACTIVE | Not needed If the device is in coverage limited scenario, SDT is not a useful mechanism | Needed Some slices may benefit from SDT whilst others may not. Case by case analysis is needed and final configuration may be left to network implementation |
| Redcap | Needed Devices such as wearables will benefit from reduced signalling overhead/power saving for small data packets in INACTIVE | X | Needed REDCAP devices need coverage extension to compensate for the coverage loss due to capability reduction | Needed Some slices may allow REDCAP whilst others may not. Case by case analysis is needed and final configuration may be left to network implementation |
| coverage ext * | Not needed If the device is in coverage limited scenario, SDT is not a useful mechanism | Needed REDCAP devices need coverage extension to compensate for the coverage loss due to capability reduction | X | Needed Some slices may benefit from coverage extension whilst others may not. Case by case analysis is needed and final configuration may be left to network implementation |
| slice indication | Needed Some slices may benefit from SDT whilst others may not. Case by case analysis is needed and final configuration may be left to network implementation | Needed Some slices may allow REDCAP whilst others may not. Case by case analysis is needed and final configuration may be left to network implementation | Needed Some slices may benefit from coverage extension whilst others may not. Case by case analysis is needed and final configuration may be left to network implementation | X |

* Note: In addition to the above, it is not necessary to support extended coverage with 2-step RACH (2-step RACH already has an RSRP threshold to ensure this, but perhaps we could preclude this combination regardless of the RSRP threshold).

Based on the above the following proposals are made:

Proposal 1: On a given RO, only the following combination of Rel-17 features will be supported (i.e., signalling can support these combinations in addition to the Rel-16 features)

- SDT with: REDCAP and/or Slice Indication (for some slices – up to network implementation)

- REDCAP with: Coverage extension and/or Slice Indication (for some slices – up to network implementation)

Note 1: combination of SDT with coverage extension is not supported

Note 2: combination of coverage extension with 2-step RACH is also not supported

Proposal 2: RO-Mask can be defined for Rel-17 features to enable configuring a subset of ROs for a given feature

# Resource configuration

Until Rel-16, the signalling framework shown in Figure 1 is used to indicate the preamble partitions within a shared RO. For indicating the allowed combinations of the Rel-17 features, the preamble space should be obtained from the reserved preambles for each SSB (i.e., the green highlighted part in Figure 1). The allowed combinations of Rel-17 features should be configured within this preamble space and the detailed signalling framework to enable this can be further determined in RAN2.

Proposal 3: Within a given RO, the allowed combination of Rel-17 features (see proposal 1) should be accommodated within the preamble space reserved for other purpose for each SSB and the details of the signalling for this should be further worked out in RAN2

Figure 1: Signalling framework for preamble partitioning in Rel-17

# Aspects related to RACH procedure

Until Rel-16 some fallback procedures are allowed during the RACH procedure. For instance, if 2-step RA is initiated, the UE is allowed to fallback to 4-step RA procedure if the 2-step RA procedure fails a configured number of times. It should be noted that the fallbacks are necessary if the RACH resource configured for a certain feature has a lower performance compared to other RACH resources. This mainly depends on the collision probability on a given RACH resource. However, a good network implementation should generally cater for a low and predictable collision probability on RACH resources across all the features. Thus, assuming this can be guaranteed, the fallbacks from one RA resource to another are not really that critical. On the other hand, supporting fallbacks (i.e., switching between features after RACH has been initiated) comes with significant complexity in MAC layer. Thus, in order to simplify the overall design, a general design principle would be to minimise or preferably to avoid fallbacks from one RACH resource to other. Keeping this in mind, the following general principles are proposed:

Proposal 4: The RA procedure design for Rel-17 should adhere to the following general principles:

- Carrier selection (between NUL/SUL) should happen ahead of the initial RACH resource selection

  - The RSRP threshold for carrier selection may be configured per each feature

- Initial RACH resource should be selected based on the selected carrier for the selected feature combination (i.e., selected slice, SDT or not, REDCAP or not etc)

- As a general rule, all RACH retransmissions (if any are needed, until RACH failure happens) shall be performed over the same RACH resources (and same carrier – NUL/SUL) as the one selected for initial RACH resource

  - The only exception to the above is the already allowed fallbacks until Rel-16 (i.e., fallback from CFRA to CBRA and fallback from 2-step RA to 4-step RA) and network implementation should ensure that the feature is supported on the new RA resources after such fallback

# Conclusions and proposals

Proposal 1: On a given RO, only the following combination of Rel-17 features will be supported (i.e., signalling can support these combinations in addition to the Rel-16 features)

- SDT with: REDCAP and/or Slice Indication (for some slices – up to network implementation)

- REDCAP with: Coverage extension and/or Slice Indication (for some slices – up to network implementation)

Note 1: combination of SDT with coverage extension is not supported

Note 2: combination of coverage extension with 2-step RACH is also not supported

Proposal 2: RO-Mask can be defined for Rel-17 features to enable configuring a subset of ROs for a given feature

Proposal 3: Within a given RO, the allowed combination of Rel-17 features (see proposal 1) should be accommodated within the preamble space reserved for other purpose for each SSB and the details of the signalling for this should be further worked out in RAN2

Proposal 4: The RA procedure design for Rel-17 should adhere to the following general principles:

- Carrier selection (between NUL/SUL) should happen ahead of the initial RACH resource selection

  - The RSRP threshold for carrier selection may be configured per each feature

- Initial RACH resource should be selected based on the selected carrier for the selected feature combination (i.e., selected slice, SDT or not, REDCAP or not etc)

- As a general rule, all RACH retransmissions (if any are needed, until RACH failure happens) shall be performed over the same RACH resources (and same carrier – NUL/SUL) as the one selected for initial RACH resource

  - The only exception to the above is the already allowed fallbacks until Rel-16 (i.e., fallback from CFRA to CBRA and fallback from 2-step RA to 4-step RA) and network implementation should ensure that the feature is supported on the new RA resources after such fallback

# References