3GPP TSG-RAN WG2 Meeting #114........................................................... R2-2106554

# Electronic Meeting, May 19 – May 27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0342 | rev | 5 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.300 Running CR for MBS in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2021-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR introduces the enhancements specified on support of MBS in NR |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of specific MBS, architecture, session management, protocol design, PTM/PTP dynamic switch, and service continuity aspects |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR MBS is not supported in NR |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

3	Abbreviations and Definitions

3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CPC	Conditional PSCell Change

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell-ID (positioning method)

EHC	Ethernet Header Compression

ETWS	Earthquake and Tsunami Warning System

GFBR	Guaranteed Flow Bit Rate

G-RNTI	Group RNTI

G-CS-RNTI	Group Configured Scheduling RNTI

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

LDPC	Low Density Parity Check

MBS	Multicast/ Broadcast Services

MCCH	MBS Control Channel

MDBV	Maximum Data Burst Volume

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MT	Mobile Termination

MTCH	MBS Traffic Channel

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

NB-IoT	Narrow Band Internet of Things

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

P-MPR	Power Management Maximum Power Reduction

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PS-RNTI	Power Saving RNTI

PSS	Primary Synchronisation Signal

PTM	Point-to-Multipoint

PTP 	Point-to-Point

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SFI-RNTI	Slot Format Indication RNTI

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TPC	Transmit Power Control

TRP	Transmit/Receive Point

UCI	Uplink Control Information

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

Next Modified Subclause (new)

16.x	Multicast and Broadcast Services

16.x.1	General

Editor’s Note: General aspects to be covered here.

NR system enables resource efficient delivery of multicast/broadcast services (MBS).

For broadcast communication service, the same service and the same specific content data are provided simultaneously to all UEs in a geographical area (i.e., all UEs in the MBS service area are authorized to receive the data). A broadcast communication service is delivered to the UEs using a broadcast session. A UE can receive a broadcast communication service in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state.

For multicast communication service, the same service and the same specific content data are provided simultaneously to a dedicated set of UEs (i.e., not all UEs in the MBS service area are authorized to receive the data). A multicast communication service is delivered to the UEs using a multicast session. A UE can receive a multicast communication service in RRC_CONNECTED state with mechanisms such as PTP and/or PTM delivery, as defined in section 16.x.5.4. HARQ feedback/retransmission can be applied to both PTP and PTM transmission.

Editor’s Note: FFS is the term “MBS service area”, if the term is kept in the normative definitions defined in the SA2 TS 23.xxx.

Editor’s Note: These definitions come from TR 23.757, but should eventually be replaced with normative definitions defined in the SA2 TS 23.xxx.

16.x.2	Network Architecture

Editor’s Note: RAN3 to provide architecture aspects here.

16.x.3	Protocol Architecture

Editor’s Note: User plane and control plane protocol architecture to be covered here.

Figure 16.x.3-1and 16.x.3-2 depict the Downlink Layer 2 architecture for multicast session and broadcast session respectively, where MBS protocol stack comprises the same layer 2 sublayers as described in section 6 with the following differences:

- SDAP sublayer provides only the following functionalities:

- Mapping between a MBS QoS flow and a MRB;

- Transfer of user plane data.

- PDCP sublayer provides only the following functionalities:

- Transfer of data;

- Maintenance of PDCP SNs;

- Header compression and decompression using the ROHC protocol;

- Reordering and in-order delivery;

- Duplicate discarding.

- For multicast session, gNB provides the following one or more MRB configuration(s) to the UE:

- MRB with one RLC-UM or RLC-AM entity for PTP transmission;

- MRB with one RLC-UM entity for PTM transmission;

- MRB with two RLC-UM entities, one RLC-UM entity for PTP transmission and the other RLC-UM entity for PTM transmission as described in section 16.x.5.4;

- MRB with two RLC-UM entities, one RLC-AM entity for PTP transmission and the other RLC-UM entity for PTM transmission as described in section 16.x.5.4;

Editor’s Note: Whether to support security in PDCP requires progress and input from other WG, i.e. SA3.

Figure 16.x.3-1: Downlink Layer 2 Architecture for Multicast Session

- For broadcast session, gNB provides the following one or more MRB configuration(s) to the UE:

- MRB with one RLC-UM entity for PTM transmission;

Figure 16.x.3-2: Downlink Layer 2 Architecture for Broadcast Session

16.x.4	Group Scheduling

Editor’s Note: Group scheduling related aspects to be covered here.

The following logical channels are used in MBS delivey:

- MTCH: A point-to-multipoint downlink channel for transmitting MBS data of either multicast session or broadcast session from the network to the UE ;

- DTCH: A point-to-point channel defined in section 6.2.2 for transmitting traffic data of multicast ;

- MCCH: A point-to-multipoint downlink channel used for transmitting MBS control information for broadcast session from the network to the UE, for one or several MTCH(s);

The following connections between logical channels and transport channels for group transmission exist:

- MCCH can be mapped to DL-SCH;

- MTCH can be mapped to DL-SCH;

The following decipts the usage of RNTI for group transmission:

- There is a one-to-one mapping between MBS Session and G-RNTI used for the reception of the DL-SCH to which a MTCH is mapped;

- There is a one-to-one mapping between MBS Session and G-CS-RNTI used for the reception of the DL-SCH to which a MTCH is mapped;

- A UE can receive different services using different G-RNTIs/G-CS-RNTIs;

- Different logical channels associated with the same G-RNTI may be multiplexed together ;

- Different logical channels associated with the same C-RNTI may be multiplexed together.

Editor’s Note: FFS if there is a need to have specific LCID spaces for the MTCH/MCCH and whether the LCID space for MTCH and DTCH is shared or separate.

Editor’s Note: FFS if Multiplexing/de-multiplexing of different logical channels associated with the same G-CS-RNTI is supported for NR MBS.

Editor’s Note: FFS whether for PTM transmission of NR MBS, DRX scheme is independent of DRX for unicast transmission, e.g. supported on a per G-RNTI basis

Editor’s Note: FFS whether for PTP transmission, DRX operation for unicast transmission is reused.

Editor’s Note: FFS whether multiple G-RNTIs/G-CS-RNTIs are supported depends on UE capability.

Editor’s Note: FFS other mappings between MBS session and G-RNTI.

16.x.5	Multicast Handling

16.x.5.1	Session Management

Editor’s Note: RAN3 to provide Session management aspects here.

16.x.5.2	Configuration

Editor’s Note: FFS how multicast configuration is provided for supporting multicast reception in RRC_CONNECTED state.

If the UE which joined the multicast session is in RRC_CONNECTED state, the gNB sends RRC Reconfiguration message with relevant MBS configuration for multicast session to the UE and there is no need for separate session activation notification for this UE.

MBS supporting gNBs notify the UEs in RRC IDLE/INACTIVE state about a multicast session activation using a group notification mechanism. The group notification is addressed with P-RNTI on PDCCH, and the paging channels are monitored by the UE as described in section 9.2.5. And each UE is not paged individually, and the same group notification identity, i.e., MBS session ID, is used for UEs with the same multicast session in both RRC IDLE and RRC INACTIVE states.

gNBs not supporting MBS may notify the UEs in RRC IDLE/INACTIVE state about a multicast session activation through Paging messages in the PO as described in section 9.2.5, where each UE is paged individually.

16.x.5.3	Service Continuity

Editor’s Note: Mobility related aspects to be covered here.

16.x.5.3.1 Handover between Multicast supporting cells

Mobility procedures for multicast reception allow the UE to continue receiving multicastservice(s) via PTM or PTP during handover.

In order to support lossless handover for multicast service, DL PDCP SN synchronization and continuity between the source cell supporting multicast and the target cell supporting multicast needs to be guaranteed. The source gNB may forward the data to the target gNB and the target gNB will deliver the forwarded data. Additionally, the UE may be configured by the network to provide PDCP status report for a MRB for multicast session during a handover.

Editor’s Note: FFS which detailed scenario but at least PTP-PTP.

Editor’s Note: a procedure flow for mobility will be provided in the future.

16.x.5.3.2 Handover between Multicast supporting cell and Mulicast non-supporting cells

Editor’s Note: Handover between multicast supporting cell and multicast non-supporting cells related aspects to be covered here.

Mobility from the multicast supporting cell to mulicast non-supporting cells can be achieved by switching the traffic from delivery via MRB to delivery via DRB.

Editor’s note: FFS whether the switching the traffic from delivery via MRB to delivery via DRB either before or during the handover.

Editor’s note: Whether and how this can be done without data losses has to be further investigated and requires progress and input from other WGs, i.e. RAN3 and SA2.

16.x.5.4	PTP/PTM Dynamic Switch

Editor’s Note: Dynamic switch related aspects to be covered here.

For multicast service, gNB may deliver MBS data packets using the following methods:

- PTP Transmission: gNB individually delivers separate copies of MBS data packets to each UEs independently, i.e. gNB uses UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule UE-specific PDSCH which is scrambled with the same UE-specific RNTI.

- PTM Transmission: gNB delivers a single copy of MBS data packets to a set of UEs, e.g., gNB uses group-common PDCCH with CRC scrambled by group-common RNTI to schedule group-common PDSCH which is scrambled with the same group-common RNTI.

Editor’s Note: FFS that RAN1 inputs are needed for the definition of PTP/PTM transmission.

If a UE is configured with a split MRB, a gNB dynamically decides whether to deliver multicast data by PTM or PTP  for a given UE based on the protocol stack defined in section16.x.3.

For a split MRB, the usage of the PTP leg cannot be deactivated, i.e. the UE needs to always monitor C-RNTI and the state of RLC entity for PTP delivery is always active, after the necessary split MRB configuration.

Editor’s Note: When two RLC entities are configured for a MRB for PTP delivery and PTM delivery respectively by RRC, it is FFS whether the state of RLC entity for PTM delivery can be active or deactive and can be dynamically controlled.

16.x.5.5	Reliability

Editor’s Note: Reliability related aspects to be covered here.

16.x.6	Broadcast Handling

16.x.6.1	Session Management

Editor’s Note: RAN3 to provide Session management aspects here.

16.x.6.2	Configuration

The UE can receive the MBS configuration for broadcast session in RRC_IDLE , RRC_INACTIVE and RRC_CONNECTED state via MCCH.. BCCH delivers the parameters needed for the reception of MCCH in the MBS specific SIB, which in turns delivers parameters needed for MTCH reception.

The following principles govern the MCCH structure:

- The MCCH provides the list of all broadcast services with ongoing sessions transmitted on MTCH(s) and the associated information for broadcast session, including MBS session ID , associated G-RNTI and scheduling information for MTCH;	Each MCCH content is transmitted within periodically occurring time domain windows, referred to as MCCH transmission window defined by MCCH repetition period, MCCH window duration and radio frame/slot offset;

- MCCH uses a modification period and MCCH contents are only allowed to be modified at each modification period boundary;A notification mechanism is used to announce the change of MCCH contents due to broadcast session start, modification and stop;

- When the UE receives a notification, it acquires the updated MCCH in the same MCCH modification period where the change notification is sent;

- The UE in RRC_IDLE or RRC_INACTIVE receives both MCCH channel and SI/Paging without BWP switch in case of BA.

Editor’s Note: FFS that RAN1 inputs are needed for the definition of MBS control information on the MCCH acquisition, e.g. the definition of common search space for MCCH scheduling, and the details of the configuration of the bandwidth for MCCH reception.

Editor’s Note: FFS that RAN1 inputs are needed for to decide about the RNTI and DCI format used for MCCH change notifications.

Editor’s note: FFS on whether this notification can be reused for modification of other information carried by MCCH, if any.

Editor’s note: FFS whether the possibility of UE missing an MCCH change notification needs to be addressed or can be left to UE implementation.

16.x.6.3 Service Continuity

Editor’s Note: Mobility related aspects to be covered here.

Editor’s Note: a procedure flow for mobility will be provided in the future.

The gNB may provide the UE with information, enabling service continuity for broadcast session.

Editor’s Note: FFS the detailed information, e.g. USD , SAI/TMGI etc.

The UE in RRC_CONNECTED state may send MBS Interest Indication to the gNB for broadcast session.

Editor’s note: It still needs to be discussed what the UE indicates in MBS Interest Indication, e.g. UE interested frequency , service etc.

Annex	- collection of RAN2 agreements on NR MBS WI

Cyan highlight – agreements captured in stage-2 specifications

Green highlight – stage-3 level agreements, not captured in stage-2 specifications

No highlight – agreements with no direct impact on specifications or agreements are not mature to be captured in the specification

RAN2#111-E agreements

Focus initially on NR SA, TBD to what extent other scenarios NR DC, NE DC can be supported.

Confirm Will support PTM transmission in a cell.

Confirm that We will, for multicast services introduce support for PTP and PTM transmission of shared traffic delivered by 5GC, at least for connected mode (this is not intended to exclude other cases)

For a UE, gNB dynamically decides whether to deliver multicast data by PTM or PTP (Shared delivery)

FFS which layer(s) handles reliability (in general), in order delivery / duplicate handling, and it is FFS how it works at PTM PTP switch.

Focus on MBS-MBS scenario initially (i.e. shared delivery), including both PTM and PTP (if applicable). Other scenarios later, TBD.

Requirements for lossless mobility are TBD. Assume for now that R2 will anyway discuss service continuity functionality for low or no data loss.

R2 assumes that for Rel-17 NR multicast Mobility in Connected mode, handover (including variants) is the baseline, TBD exactly which variants.

R2 expect that there may be HARQ with feedback (for PTM) and this is specified by R1.

RAN2#112-e agreements

Broadcast and multicast sessions support, RRC states and other aspects related to SA2 LS

For Rel-17, R2 specifies two modes:

1: One delivery mode for high QoS (reliability, latency) requirement, to be available in CONNECTED (possibly the UE can switch to other states when there is no data reception TBD)

2: One delivery mode for “low” QoS requirement, where the UE can also receive data in INACTIVE/IDLE (details TBD).

R2 assumes (for R17) that delivery mode 1 is used only for multicast sessions.

R2 assumes that delivery mode 2 is used for broadcast sessions.

The applicability of delivery mode 2 to multicast sessions is FFS.

No data: When there is no data ongoing for the multicast session, the UE can stay in RRC_CONNECTED. Other cases FFS

It is up to SA2 to decide whether the multicast session activation/deactivation mechanism is supported or not, and RAN2 will discuss if there is any RAN2 impacts based on SA2 inputs.

It is up to SA2 to decide on the support of local MBS service, and RAN2 will discuss the RAN2 impacts based on SA2 inputs.

In general, Information of MBS services/groups subscribed by the UE (e.g. TMGI) and QOS requirements of a MBS service should be provided to RAN. Detail information e.g. for PTM PTP switch if any is FFS.

Layer 2 architecture

The function of mapping from QoS flows to MBS RBs in SDAP is needed for NR MBS. TBD whether any SDAP header is needed.

(Working assumption) no SDAP functions other than “mapping from QoS flows to radio bearers” and “transfer of user plane data” are supported for MBS. FFS whether to support QoS flows to radio bearers remapping.

In general: RAN2 wait for SA3’s progress for discussing security issues. TBD whether we need to send LS to SA3.

RoHC (at least U-mode) can be configured for NR MBS bearers. This is applicable for Mcast, assume this is applicable also to broadcast.

RoHC is located at PDCP.

The reordering and in-order delivery function in PDCP is supported for NR MBS.

The following PDCP functions are also supported for NR MBS: transfer of data; maintenance of PDCP SNs; duplicate discarding. Other PDCP functions are FFS.

RLC AM is supported for PTP transmission of NR MBS.

RLC UM is supported for PTP transmission of NR MBS.

RLC UM is supported for PTM transmission of NR MBS.

RLC TM is not supported for PTP transmission of NR MBS.

RLC TM is not supported for PTM transmission of NR MBS.

FFS for PTM if multiplexing/de-multiplexing of different logical channels are to be supported in MAC for NR MBS.

Working assumption: RLC-AM for PTM is not supported (can be revisited but it means that proponents of RLC-AM for PTM need to demonstrate the need, to change this).

Service continuity

R2 aim to support lossless handover for MBS-MBS mobility for service that requires this (TBD which detailed scenario but at least PTP-PTP)

In order to support the lossless handover for 5G MBS services, at least DL PDCP SN synchronization and continuity between the source cell and the target cell should be guaranteed by the network side to realize. The design of specific approach to realize this can be involved with WG RAN3.

From network side, the source gNB may forward the data to the target gNB and the target gNB will deliver the forwarding data. Meanwhile, the SN STATUS TRANSFER should be extended to cover the PDCP SN for MBS data; Then (TBD after or in parallel) the UE receives the MBS in the target cell by the target cell according to target configuration.

From UE side, PDCP status report may be supported as well.

Idle/Inactive support

UE receives the MBS configuration (for broadcast/delivery mode 2) by BCCH and/or MCCH (TBD), and this can be received in Idle / Inactive mode. Connected mode FFS (dep on UE cap and where service is provided etc). A notification mechanism is used to announce the change of MBS Control information.

RAN2#113-e agreements

Reply LS on 5MBS progress and issues to address

[037] RAN2 assumes that MBS session join/leave indications are sent using NAS signalling regardless of the RRC state the UE is in. 5GC should inform RAN about the UE leaving the MBS session.

[037] If the UE which joined the multicast session is in RR CONNECTED state when the session is started, the gNB sends RRC Reconfiguration message with relevant MBS configuration to the UE and there is no need for separate session start notification for this UE. FFS for session activation.

[037] RAN2 assumes that from RAN2 perspective, mobility from the source gNB supporting MBS to target gNB not supporting MBS can be achieved by switching the traffic from delivery via MRB to delivery via DRB either before or during the handover. Whether and how this can be done without data losses has to be further investigated and requires progress and input from other WGs, i.e. RAN3 and SA2.

[037] RAN2 will not provide further reply to SA2 on assistance information from CN to RAN on PTP/PTM delivery method decision and switching.

[037] RAN2 will reply that it will wait for SA3 to finalize their study on security for MBS before discussing security aspects in RAN2.

[037] Request a clarification from SA2 on whether and what the difference is between session start and session activation and between the session stop and session deactivation.

[037] RAN2 will not address the note on 5GC Shared MBS delivery in the reply LS to SA2.

[037] Reply to SA2/SA4 that:

- SYNC protocol is not supported in the specifications in Rel-17

- RAN2 has agreed that ROHC is to be located in RAN

Reliability and UP architecture

Confirm P1 P2 P3 (assume that MRB may include both PTP and PTM)

For the case that both PTM and PTP are RLC-UM, configuration with No L2 ARQ and with PDCP anchored PTM – PTP switching shall be supported (e.g. for services that would typically be configured with RLC UM for unicast).

Idle and Inactive mode Ues

Both idle/inactive UEs and connected mode UEs can receive MBS services transmitted by NR MBS delivery mode 2 (Broadcast service as already agreed, TBD other). The ability for connected mode UEs to receive this may depend on the network provisioning of the service (e.g. which freq), UE connected mode configuration and UE capabilities.

The two-step based approach (i.e. BCCH and MCCH) as adopted by LTE SC-PTM is reused for the transmission of PTM configuration for NR MBS delivery mode 2.

Assume it is possible to reuse LTE SC-PTM mechanism for the CONNECTED UEs to receive the PTM configuration for NR MBS delivery mode 2, i.e. broadcast based manner.

Assume that MCCH change notification mechanism is used to notify the changes of MCCH configuration due to session start for delivery mode 2 of NR MBS (other cases FFS, if any).

Assume that MBS Interest Indication is supported for UEs in connected mode for Broadcast service (assume that as usual there is no mandatory network requirement, network action is up to network).

MBS Interest Indication is NOT supported for UEs in idle/inactive mode for NR MBS delivery mode 2.

Assume that some information for purpose of service continuity can be provided for NR MBS delivery mode 2. (FFS what - need to be revisited, e.g. based on progress in other groups, e.g. USD, SAI/TMGI etc)

FFS whether support UE awareness of MBS services on frequency basis for service continuity for NR MBS delivery mode 2 (i.e. Reuse LTE SC-PTM mechanism).

FFS Support frequency prioritization during cell reselection for service continuity for NR MBS delivery mode 2 (i.e. Reuse LTE SC-PTM mechanism).

P2: Whether UEs that receive Multicast can be released to RRC Inactive / Idle and continue receiving Multicast is Postponed. Should limit to RRC inactive in future discussions

RAN2#113bis-e agreements

Session activation

There is Support to have group notification for multicast for MBS supporting nodes (e.g. paging)

Support group notification for multicast for MBS supporting nodes

For delivery mode 1 UE is not expected to monitor Group notification channel in RRC_CONNECTED

It is FFS whether RAN2 needs to handle PRACH capacity issues due to group notifications

Use same group notification identity for both RRC_IDLE and RRC_INACTIVE states

For the reply LS

For non-supporting nodes, using MBS session ID will not work as it would impact non-MBS nodes. Unicast paging would work.

For supporting nodes, using MBS session ID is feasible.

Connected mode UEs

Reliability

For a given UE, if the MRB’s QoS requirements are not met via PTM, switching to PTP with RLC-AM shall be supported.

Dynamic PTM PTP switch and service continuity

- Chair: NOTE that the below agreements are only based on architecture decisions so far. The reliability discussion not concluded yet i.e. other cases than RLC UM + RLC UM. PTM PTP switch for such other cases is FFS

Dynamic PTM/PTP switch is supported for a split MRB bearer (type) with a common (single) PDCP entity.

As a baseline, no new UE based signalling is introduced to support gNB switch decision (e.g. PDCP SR for high reliability is still TBD)

Assuming a split-MRB (as agreed during the online session) configured with a PTM leg and PTP leg, the usage of the PTP leg cannot be deactivated (i.e. the UE needs to always monitor C-RNTI) after the necessary split-MRB configuration.

Assuming a split-MRB (as agreed during the online session) configured with a PTM leg and PTP leg, it is FFS whether the usage of the PTM leg of the split-MRB may be subject to activation or deactivation and the details of such.

Support of Multicast in Idle Inactive

Chair: RAN2 will prioritize Active Multicast support in RRC Connected mode in Rel-17. If time permits Multicast support for RRC Inactive can be considered later (once connected mode Multicast solution, and Broadcast solution has become more mature).

Idle and Inactive mode Ues

The MCCH transmission window is defined by MCCH repetition period, MCCH window duration and radio frame/slot offset.

New RNTI is defined for scheduling MCCH.

The concept of MCCH transmission window, similar to the one used for LTE SC-PTM, is used for NR MCCH scheduling. The exact parameters to define the window are FFS (discussed in the following proposals).

Common search space is needed for MCCH scheduling. RAN2 should request RAN1 to discuss the details of CSS for MCCH.

R2 assumes PDCCH occasions for MCCH search space are associated with SSBs in a pre-defined manner so that the UE can receive MCCH scheduling on PDCCH occasions according to its detected SSB.

R2 assumes, In case searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the mapping between PDCCH occasions and SSBs is the same as for SIB1.

R2 assumes that If common search space other than searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the PDCCH monitoring occasions for MCCH message which are not overlapping with UL symbols are sequentially numbered from one in the MCCH transmission window and mapped to SSBs using the similar rule as defined for OSI in TS 38.331.

Request RAN1 to discuss the details of the configuration of the bandwidth for MCCH reception.

The modification period is defined for NR MCCH and NR MCCH contents are only allowed to be modified at each modification period boundary.

The updated MCCH message should be sent in the same MCCH modification period where the change notification is sent.

UE in RRC IDLE/INACTIVE should be able to monitor/read both MCCH channel and SI/Paging without BWP switch. It is up to RAN1 to decide how this is ensured.

It is up to RAN1 to to decide about the RNTI and DCI format used for MCCH change notifications.

FFS whether to support multiple MCCH, e.g. to support different service types.

RAN2 will discuss and down-select from the following two options for the UE to get aware of session stop/modification:

- Reading MCCH once per each MCCH modification period when receiving an ongoing broadcast session

- DCI used for MCCH notification indicates the change of an ongoing broadcast session

RAN2#114-e agreements

Stage-2 and Multicast activation

Use PCCH for Multicast activation notification (also for MBS supporting nodes).

Confirm that we convey the MBS session ID in the notification.

Use of paging in all (legacy) PO with PRNTI is the baseline assumption (can still discuss other variants)

Reliability

RLC-AM is not supported for PTM (for MBS R17 WI).

Group scheduling and others

One-to-one mapping between G-RNTI and MBS session is supported in NR MBS. Other mappings FFS

One-to-one mapping between G-CS-RNTI and MBS session is supported in NR MBS. Other mappings FFS.

A UE can support multiple G-RNTIs/G-CS-RNTIs, It is FFS whether this depends on UE capability. Inform RAN1 of this agreement.

Multiple MBS QoS flows corresponding to the same MBS session can be mapped to one or more than one MBS radio bearers.

MCCH is mapped to the DL-SCH for NR MBS delivery mode 2.

MTCH is specified for PTM transmission of NR MBS.

MTCH is mapped to the DL-SCH.

DTCH is reused for PTP transmission of NR MBS.

FFS if there is a need to have specific LCID spaces for the used channels.

Multiplexing/de-multiplexing of different logical channels associated with the same G-RNTI is supported for NR MBS.

FFS if Multiplexing/de-multiplexing of different logical channels associated with the same G-CS-RNTI is supported for NR MBS.

Multiplexing/de-multiplexing of different logical channels associated with the C-RNTI is supported for NR MBS.

For NR MBS delivery mode 2, LTE SC-PTM DRX scheme is used as baseline.

FFS whether For PTM transmission of NR MBS, DRX scheme is independent of DRX for unicast transmission, e.g. supported on a per G-RNTI basis

FFS whether For PTP transmission, DRX operation for unicast transmission is reused.

Idle and Inactive UEs

MBS specific SIB is defined to carry MCCH configuration.

MCCH contents should include information about broadcast sessions such as G-RNTI, MBS session ID as well as scheduling information for MTCH (e.g. search space, DRX). L1 parameters that need to be included in MCCH are pending further RAN1 progress and input.

Postpone the discussion on whether dedicated MCCH configuration is required until RAN1 makes progress on BWP/CFR for MCCH.

Indication of an MCCH change due to modification of an ongoing session’s configuration (including session stop) is provided with an explicit notification from the network  (provided that RAN1 confirms a separate bit for this purpose can be accommodated in the MCCH change notification DCI, in addition to a bit for session start notification). FFS on whether this notification can be reused for modification of other information carried by MCCH, if any.

FFS whether the possibility of UE missing an MCCH change notification needs to be addressed or can be left to UE implementation.

At least in case RAN1 decides to utilize RNTI other than MCCH-RNTI for MCCH change notification, MCCH change notification is sent in the first MCCH monitoring occasion of each MCCH repetition period.

We support single MCCH (in this release)