3GPP TSG-RAN WG2 Meeting #114-e	R2-2106764

# Electronic Meeting, 19th – 27th May 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.340 | CR | 0018 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the handling of unknown, unforeseen, and erroneous protocol data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T, Samsung, Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to Rel.16 BAP specification, if an IAB node receives a BAP PDU with a BAP header containing reserved or invalid values, it will discard the BAP PDU. However, currently, BAP Data PDU cannot consist of reserved or invalid values – only BAP Control PDU can consist of reserved values. Furthermore, in case the receiving BAP entity forwards BAP SDUs to the collocated transmitting part of the BAP entity, only the BAP Routing ID of the BAP header is copied. However, in case RAN2 decides in a future release to support new features that imply enhancements to the BAP header, it might happen that a “new” IAB node uses such new features that are not supported by a peer receiving “old” IAB node. When the BAP SDUs are forwarded, the “old” IAB node would not consider the setting of the new content (e.g., the Rel-16 R bits) in the BAP header of the received BAP packet as containing information and would not set those accordingly. Hence, the second “new” IAB node would not necessarily decode the BAP PDU correctly if an “old” IAB node was in between the two new ones. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The specification is clarified such that only for BAP Control PDU it is checked if it includes reserved or invalid values.. Additionally, in section 4.2.2, it is change “BAP routing ID” to “BAP header content”.. Impact analysis Impacted functionality: Handling of unknown, unforeseen, and erroneous protocol data Inter-operability: If the network is implemented according to the CR while the IAB-MT is not, there is no interoperability issue, since the CR only affects the IAB-MT receptions functionalities. If the IAB-MT is implemented according to the CR while the network is not, there is no interoperability issue, since the CR only affects the IAB-MT receptions functionalities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If in future releases the BAP header is enhanced to support new features, a Rel.16 IAB node may not set the BAP header correctly in case BAP SDU forwarding is used in the IAB node. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5, 4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 4.2.2	BAP entities

On the IAB-node, the BAP sublayer contains one BAP entity at the MT function and a separate collocated BAP entity at the DU function. On the IAB-donor-DU, the BAP sublayer contains only one BAP entity. Each BAP entity has a transmitting part and a receiving part.

NOTE: The modelling of BAP entities does not restrict internal implementation of IAB-nodes, i.e. the exact modelling of BAP sublayer may vary for different IAB-node implementations.

The transmitting part of the BAP entity has a corresponding receiving part of a BAP entity at the IAB-node or IAB-donor-DU across the BH link.

Figure 4.2.2-1 shows one example of the functional view of the BAP sublayer. This functional view should not restrict implementation. The figure is based on the radio interface protocol architecture defined in TS 38.300 [2].

In the example of Figure 4.2.2-1, the receiving part on the BAP entity delivers BAP PDUs to the transmitting part on the collocated BAP entity. Alternatively, the receiving part may deliver BAP SDUs to the collocated transmitting part. When passing BAP SDUs, the receiving part removes the BAP header and the transmitting part adds the BAP header with the same BAP header content as carried on the BAP PDU header prior to removal. Passing BAP SDUs in this manner is therefore functionally equivalent to passing BAP PDUs, in implementation. The following specification therefore refers to the passing of BAP Data Packets.

Besides, BAP entity generates, delivers/receives BAP Control PDU(s) as described in clause 6.1.2. BAP Control PDU can only be exchanged between peer BAP entities across the BH link.

NEXT CHANGE

## 5.5	Handling of unknown, unforeseen, and erroneous protocol data

When a BAP Data PDU that contains a BAP address which is not included in the configured BH Routing Configuration and is not the BAP address of this node is received;or when a BAP Control PDU that contains reserved or invalid values is received the BAP entity shall:

-	discard the received BAP PDU.

END OF CHANGE