3GPP TSG-RAN WG2 Meeting #114-e Electronic	DRAFT R2-210xxxx

Online, 19 – 27 May 2021

Title:	[DRAFT] LS on inter-MN handover without SN change

Response to:	-

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN3

Cc:

Contact Person:

#### Name:	Amaanat Ali

E-mail Address:	amaanat.ali@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

For the inter-MN handover without SN change scenario, RAN2 discussed how MN can indicate whether SN should apply delta signalling or full configuration. For one of the options discussed, the signalling of the fields is as shown by the example below:

- Inter-MN HO without SN change (implies SN may provide delta configuration)

  - SN UE X2/XnAP ID        present

  - sourceConfigSCG          not present

  - scg-RB-Config               not present

- Inter-MN HO without SN change (implies SN must apply full configuration)

  - SN UE X2/XnAP ID        not present

  - sourceConfigSCG          not present

  - scg-RB-Config               not present

RAN2 would like to ask RAN3 the following questions:

Question 1: In the inter-MN handover without SN change scenario, is the SN UE X2/XnAP ID always required to be present when target MN sends SN Addition Request to SN regardless of delta configuration or full configuration?

Question 2: For the same scenario, RAN2 would like to confirm with RAN3 if the receipt of SN UE X2/XnAP ID alone may be interpreted by SN to retrieve the SCG configuration to provide delta configuration?

2. Actions:

To RAN3 group.

ACTION: 	RAN2 respectfully asks RAN3 to answer the above questions.

3. Date of Next TSG-RAN WG2 Meeting:

TSG-RAN2 Meeting #115-e 	Aug 23 – Aug 27, 2021		Meeting