3GPP TSG-RAN WG2 Meeting #114 Electronic	R2-210xxxx

Elbonia, 19 – 27 May 2021

Agenda item:	6.1.3.1

Source:	Nokia (Rapporteur)

Title:	[AT114-e][018][NR16] MAC III (Nokia)

WID/SID:	NR_unlic-Core, NR_IIOT-Core, NR_IAB-Core, NR_2step_RACH-Core, TEI16 - Release 16

Document for:	Discussion and Decision

# 1	Introduction

This document is the report of the following email discussion:

[AT114-e][018][NR16] MAC III (Nokia)

Scope: Treat R2-2104724, R2-2105231, R2-2105865, R2-2105232, R2-2105749, R2-2106031, R2-2106321, R2-2105851

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed CRs.

Deadline: Schedule A

# 2	Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Nokia (Rapporteur) | Chunli Wu | Chunli.wu@nokia-sbell.com |
| Qualcomm | Linhai He | linhaihe@qti.qualcomm.com |
| LG | SunYoung LEE | ssunyoung.lee@lge.com |
| Xiaomi | Yumin Wu | wuyumin@xiaomi.com |
| Lenovo | Joachim LÖhr | jlohr@lenovo.com |
| Ericsson2 | Martin van der Zee | martin.van.der.zee@ericsson.com |

# 3	Discussion

NR-U

R2-2104724	LS on SCell activation requirements for NR-U (R4-2105699; contact: Nokia)	RAN4	LS in	Rel-16	NR_unlic-Core	To:RAN2

Moved here

R2-2105231	Analysis on SCell activation/deactivation requirements for NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

No action is required from the RAN4 LS R2-2104724, which also stated in the contribution R2-2105231 “Proposal 1: The RAN4 LS on SCell activation requirements for NR-U has no explicit impacts to RAN2 specs.” Rapporteur propose to note the LS.

Question 1: Do companies agree there is no impact to RAN2 from the RAN4 LS and the LS can be noted?

| Answers to Question 1 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Qualcomm | Yes |  |
| LG | Yes |  |
| Xiaomi | Yes |  |
| Lenovo | Yes |  |
| Nokia | Yes |  |

Summary 1: TBD.

Proposal 1: TBD.

The following proposal is proposed in R2-2105231 for sCellDeactivationTimer handling in MAC:

“Proposal 2: If an SCell Activation/Deactivation MAC CE is received deactivating the SCell configured with shared spectrum channel access, UE may stop the sCellDeactivationTimer associated with the SCell after the HARQ feedback for the SCell deactivation MAC CE is successfully transmitted. ”

Question 2: Do companies think the change proposed in proposal 2 in R2-2105231 is needed?

| Answers to Question 2 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Qualcomm | No | The problem described in the paper could happen in theory but it should be a rare event. Even if LBT failure persists for a very long time, it is likely that RLF will be triggered. Lastly, the proposed solution itself is not completely fault proof either, e.g. in the case where there is persistent HARQ feedback failure. |
| LG | No | Although HARQ feedback is not transmitted due to LBT failure, sCellDeactivationTimer will be finally expired and the SCell will be in deactivated state. The statement in TS38.321 should not prevent expiring sCellDeactivationTimer. ‘The MAC does not stop sCellDeactivationTimer’ is not equal to ‘The MAC prevents sCellDeactivationTimer expired’. 1> else if an SCell Activation/Deactivation MAC CE is received deactivating the SCell; or 1> if the sCellDeactivationTimer associated with the activated SCell expires: 2> deactivate the SCell according to the timing defined in TS 38.213 [6]; 2> stop the sCellDeactivationTimer associated with the SCell; |
| Xiaomi | No | If the HARQ feedback for the SCell deactivation command is not transmitted due to LBT failure, the UE will trigger “5.21.2 LBT failure detection and recovery procedure”. It seems there is no issue to resolve. |
| Lenovo | No | Agree with others that in the rare case that LBT failure occurs for very long time, other mechanism will kick in like e.g. LBT failure recovery mechanism |
| Nokia | No | Agree with others. |

Summary 2: TBD.

Proposal 2: TBD.

R2-2105865	Clarification on prioritization of retransmission over initial transmission for HARQ PID selection in NR-U	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.321	16.4.0	1115	-	F	NR_unlic-Core

R2-2105865 proposed to clarify that the UE shall prioritize retransmissions before initial transmissions is only applicable to HARQ PID selection but not for intra-UE prioritization for multiple UL grants, since there has been some different understandings in the context of I-IoT discussions:

For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. For HARQ Process ID selection, the UE shall prioritize retransmissions before initial transmissions. The UE shall toggle the NDI in the CG-UCI for new transmissions and not toggle the NDI in the CG-UCI in retransmissions.

Question 3: Do companies agree with the issue and if yes, are the suggested changes fine or does the text need to be improved / corrected ?

| Answers to Question 3 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Qualcomm | No | We do not think the reasons for change is correct. The agreement was for grant selection. Moreover, we do not think there is HARQ PID selection for retransmission. |
| LG | Yes | Indeed, it is a necessary clarification. The intention is to prioritize retx over initial when selecting HARQ PID. Regarding QC’s comment, we don’t think there is an intention to select HARQ PID even for retransmission. |
| Xiaomi | No | We understand that the current specification text is to avoid new data flushing the old data in the HARQ process when the cg-RetransmissionTimer expires. However the whole paragraph is under the condition “configured uplink grants configured with cg-RetransmissionTimer”. This is definitely not for I-IOT CG. And RAN2 already agreed that the gNB implementation will ensure that no functional collision for the CG handling between NR-U and IIOT, and no CR is needed. R2 Confirm the assumption that network implementation is to handle the potential ambiguities for R16 UEs, e.g. by not configuring both features at the same time (cg-RetransmissionTimer and autonomousTransmission). R2 will not further work on this for R16 UEs. No R16 CRs are expected. |
| Lenovo | Yes/No | We agree that the prioritization is for the HARQ process selection, i.e. UE shall prioritize the HARQ process with pending retransmission over a different HARQ process used for initial transmission. However we are not sure whether the clarification is really needed. We would be OK to go with the clarification is majority prefers to have it. |
| Nokia | Yes | We disagree with Qualcomm. The text is for NR-U process selection other than for IIoT UL grant selection. That is exactly why we should clarify due to such misunderstanding, because apparently there is a huge discrepancy in understanding among companies about what this sentence really means. For Xiaomi’s comments, we would like to point out that this is independent to whether IIOT features are configured or not. Even when IIoT features are not configured, the sentence “the UE shall prioritize retransmissions before initial transmissions” should not be applicable to grant selection in any case. Prioritizing retransmission for grant selection is not what we have agreed in Rel-16. |

Summary 3: TBD.

Proposal 3: TBD.

Secondary DRX

R2-2105232	Clarification on secondary DRX group	Samsung	CR	Rel-16	38.321	16.4.0	1104	-	F	TEI16

R2-2105232 proposed to remove “associated” drx-onDurationTimer and specify in MAC “secondary DRX group is not configured, when DCP monitoring is configured”. Note that the restriction has already captured in RRC:

drx-ConfigSecondaryGroup Used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3]. The network does not configure secondary DRX group with DCP simultaneously nor secondary DRX group with a dormant BWP simultaneously.

Question 4: Do companies agree with the issue and if yes, are the suggested changes fine or does the text need to be improved / corrected ?

| Answers to Question 4 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Qualcomm | No | We do not think #1 change is correct. Each DCP occasion has an associated on duration timer, which is the first instance of on duration timer after a DCP. That's why "associated" is used in the current text. We do not think #2 change is not necessary. This restriction is already captured in the field description of drx-ConfigSecondaryGroup in 331. |
| LG |  | 1st change (to remove 'associated') – not necessary because drx-onDurationTimer is associated with the DRX group. 2nd change (to specify secondary DRX in presence of DCP monitoring) – no strong view, but no need to redundantly capture that across TSs. |
| Xiaomi | No | The configuration restriction captured in 38.331 seems sufficient. |
| Lenovo | No |  |
| Ericsson | No | We agree with the comments from QC. |
| Nokia | No | Agree with Qualcomm. |

Summary 4: TBD.

Proposal 4: TBD.

eLCID

R2-2105749	Clarification on MAC PDU assembly with eLCID	Huawei, HiSilicon	discussion	Rel-16	NR_IAB-Core

R2-2106031	Clarification to transmission of padding and padding BSR with eLCID in IAB	Ericsson, Apple	CR	Rel-16	38.321	16.4.0	1116	-	F	NR_IAB-Core

R2-2106321	CR for not transmitting only padding and padding BSR with eLCID	Samsung, Nokia, Nokia Shanghai Bell, Qualcomm, LG, ZTE, MediaTek, Intel	CR	Rel-16	38.321	16.4.0	1118	-	F	NR_IAB-Core

It has been agreed in the previous meeting to clarify this in MAC. Different styles are proposed in the above 3 contribution/CRs.

Question 5: Which of the changes proposed in the above TDocs do companies support?

| Answers to Question 5 |  |  |
| --- | --- | --- |
| Company | R2-2105749 /R2-2106031 /R2-2106321 | Technical Arguments |
| Qualcomm | R2-2106321 | We are fine with the TP in R2-2105749 too. |
| LG | 6321 | We think it is possible that MAC includes zero RLC SDU segment but only the RLC header. Therefore, 5749 is not the way to go. As a proponent of 6321, the change in 6321 is simple and sufficient. |
| Xiaomi | R2-2106321 |  |
| Lenovo | R2-2106321 |  |
| Ericsson | R2-2106031 | Regarding R2-2106321, we have concerns on the terminology used. In particular, the term “is used” is not clear whether it refers to the IAB node being configured with eLCID or using it for this transmission. In our understanding the “is used” should refer to the LCID associated to “data available and allowed” that are going to be transmitted. However, that is not clear from R2-2106321. In any case if R2-2106321 is agreed, we believe that the inter-operability analysis should be changed since it seems that also UEs are involved, which is not correct. Additionally, we do not foresee any inter-operability issues, since in any case the NW will always decode the MAC PDU, and if data are detected, those will not be discarded. For this reason, we believe that R2-2106031 is technically more correct, because it solves the issue related to the “is used” above. It also increases readability, since it clearly separates the scenarios of 8 and 10 bytes, and clarifies that the 10 bytes scenario is only applicable to the case two-octet LCID. Regarding R2-2105749, we agree with the intention to find a general statement that applies to all cases, however that modifies a legacy text (since the 8 bytes handling is removed), which is better to avoid at this stage. |
| Nokia | R2-2106321 | It is also the outcome of the lengthy offline from previous meeting email discussion. |

Summary 5: TBD.

Proposal 5: TBD.

2-Step RACH

R2-2105851	Correction to 38.321 on msga-TransMax selection for 2-step RACH	ZTE, Sanechips	CR	Rel-16	38.321	16.4.0	1112	-	F	NR_2step_RACH-Core

R2-2105851 proposed to change the rach-ConfigDedicated to cfra-TwoStep-r16 for the application of the msgA-TransMax in subclause 5.1.1a to correct the behaviour for HO:

1> if RA_TYPE is set to 2-stepRA: 2> set PREAMBLE_POWER_RAMPING_STEP to msgA-PreamblePowerRampingStep; 2> set SCALING_FACTOR_BI to 1; 2> apply preambleTransMax included in the RACH-ConfigGenericTwoStepRA; 2> if the Random Access procedure was initiated for handover; and 2> if cfra-TwoStep is configured for the selected carrier: 3> if msgA-TransMax is configured in the cfra-TwoStep; 4> apply msgA-TransMax configured in the cfra-TwoStep. 2> else if msgA-TransMax is included in the RACH-ConfigCommonTwoStepRA: 3> apply msgA-TransMax included in the RACH-ConfigCommonTwoStepRA.

Question 6: Do companies agree with the issue and if yes, are the suggested changes fine or does the text need to be improved / corrected ?

| Answers to Question 6 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Qualcomm | Yes | We think the reasons for change are correct, and we are fine with the TP. |
| LG | No | In our understanding, the original intention is not to allow switching from 2-step RA to 4-step RA if msaA-TransMax is not present in rach-ConfigDedicated. Please see the parameter description copied from 38.331: msgA-TransMax Max number of MsgA preamble transmissions performed before switching to 4-step type random access (see TS 38.321 [3], clauses 5.1.1). This field is only applicable when 2-step and 4-step RA type are configured and switching to 4-step type RA is supported. If the field is absent in RACH-ConfigDedidated, switching from 2-step RA type to 4-step RA type is not allowed. |
| Xiaomi | Yes | It seems that the two bullets 2> and 3> can be combined as one, which may make the specification more readable. 2> if msgA-TransMax is configured in the cfra-TwoStep for the selected carrier: 3> apply msgA-TransMax configured in the cfra-TwoStep. We think that the 38.331 sentence quoted by LG should be removed as it does not cover the case when “msgA-TransMax is only included in the RACH-ConfigCommonTwoStepRA”. |
| Nokia | Yes | Agree with the proposed text in R2-2105851. The proposal from Xiaomi would not work since it is possible that msgA-TransMax is not configured in cfra-TwoStep, but cfra-TwoStep is configured, then the UE still should not apply the msgA-TransMax in RACH-ConfigCommonTwoStepRA. |

Summary 6: TBD.

Proposal 6: TBD.

# 4	Conclusion

TBD.