3GPP TSG-RAN WG2 #114-e electronic	R2-210xxxx

Electronic Meeting, 19th – 27th May, 2021

Agenda Item:	8.17.1

Source:	Intel Corporation

Title:	[AT114-e][035][feMIMO] TCI states indication for PDCCH (Intel)

Document for:	Discussion, Decision

# 1	Introduction

This contribution summarizes the following discussion:

- [AT114-e][035][feMIMO] TCI states indication for PDCCH (Intel)

Scope: Treat R2-2104712 and the related submitted tdocs.

Discuss the topic, attempt to make some basic agreements, e.g. agree to have the requested MAC CE, and potentially identify FFS.

Intended outcome: Report

Deadline: Monday May 24 for on-line CB

Contact person(s) for each participating company:

| Company | Name | Email Address |
| --- | --- | --- |
| Intel | Youn Heo | Youn.hyoung.heo@intel.com |

# 2	Discussion

RAN1 sent an LS [1] to inform RAN1 agreement on PDCCH enhancement. In this LS, RAN1 agreed to introduce enhanced MAC CE signaling for PDCCH activating two TCI states for SFN-based PDCCH transmission.

Agreement - Introduce enhanced MAC CE signaling for PDCCH activating two TCI states for SFN-based PDCCH transmission - The corresponding MAC CE includes at least the following fields - Serving cell ID - CORESET ID - Two TCI state IDs - FFS whether for CA scenario additionally support RRC configured set of the serving cells which can be addressed by a single MAC CE - FFS whether or not enhanced MAC CE signaling is applicable to a CORESET configured with CORESETPoolindex

All contributions [2-5] seem to agree that the existing TCI State Indication for UE-specific PDCCH MAC CE is not enough.

Q1: Do you agree that the exsiting MAC CE is not sufficient to support Rel-17 PDCCH enhancement and therefore we should introduce enhanced MAC CE?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes |  |

RAN1 requested to include the following fields 1) serving cell ID, 2) CORESET ID, 3) Two TCI state IDs.

Q2: Do you agree that the enhanced MAC CE should include the following fields 1) serving cell ID, 2) CORESET ID and 3) Two TCI state IDs?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes |  |

Three companies provide the possible enhanced MAC CE structure. Two of them ([2,4]) are the same format, while the other [5] is slightly different. However, there seems no big difference. Is there any preference between two formats?

Q3: Which MAC CE strcture is preferred?

Option 1 [2,4]						Option 2 [5]

| Company | Preference | Comments |
| --- | --- | --- |
| Intel | Either one is ok. |  |

There are some views that RAN2 should ask RAN1 on some questions to clarify [4,5].

- Whether the enhanced TCI state indication for UE specific PDCCH MAC CE can be applied to a set of serving cells configured in simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2? [4]

- Which CORESET can be indicated with two TCI states ? [5]

- How many TCI states (i.e. maximum number) can be configured for the CORESET indicated with two TCI states? [5]

- Anything else?

Q4: Do you agree to send LS to aks RAN1 some questions? If yes, are the above question A-C reasonable to ask? Companies are also invited to provide more questions if deemed useful.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | Question A: This issue is currently captured as FFS bullet in RAN1 LS [1]. Question B: we understand that it should not be applicable to CORESET 0 similar to Rel-15 MAC CE design. Question C: There is no proposal in RAN1 to increase the number of TCI states to larger value comparing to Rel-15. |

Q5: Is there any aspect that RAN2 needs to discuss?

| Company | Comments |
| --- | --- |

# 3	Conclusion

To be updated.

# 4	References

- R2-2104712	LS on TCI states indication for PDCCH (R1-2104064; contact: Intel)	RAN1	LS in	Rel-17	NR_feMIMO-Core	To:RAN2

- R2-2105027	Enhanced MAC CE for PDCCH in multi-TRP deployment	Intel Corporation	discussion	Rel-17	NR_feMIMO

- R2-2105907	On the LS about Activating two TCI states with a MAC CE	Ericsson	discussion	NR_feMIMO-Core

- R2-2105858	Consideration on the enhanced TCI state indication MAC CE for PDCCH	ZTE, Sanechips	discussion	Rel-17	NR_feMIMO-Core

- R2-2105731	Enhanced TCI State Indication for UE-specific PDCCH MAC CE	Xiaomi Communications	discussion	Rel-17	NR_feMIMO-Core