3GPP TSG-RAN WG2 #114-e	Tdoc R2-21xxxxx

Electronic meeting, 2021-05-19 - 2021-05-27

Title:	DRAFT Reply LS on PDB for new 5QI

Reply to:	LS on PDB for new 5QI (S2-2103552)

Release:	Release 17

Work Item:	5GSAT_ARCH, NR_NTN_solutions-Core

Source:	Ericsson (to be: TSG RAN WG2)

To:	SA2

Cc:	RAN1, RAN3

Contact Person:

#### Name:	Robert S Karlsson

E-mail Address: robert.s.karlsson AT ericsson dot com

Attachments:	None

1. Overall Description:

RAN2 would like to thank SA2 for sending their LS on PDB for new 5QI.

According to TR 38.821, the max round trip delay (propagation delay only) for GEO satellite access with transparent payload is 541.46 ms. Thus, the AN PDB of 812 ms is about 1.5 RTT of the maximum round trip delay.

1.5 RTT can in best case cover one HARQ retransmission, it will not be possible with RLC retransmissions. GEO systems may not use HARQ retransmission.

Further the PER of 10-6 while meeting a PDB of about 1.5 RTT will be challenging and will require excessive resources and thus lead to low spectral efficiency.

We also request SA2 to consider providing more flexible QoS by supporting flexible scaling factors for Packet Delay Budget (PDB) and Packet Loss Error Rate (PLER) for various 5QI is so that the NTN operators can support diverse services (e.g., VoIP and gaming with relaxed QoS compared to the existing QoS) to their customers, flourishing the NTN ecosystem.

2. Actions:

To SA2 group.

ACTION: 	RAN2 respectfully requests SA2 to take the above into account.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG1 Meeting #115-e 	16 – 27 August 2021		Electronic Meeting

TSG-RAN WG1 Meeting #116-e 	01 – 12 November 2021	Electronic Meeting