3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2107993

Online, Aug 16 – Aug 27, 2021	Revision of R2-2105886

Agenda item:	8.6.4

Source:	Qualcomm Incorporated

Title:	Open issues for RACH based SDT

WID/SID:	NR_SmallData_INACTIVE-Core – Release 17

Document for:	Discussion and Decision

# Introduction

In RAN2 #113e, several agreements were made related to RACH based SDT.

Agreements

1	For RA-SDT, up to two preamble groups (corresponding to two different payload sizes for MSGA/MSG3) may be configured by the network

4	If RACH procedure is initiated for SDT (i.e. RA-SDT initiated), the UE first performs RACH type selection as specified in MAC (i.e. Rel-16). FFS whether threshold is SDT specific or not

Agreements:

- RAN2 continues to progress the work based the separate RACH resources for SDT (i.e. explicit mechanisms to support common resources won’t be pursued unless there is sufficient support for this. However, use of common RACH resources will not be precluded if possible via implementation

- RAN2 design assumes that RRCRelease message is sent at the end to terminate the SDT procedure from RRC point of view.   The RRCRelease sent at the end of the SDT may contain the CG resource (as per previous agreement).   Write an LS to SA3 to explain SDT procedure and agreement.

- The UE behaviour for handling of non-SDT data arrival after sending the first UL data packet is fully specified (i.e. not left to UE implementation)

- FFS RAN2 will consider the additional option of using DCCH message to indicate arrival of non-SDT data (details to be discussed).  Discussion will continue on all three options.

- FFS: RSRP threshold to select between SDT and non-SDT procedure.

- FFS also whether this RSRP threshold to select between SDT and non-SDT procedure is used for CG-SDT, RA-SDT, or both and whether the RSRP threshold is the same for CG-SDT and RA-SDT. FFS when the RSRP threshold check is made

- FFS If both carriers can be selected and CG resources are available on one carrier only, does the UE select the carrier with CG?

- For SDT, UE performs UL carrier selection (i.e. if SUL is configured in the cell, UL carrier selected based on RSRP threshold).  FFS whether the RSRP threshold for carrier selection is specific to SDT)

- If CG-SDT resources are configured on the selected UL carrier and are valid, then CG-SDT is chosen.  Otherwise,

•	 If 2 step RA-SDT resources are configured on the UL carrier and criteria to select 2 step RA SDT is met, then 2 step RA-SDT is chosen

•	else If 4 step RA-SDT resources are configured on the UL carrier and criteria to select 4 step RA SDT is met, then 4 step RA-SDT is chosen

•	else UE does not perform SDT (i.e. perform non-SDT resume procedure)

•	 If both 2 step RA-SDT and 4 step RA-SDT resources are configured on the UL carrier, RA type selection is performed based on RSRP threshold.

FFS whether RSRP threshold for RA type selection is common or different for SDT and non SDT.

FFS what validity includes if we need to deal with CG resource availability delay?

RAN2 #113bis-e further made following agreements related to RACH based SDT.

Agreements: - RSRP threshold is used to select between SDT and non-SDT procedure, if configured (RSRP refers to the same RSRP measured for carrier selection). - RSRP threshold to select between SDT and non-SDT procedure is used for both CG-SDT and RA-SDT - RSRP threshold to select between SDT and non-SDT procedure is same for both CG-SDT and RA-SDT - RSRP threshold for carrier selection is specific to SDT (i.e. separately configured for SDT). This is optional for the network. - Confirm that cell selection mechanism is not modified - RSRP threshold for RA type selection is specific to SDT (i.e. separately configured for SDT) - Data volume threshold is the same for CG-SDT and RA-SDT (can be checked further in stage 3 if we obtain majority support) 8 FFS on the order and missing pieces (e.g. failure, fallback) of the high level procedure. The details of the procedures are left for stage 3. FFS on the procedure below, but copied for information. A. Upon arrival of data only for DRB/SRB(s) for which SDT is enabled, the high level procedure for selection between SDT and non SDT procedure is as follows: If CG-SDT criteria is met: UE selects CG-SDT. UE initiate SDT procedure Else if RA-SDT criteria is met: UE selects RA-SDT. UE initiate SDT procedure Else: UE initiate non SDT procedure. B. CG-SDT criteria is considered met, if all of the following conditions are met, 1) available data volume <= data volume threshold 2) RSRP is greater than or equal to a configured threshold FFS 3) CG-SDT resources are configured on the selected UL carrier and are valid C. RA-SDT criteria is considered met, if all of the following conditions are met, 1) available data volume <= data volume threshold 2) RSRP is greater than or equal to a configured threshold 3) 4 step RA-SDT resources are configured on the selected UL carrier and criteria to select 4 step RA SDT is met; or 2 step RA-SDT resources are configured on the selected UL carrier and criteria to select 2 step RA SDT is met - Switching from SDT to non-SDT is supported. - FFS Switching from CG-SDT to RA-SDT is not allowed 11 UE switches from SDT to non-SDT in following cases: - Case 1 (27/0): UE receive indication from network to switch to non-SDT procedure. Network can send RRCResume. FFS whether network can send indication in RAR/fallbackRAR/DCI to switch to non-SDT procedure. - FFS Case 2 (18/9): Initial UL transmission (in msgA/Msg3/CG resources) fails configured number of times

RAN2 #114e further made following agreements related to RACH based SDT.

Agreement

- CFRA is not supported for RA-SDT

- The separate search space is common to the UEs performing RA-SDT. Inform RAN1 of this agreement

In this paper, we would like to discuss more details specific to the RACH based SDT and provide our view.

# Discussion

Switching between RACH based SDT and legacy RRC resume procedure (non-SDT)

RAN2 #113bis-e meeting has agreed that switching from SDT to non-SDT should be supported. For network-based solution, network can send RRC Resume message and UE may receive the indication from network to switch to non-SDT procedure. For UE based solution, one possible solution is to support switching when there are number of times of failed initial uplink transmission.

In Rel-16 2-step RACH procedure, network indicates UE to retransmit MSGA payload by fallbackRAR if only preamble is decoded while payload is not. If UE still cannot receive any response after transmits MSGA or the contention resolution is still not successful after MSGA payload retransmission in Msg3, UE has chance to reattempt 2-step RACH. For 4-step RACH procedure, if contention resolution is not successful, UE should go back to Msg1 to reattempt RACH.

For RACH based SDT, the uplink data can be transmitted in MSGA or Msg3 if RA-SDT criteria are met, i.e. data volume threshold or RSRP threshold. Given the relative larger size of user data (compared to the CCCH) and unpredicted interference in the contention based PUSCH resource in MSGA or Msg3, it is possible the user small data is still failed to send to network after a certain number of payload retransmission in 2-step or 4-step RACH procedure.

Observation 1: It may be still difficult to decode the user data for network even after a number of payload retransmission in 2-step or 4-step RACH procedure.

Furthermore, the retransmitted user data in the contention based PUSCH may cause potential large interference to the legacy RACH users or other small data users. UE will endure a large latency for user data transferring during MSGA or Msg3 retransmission which obviously wastes radio resource and user power which is obviously inefficient.

Observation 2: It is inefficient for MSGA or Msg3 retransmission including user data with the same number of attempts of regular RACH.

A simple solution is to introduce a new counter threshold or timer for MSGA/Msg1 retransmission for SDT case. In regular RACH procedure, several parameters are configured for UE to control the RACH reattempts, such as the maximum value of MSGA retransmission or maximum value of preamble transmission. When the number of retransmissions reaches the configured maximum value, UE is expected to switch to another type of RACH or report failure.

For the RACH based SDT procedure, a smaller maximum number of retransmission counter or timer can control UE to stop performing the SDT in time. This can help to avoid the waste of radio resources of the failed user data retransmission in RACH procedure, especially when network cannot provide any response during the SDT procedure. Thus, if UE has already performed the user data transmission with the newly configured counter or timer, UE should fallback to finish the RRC resume procedure first and transmit the data after the legacy RRC resume procedure is finished.

Proposal 1: A new counter threshold or timer for MSGA/Msg1 retransmission is introduced for RACH based SDT.

Meanwhile, it is possible for network to send an explicit indication, such as in RAR or fallbackRAR to switch UE from SDT procedure to non-SDT procedure if network decides to do it. For example, network may indicate a flag in RAR of 4-step RACH to disallow UE sending user data in Msg3. Then when UE receives the Msg2 with such indication, UE falls back to legacy RRC Resume procedure.

Proposal 2: Network sends an indication in RAR or fallbackRAR to switch UE from RACH based SDT to legacy RRC resume procedure.

RACH monitoring enhancement for SDT

For RACH based SDT, when RACH is performed, a significant amount of power is spent on monitoring the response messages. The UE has to check PDCCH every slot whether there is a response to its transmission (Msg2/Msg4 in 4-step RACH) and msgB in 2-step RACH.

For regular UEs and traffic, such power consumption is not an important issue considering the overall activity. However, for IoT type devices (i.e. RedCap UE), the above power can dominate their overall consumption. This is especially the case when “small data” transmission is used, where the UE occasionally sends a small amount of data and goes back to sleep (e.g. sensors, meters).

Since the SDT and non-SDT users will be configured with different RACH resources, network distinguishes SDT users by detecting the RACH occasions and partitioned preambles. When network detects the SDT users successfully and receives the small data, network needs more time to decode user data due to the larger payload size of user data compared to the CCCH message. Furthermore, considering fetching UE-specific RLC configuration to decode the first user data in the serving gNB, it should give network more flexibility to take more time to response to SDT users.

Observation 3: Network needs more time to decode user data contained in MSGA/Msg3 than the legacy RACH users.

In addition, since network has to require additional time to fetch the RLC configuration from the anchor gNB and to decode the potential larger payload size, it is beneficial to allow UE starting a little late to monitor the network response for saving UE power. The duration of the response window could be also be limited which could be different from the one configured for the legacy RACH user. The network could determine the different monitoring time for the SDT users based on detecting the different RACH occasions and partitioned preambles.

Observation 4: It is beneficial for saving UE power if UE could start a little late to monitor the network response within a shorter response window.

Meanwhile, Rel-17 NTN study has already agreed to have an offset to be applied to the start of RAR response window as well as to the start of contention resolution timer for NTN scenarios, which is needed due to the large latency to the satellites. Since it is similar to the small data handling by network, we believe a common solution could be introduced. Thus, we have the following proposal.

Proposal 3: An offset should be introduced for the start of the response window which applies to RACH based small data transmission.

Proposal 4: A different monitor/response window timer should be introduced which applies to RACH based small data transmission.

Context fetch with and without anchor relocation

For anchor relocation case, UE resumes from RRC_INACTIVE. The receiving gNB, if able to resolve the gNB identity contained in the I-RNTI, should send the Retrieve UE Context Request to the anchor gNB and the anchor gNB feed back to the receiving gNB with the Retrieve UE Context Response to provide the UE AS context, and keeps the UE in the RRC_INACTIVE.

The user small data should be security protected with the same key as resumeMAC-I and could be transferred to anchor gNB for deciphering when the UE’s context relocation is retrieved successfully. If UE is verified successfully by anchor gNB, the anchor gNB delivers the data to 5GC.

The path switch procedure is performed, and the new N3 tunnel is built from receiving gNB to UPF. After context retrieval to receiving gNB, receiving gNB sends UE the RRC release with suspendConfig indication. When downlink data is arrived at UPF, the data can be forwarded to the receiving gNB directly and sent to UE along with RRC release message. The subsequent uplink data transmission, if any, should be sent through receiving gNB to UPF after anchor relocation.

Basically, the RNA update procedure with UE context relocation could be reused in principle for the RACH based SDT with anchor relocation case.

Proposal 5: The RNA update procedure with UE context relocation could be reused in principle for the RACH based SDT with anchor relocation case.

For without anchor relocation case, in which node and how to process the uplink data and subsequent data transfer should be specified since the UE context is stored in the anchor gNB. In last RAN2 meeting, RAN2 has sent LS [1] to RAN3 to confirm the agreement that RLC configuration used for SDT is based on UE stored configuration, and RAN2 assumption is that the RLC PDU will be processed in the receiving gNB (i.e. MAC is in the same node as RLC). It is also RAN2 understanding that it is up to RAN3 to make final decision on handling the RLC PDU. RAN3 should inform RAN2 if RAN3 has another solution.

According to the RAN2 assumption, one option is that the first uplink data might be stored in the receiving gNB waiting for the UE-specific RLC configuration to be temporarily relocated from the anchor gNB. Then PDCP PDUs are forwarded in Xn tunnels. In fact, this option violates the intention of ‘without anchor relocation’. The procedure has to retrieve at least parts of UE context (i.e. RLC configuration) through the Retrieve UE Context Message so that the receiving gNB is able to establish RLC entity to decode the RLC PDU. Further, this option causes additional latency to decode the first packet and unnecessary signaling overhead which is inefficient especially if the case is that UE’s uplink traffic is only one single small packet. The whole flow should also consider combining with non-relocation of full UE context which currently is possible contained in the Retrieve UE context Failure message.

In our view, if the user traffic is only one single UL packet, the simplest scheme should be to forward the first UL packet with the Retrieve UE Context Request message directly to anchor gNB once the receiving gNB gets this packet, and the anchor gNB makes decision to perform anchor relocation or partial anchor relocation (i.e. RLC configuration) or other actions.

Since the procedure and solution has big impacts on RAN3 protocol and architecture and it is obviously under RAN3 scope, the final decision should be made by RAN3.

Observation 5: The scheme of relocating UE-specific RLC configuration causes additional latency to decode the first packet and unnecessary signaling overhead especially if the case is that UE’s uplink traffic is only one single small packet.

Regarding the context fetch procedure, no matter with anchor relocation or without anchor relocation case, we think it should be anchor gNB’s responsibility to decide whether to forward the UE context to the receiving gNB or not. If anchor gNB decides to keep the UE context in itself, it responds to the receiving gNB with the Retrieve UE Context Failure message including an encapsulated RRC release message. Then the receiving gNB forwards the RRC release with suspend config to the UE.

Proposal 6: The anchor gNB decides whether to forward the UE context to the receiving gNB for UE SDT in RRC_INACTIVE.

The UE assistance information (or CG resource request message) is introduced in [2], which could be sent together with CCCH message as well as the user data in MSGA or Msg3. The receiving gNB forwards the assistance information to the anchor gNB which could help the anchor gNB to decide whether to relocate UE context and determine to transition UE RRC state.

Observation 6: It is beneficial for the anchor gNB to decide the UE context relocation and UE RRC state transition based on the assistance information.

Proposal 7: The Retrieve UE Context Request message contains the assistance information provided by the serving gNB. It can be up to RAN3 to decide the details.

# Conclusion

We have the following observations and, we’d recommend RAN2 to discuss and adopt the following proposals:

Switching between RACH based SDT and legacy RRC resume procedure (non-SDT)

Observation 1: It may be still difficult to decode the user data for network even after a number of payload retransmission in 2-step or 4-step RACH procedure.

Observation 2: It is inefficient for MSGA or Msg3 retransmission including user data with the same number of attempts of regular RACH.

Proposal 1: A new counter threshold or timer for MSGA/Msg1 retransmission is introduced for RACH based SDT.

Proposal 2: Network sends an indication in RAR or fallbackRAR to switch UE from RACH based SDT to legacy RRC resume procedure.

RACH monitoring enhancement for SDT

Observation 3: Network needs more time to decode user data contained in MSGA/Msg3 than the legacy RACH users.

Observation 4: It is beneficial for saving UE power if UE could start a little late to monitor the network response within a shorter response window.

Proposal 3: An offset should be introduced for the start of the response window which applies to RACH based small data transmission.

Proposal 4: A different monitor/response window timer should be introduced which applies to RACH based small data transmission.

Context fetch with and without anchor relocation

Proposal 5: The RNA update procedure with UE context relocation could be reused in principle for the RACH based SDT with anchor relocation case.

Observation 5: The scheme of relocating UE-specific RLC configuration causes additional latency to decode the first packet and unnecessary signaling overhead especially if the case is that UE’s uplink traffic is only one single small packet.

Proposal 6: The anchor gNB decides whether to forward the UE context to the receiving gNB for UE SDT in RRC_INACTIVE.

Observation 6: It is beneficial for the anchor gNB to decide the UE context relocation and UE RRC state transition based on the assistance information.

Proposal 7: The Retrieve UE Context Request message contains the assistance information provided by the serving gNB. It can be up to RAN3 to decide the details.

# References

- R2-2104400, LS to RAN3 to reply on small data transmission, Ericsson.

- R2-2107992, CP common aspects of SDT, Qualcomm.

# Annex