3GPP TSG-RAN WG2 #115-e	R2-2108193

Electronic meeting, 9th August – 27th August 2021

Agenda Item:	8.7.2.2

Source:	Ericsson

Title:	Discussion on service continuity for L2 sidelink relay

Document for:	Discussion, Decision

# 1	Introduction

According to the Sidelink relay WID that was agreed in [1], one of the objectives for the L2 U2N relay is as follow:

- Specify mechanisms for service continuity

NOTE 4:	Work specific to the mobility scenario of “between indirect (via a first Relay UE) and indirect (via a second Relay UE)”, and the group mobility is not supported in this release.

In this contribution service continuity aspects for L2 U2N are discussed.

# 2	Discussion

## 2.1	Path switch scenarios and procedures

### 2.2.2	Path switch procedure from direct path to indirect path

For the case where the path switch happens from a direct path to an indirect one, according to what has been captured in TS 38.836, the procedure it looks as follow:

Figure 1. Path switch procedure from direct to indirect path

However, regarding this procedure the following aspects were left out to be discussed during the WI phase:

NOTE:	Following are further discussed in WI phase, including: 
-	Whether Step 2 should be after Relay UE connects to the gNB (e.g., after step 4), if not yet before;
-	Whether Step 4 can be before step 2/3.

Given this, one aspect to highlight is whether step 4 should be executed before step 2/3. According to this, it is worth to clarify that the remote UE may select multiple relay UE candidates to connect to.

Remote UE may discover/select between multiple relay UE candidate to connect to.

If step 4 is allowed to be executed before step 2/3, what this implies is that multiple PC5 connections should first been setup and then the ones that are not related to a relay UE that has been selected should be released/aborted. This will of course cause a huge signalling overhead to the network (because it needs to provide multiple PC5 configurations) and to the remote UE.

If step 4 is done before step 2/3, this may cause a huge signalling overhead to the network and remote UE.

Therefore, it would be more natural that the remote UE, after discovering/selecting a relay UE, first wait for the configuration from the network to come and then setup the PC5 connection. This will also avoid the execution of unnecessary RRC reconfiguration procedures.

For the path switch from direct to indirect path, the PC5 connection establishment (step 4 of Fig. 4.5.4.2-1 of TR 38.836) shall be done only after the RRC reconfiguration procedure (step 2/3 of Fig. 4.5.4.2-1 of TR 38.836).

### 2.2.2	Path switch procedure from indirect path to direct path

For the case where the path switch happens from a direct path to an indirect one, according to what has been captured in TS 38.836, the procedure it looks as follow:

Figure 2. Path switch procedure from indirect to direct path

However, regarding this procedure the following aspects were left out to be discussed during the WI phase:

NOTE:	The order of step 6/7/8 is not restricted. Following are further discussed in WI phase, including: 
-	Whether Remote UE suspends data transmission via relay link after step 3; 
-	Whether Step 6 can be before or after step 3 and its necessity; 
-	Whether Step 7 can be after step 3 or step 5, and its necessity/replaced by PC5 reconfiguration; 
-	Whether Step 8 can be after step 5.

The focus here is whether the remote UE suspends data transmission via relay link after step 3. Regarding this, it is worth noticing that in the Sidelink Relay WID approved in [1] there is note that is stated that dual connectivity by the remote UE is not supported.

NOTE 2:	For L2 UE-to-Network Relay, it is assumed that the Remote UE has a single active connection towards gNB via only a single Relay UE at a given time in this release.

Given this outcome coming from the last RAN#91-e plenary meeting, it is straightforward that the UE should, indeed, suspend data transmission via relay UE after step 3. Thus, we propose:

For the path switch from indirect to direct path, the remote UE shall suspend data transmissions via relay link after getting the RRC reconfiguration message from the network (i.e., step 3 of Fig. 4.5.4.1-1 of TR 38.836).

A further aspect to be considered is whether the Step 7 can be after step 3 or step 5, and its necessity/replaced by PC5 reconfiguration. For this, we need to take into account two different use cases that may happen in step 7. One case is that the PC5 link is release, but another case it could be that the PC5 link is not released but simply reconfigured and used as sidelink standalone link (e.g., according to Rel-16 NR sidelink). Nevertheless, in either case since the remote UE shall not be allowed to keep two active transmission at a given time, the remote UE should release/reconfigure the PC5 link as soon as apply the RRC reconfiguration message received by the network.

For L2 U2N Relay, it is assumed that the Remote UE has a single active connection towards gNB via only a single Relay UE at a given time in this release.

For the path switch from indirect to direct path, the remote UE shall execute the PC5 release/reconfiguration (i.e., step 7 of Fig. 4.5.4.1-1 of TR 38.836) right after applying the RRC reconfiguration received by the network (i.e., step 3 of Fig. 4.5.4.1-1 of TR 38.836).

## 2.2	Packet forwarding during path switch

Further, another aspect that was completely overlooked during the study item was how the data packet forwarding is handled during the path switch. In fact, this should be addressed in order to ensure that packets are not lost when switching to the new path or after performing handover, the remote UE or the relay UE may eventually ask the (target) gNB to provide the status report about the latest received packets.

For path switch and handover procedure, it is unclear how to perform packet forwarding once the new path is setup.

Given that the entity involved during the path switch are the gNB and relay UE, and that is basically different from the normal case of handover, RAN2 should clearly check whether the current framework does it work. The feeling is that there are some differences and that additional functionalities are needed on the remote and relay UE side.

If we consider the case of path switch from a direct to an indirect path, in the uplink direction the remote UE may ask for a PDCP status report via the relay to the gNB. In such a case, the remote UE will be aware on what are the latest packet received by the network and schedule the necessary retransmissions.

In case of path switch from a direct to an indirect path, for the uplink direction the remote UE may ask for a PDCP status report via the relay to the gNB.

Similar, for the downlink direction the gNB may ask to the remote UE, via the relay UE, the PDCP status report so to understand which are the latest packets received by the remote UE. Therefore, we propose:

In case of path switch from a direct to an indirect path, for the downlink direction the gNB may ask for a PDCP status report via the relay to the remote UE.

Similarly, when performing a path switch from an indirect to a direct path, in the uplink direction the remote UE may ask directly to the gNB from a PDCP status report so to retransmit the packet that have not been received during the path switch procedure and vice versa for the downlink direction.

In case of path switch from an indirect to a direct path, for the uplink direction the remote UE may ask directly to the gNB for a PDCP status report.

In case of path switch from an indirect to a direct path, for the downlink direction the gNB may ask directly to the remote UE for a PDCP status report.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Remote UE may discover/select between multiple relay UE candidate to connect to.

Observation 2	If step 4 is done before step 2/3, this may cause a huge signalling overhead to the network and remote UE.

Observation 3	For L2 U2N Relay, it is assumed that the Remote UE has a single active connection towards gNB via only a single Relay UE at a given time in this release.

Observation 4	For path switch and handover procedure, it is unclear how to perform packet forwarding once the new path is setup.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For the path switch from direct to indirect path, the PC5 connection establishment (step 4 of Fig. 4.5.4.2-1 of TR 38.836) shall be done only after the RRC reconfiguration procedure (step 2/3 of Fig. 4.5.4.2-1 of TR 38.836).

Proposal 2	For the path switch from indirect to direct path, the remote UE shall suspend data transmissions via relay link after getting the RRC reconfiguration message from the network (i.e., step 3 of Fig. 4.5.4.1-1 of TR 38.836).

Proposal 3	For the path switch from indirect to direct path, the remote UE shall execute the PC5 release/reconfiguration (i.e., step 7 of Fig. 4.5.4.1-1 of TR 38.836) right after applying the RRC reconfiguration received by the network (i.e., step 3 of Fig. 4.5.4.1-1 of TR 38.836).

Proposal 4	In case of path switch from a direct to an indirect path, for the uplink direction the remote UE may ask for a PDCP status report via the relay to the gNB.

Proposal 5	In case of path switch from a direct to an indirect path, for the downlink direction the gNB may ask for a PDCP status report via the relay to the remote UE.

Proposal 6	In case of path switch from an indirect to a direct path, for the uplink direction the remote UE may ask directly to the gNB for a PDCP status report.

Proposal 7	In case of path switch from an indirect to a direct path, for the downlink direction the gNB may ask directly to the remote UE for a PDCP status report.

# References

RP-210904, WID on Sidelink Relay, OPPO, RAN#91-e, March 2021.